3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206075

Elbonia, Aug 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#6, New Sol: multicast access control for high number of public safety UEs

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2/ Rel-18

Abstract: This document try to introduce a new solution to KI#6 to address the high number of public safety UEs.

1. Introduction/Discussion

In TR 23.700-47, clause 4.2:

- Solutions shall ensure that the pre-emption and admission control mechanisms result in public safety UEs being able to complete the setup of the MBS services and then operate according to regulation and operator policy, when a mix of UEs participating in one or more services and/or one or more sessions within each service is present in the cell.

From this aspect, there is a case that multiple departments from the same mission critical organization may be served by different ASs and utilize the same PLMN network. With current mechanism, it is possible that one department/AS may occupy all the multicast resource, or pre-empt the multicast resource used by other group communication from other ASs/departments which leads to service interrupt. So it is important to enable the multicast resource to be fairly used by all the departments.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47 0.3.0.

# * * * * First change (All text new)* * * *

6.x	Solution #x: Multicast access control for high number of public safety UEs

6.x.1	Introduction

This solution addresses Key Issue #6. Considering the case that multiple departments from the same mission critical organization served by different AS may utilize the same PLMN network, as described in bullet 2 in clause 4.2, it is important to support the multicast resource is fairly used be by the different departments.

6.x.2	Functional description

During the UE join or the MBS session activation procedure, when the RAN is going to allocate resource for the join requesting UE, the RAN needs to check the resource availability for this UE who is mapped to a certain slice. Different departments’ users may be mapped into different slices. If there are available resource in the dedicated/shared multicast resource for that slice, then the join/MBS session activation will success and continue, else the RAN reject the PDU Session Resource setup and informs the UE, which may further reports the resource failure towards the AF to make further decision, e.g., switch to broadcast MBS session.

6.x.3	Procedures

Figure 6.x.3.1-1: Multicast control at NG-RAN

1.	AF triggered the multicast MBS session creation as defined in clause 7.2.1.2 or 7.2.1.3 of TS 23.247 [4]. The AF sent the service announcement to the UE for MBS join.

2.	UE joins the multicast MBS session via the procedure as defined in clause 7.2.1.3 of TS 23.247 [4]. Step 1 - step 6 clause 7.2.1.3 of TS 23.247 [4] is performed.

3. 	The NG-RAN checks the radio resource usage corresponding to the S-NSSAI.

The step 4 and step 5-7 are conditionally performed according to the radio resource status.

4.	If there are available resource in the dedicated/shared multicast resource for that slice, the residual steps in clause 7.2.1.3 of TS 23.247 [4] are continued. The NG-RAN updates the radio resource usage.

5.	The following steps are performed when the NG-RAN determined that there is no available dedicated and shared multicast resource for the slice. The NG-RAN performs AN specific signalling exchange with the UE to indicate that no radio resources for the UE to receive the multicast MBS session.

6.	The UE(s) can report the radio resource failure to the AF. Based on the reports from UE(s).

7.	The AF may determine to switch the group call over broadcast MBS session if existed, or else the AF create a new broadcast MBS session for this group call.

6.x.4	Impacts on services, entities, and interfaces

NG-RAN:

-	Determine the radio resource for each S-NSSAI to determine whether to allocate radio resource for a join requesting UE.

-	Send radio failure info to the UE.

UE:

-	Receive radio failure from NG-RAN and report it to the AF.

Editor's note:	Detailed impact will be determined by (and/or in collaboration with) RAN WGs.

# * * * * End of changes * * * *