SA WG2 Meeting #152e	S2-2206247

# August 17th – 26th, 2022; Elbonia               			(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	23.700-89: Conclusion on KI#1

Document for: 	Approval

Agenda Item: 	9.15

Work Item / Release:	FS_AMP / Rel-18

Abstract of the contribution: This paper proposes conclusions for KI#1

# 1 Discussion

As per the KI#1, description in TR 23.700-89, below questions are currently studied:

1.	Whether the current interworking procedure supports MME received the RFSP Index in use from 5GC? If no, what enhancements is needed.

2.	If the MME get the RFSP Index in used in handover procedure or idle mode mobility procedure, how and when it resumes to the subscription RFSP Index.

3.	When UE is under EPC, should MME receive any update of RFSP Index from 5GC and how.

Whereas the main objective of the KI is to avoid a ping-pong issue that may arise when a 5G prioritized UE is moved to 4G due to policy decision taken at the 5GC. There is no real need for updating RFSP Index value dynamically when the UE is served by the EPC. It is therefore proposed to conclude that for question 3) above, there is no need for updating RFSP Index when the UE is served by the EPC.

Proposal 1: When served by the EPC, there is no use case or requirement to provided updated RFSP Index from 5GC to EPC.

For question 1) and 2) above there are various solutions proposed considering the deployment scenarios where there is N26 interworking available and there is no N26 interworking. It is to be noted that the solutions that are proposed for deployments without N26 interface also works when N26 interface is used. To reduce impact on the products and minimize deployment complexity, while still provide solution for different deployment scenarios it is proposed to go with a single solution for both when N26 interworking is available and when N26 interworking is not available.

Proposal 2: Specify a single solution that works for both deployments scenarios of with and without N26 interworking.

# 2 Proposal

It is proposed to update 23.700-89 as follows

FIRST CHANGE

# 8	Conclusions

The following are concluded for KI#1:

-	When a UE is served by the EPC, there is no use case or requirement to provide updated RFSP Index from 5GC to the UE via the EPC.

-	A single solution will be specified that works for both deployment options of with and without N26 based interworking.

-

END OF CHANGES