3GPP SA WG2 Meeting #152E	S2-2206430

E-meeting, 17-26 Aug 2022

Source:	CATT

Title:	KI #4, Sol #18: Update to remove ENs

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract of the contribution: This paper addresses open Editor’s notes and provides updates for Solution #18 in TR 23.700-48.

1. Discussion

There are three open Editor’s notes for Solution #18 in TR 23.700-48.

In clause 6.18.1 and 6.18.2.1, there are two same Editor notes:

Editor's note: If the pre-condition is provided via AF request, how SMF gets the location information is FFS.

As specified in clause 5.6.7 of TS 23.501[x], if the Spatial Validity Condition is provided in AF request, the PCF subscribes to the SMF to receive notifications about change of UE location in an area of interest. The SMF is aware the UE location is in or out of the subscribed area of interest and sends notifications to the PCF. The PCF provides to the SMF the PCC rules. If the indication of EAS correlation is included in the PCC rule, and based on the UE location is in specific Area of Interest(s), and, the SMF determines which UE collection the UE belongs to without retrieving the location information provided by AF.

In clause 6.18.2.1:

Editor's note: How to solve multiple SMF instances is FFS.

It is proposed to solve the multiple SMF instances issue based on solution #16. The SMF selects and stores the common EAS in the corresponding AF traffic influence request information in UDR, and then the EASDF directly responds the common EAS to UEs. Alternatively, if the SMF determines to use the common DNS server to resolve the FQDN, it selects and stores the common DNS server in the corresponding AF traffic influence request information in UDR.

Editor's note: Whether and how to use EDNS Client Subnet option in DNS message handling rules to resolve same EAS IP address (es) is FFS.

The information of EDNS Client Subnet option should be not applied to DNS message handling rules for the case “use the same DNS server”. It is proposed to remove the related descriptions.

2. Proposal

It is proposed to agree the following texts in clause 6.18 of TR 23.700-48.

>>>> Start of Changes <<<<

6.18	Solution 18 (KI#4): Discovery of the same EAS for collections of UEs

6.18.1	Description

In order to realize the discovering the same EAS to collections of UEs, the following conditions should be satisfied:

-	The same FQDN in DNS query or a group of FQDNs that can be resolved to a certain EAS IP address by DNS server. The same EAS can be mapped to one FQDNs or a group of FQDNs. Only the collections of the UEs request the same DNS query with these FQDNs, that the DNS server can provide the same EAS IP address.

-	The collections of the UEs have the similar UE location. For example, in a stadium, some of the players can access the 5GC by the same gNB or a group of gNBs, UEs are located in a specific location area. According to the UE location information, the 5GC can identify that these UEs should be served by the same EAS.

-	The collections of the UEs are identified by UE Identifiers, e.g. UEs belong to an Ad-hoc group.

An AF request may be created to include indication of EAS correlation and pre-conditions as defined in step 1 of figure 4.3.6.2-1 of TS 23.502 [9], which is used to request selecting the same EAS for a collection of UEs. The pre-conditions indicate the conditions whether a UE belongs to a collection of UEs. The pre-conditions may reuse the information elements as defined in clause 5.6.7 of TS 23.501 [2], which includes Spatial Validity Condition (e.g. Area of Interest(s)), FQDN(s)/Application Identifier(s), Target UE Identifier(s). The PCF creates the PCC rule based on information provided in AF request, and sends the PCC rule to SMF. If the Spatial Validity Condition is provided in AF request, the PCF subscribes to the SMF to receive notifications about change of UE location in an area of interest. The SMF is aware the UE location is in or out of the subscribed area of interest and sends notifications to the PCF. The pre-conditions may also be a local policy preconfigured on SMF. The SMF determines whether the pre-conditions are satisfied, and if the indication of EAS correlation is included in the PCC rule, then SMF determines which UE collection the UE belongs to, according to the pre-conditions and received information (e.g. UE location is in the Area of Interest, requested FQDN in DNS query).

NOTE 1:	Target UE Identifier(s) can be group identifier and stored in UDR as current mechanism.

When the pre-conditions above are guaranteed, that for the 5GC side, the following procedures can be used to realize to discover the same EAS for these UEs:

-	Use the same DNS server to resolve the FQDN. It needs the 5GC to deliver the same DNS server IP address to these UEs, for example, the EASDF or local DNS server.

-	If the same local DNS server is selected, that whether the EAS will be resolved depends on the DNS server's mechanism that is out of scope.

-	If the same EASDF is selected, that all these DNS query should be treated with the same way, that applied to the same DNS message handling rules. Whether and how the C-DNS or L-DNS to resolve the same EAS depends on the DNS server's mechanism that is out of scope.

-	Directly responds to the DNS query to UE. This way only applies to the EASDF. That if the EASDF identifies that the UE's DNS query should be replied with the same EAS that other UEs use, the EASDF can directly respond with the DNS query without having the DNS related procedure to DNS server.

Table 6.18.1-1: Example of pre-conditions of configured the same EAS

| Pre-condition | Actions to configured the same EAS to the UEs which satisfies the pre-condition |
| --- | --- |
| UE location: - Cell ID: from XXX1 to XXX9 - DNAI: from XXA to XXE - TAI Area(s) of Interest FQDN: FQDN = ABC.com UE Identifiers | Action 1: DNS message handling rules - Local DNS server = 10.1.1.1 - ECS option = X.X.X.X Action 2: EAS IP address - 192.168.1.1 |

6.18.2	Procedures

6.18.2.1	EASDF-related procedure

Figure 6.18.2.1-1: EASDF related procedure to discover the same EAS

0.	The AF may request the indication of EAS correlation and pre-conditions and the PCF creates the PCC rule correspondingly as described in clause 6.18.1.

0.	The SMF determines the pre-conditions of the UEs that should be configured with the same EAS. The pre-conditions may be a local policy or derived based on PCC rule received from PCF. The pre-conditions are defined below:

-	The same FQDN in DNS query or range of FQDNs that can be resolved to a certain EAS IP address by DNS server. The same EAS can be mapped to one FQDNs or a group of FQDNs. Only the collections of the UEs request the same DNS query with these FQDNs, that the DNS server can provide the same EAS IP address(es).

-	The collections of the UEs have the similar UE location, for example, the same UE location or the UE location in the Area of Interest. According to the UE location information or UE is IN/OUT of Area(s) of Interest, the 5GC can identify that these UEs should be served by the same EAS. The UE location includes: Cell ID, range of Cell ID, TAI lists, DNAI, range of DNAI, gNB ID, range of gNB ID, DNN, S-NSSAI.

NOTE 2:	If the pre-conditions is provided by AF and include location information, then the SMF is aware the UE location is in or out of the subscribed area of interest and sends notifications to the PCF, and the SMF determines which UE collection the UE belongs, as described in clause 6.18.1.

0.	The 5GC can deliver the same DNS server IP address to these UEs that satisfy the pre-conditions, for example, the EASDF.

1.	The UE sends a DNS Query message to the EASDF.

2.	The same procedure as step 8 defined in figure 6.2.3.2.2-1 of TS 23.548 [3]. The EASDF provides the FQDN in DNS query to SMF.

3.	The same procedure as step 9 defined in figure 6.2.3.2.2-1 of TS 23.548 [3].

4.	[Conditional] The SMF determines the Area(s) of Interest and subscribes to UE mobility event notification from the AMF (e.g. location reporting, UE moving into or out of Area of Interest), or the UE location from AMF by Namf_EventExposure_Subscribe (event ID = UE moving in or out of Area of Interest, UE location). The UE location information and UE presence IN or OUT of Area of Interest can be used by SMF to identify whether this UE should be configured with the same EAS.

5.	[Conditional] The AMF responds to SMF.

6.	[Conditional] The AMF notifies the SMF of UE(s) presence (IN, OUT, or UNKNOWN) in the Area(s) of Interest or the UE location information, including: TAI, Cell ID and etc.

7.	SMF determines whether the UE is satisfied with the pre-conditions, according to UE location information and/or FQDN(s). If the UE location information is in the scope of UE location that defined in pre-conditions, and the FQDN is also in the scope of pre-defined scope, the SMF determines:

-	use the same DNS server to resolve the FQDN. The DNS query should be treated with the same way, that applied to the same DNS message handling rules, e.g. the same DNS server address is indicated in the Forwarding Action.

-	directly responds to the DNS query to UE. This way only applies to the EASDF. That if the SMF identifies that the UE's DNS query should be replied with the same EAS that other UEs use, the SMF can set the DNS message handling rules to EASDF to directly respond with the DNS query without having the DNS related procedure to DNS server, e.g. add a new action for Forwarding Action: forward the DNS response with specific EAS IP address to UE.

For the multiple SMF instances case, the solution is basically based on solution #16. The SMF selects and stores the common EAS in the corresponding AF traffic influence request information in UDR, and then the EASDF directly responds the common EAS to UEs. Alternatively, if the SMF determines to use the common DNS server to resolve the FQDN, it selects and stores the common DNS server in the corresponding AF traffic influence request information in UDR.

8.	The SMF updates the DNS message handling rule to guarantee the same EAS discovery for this UE. The SMF invokes Neasdf_DNSContext_Update Request (DNS message handling rules) to EASDF, and the DNS message handling rules are the same as pre-defined of other UEs.

For Option A, the DNS handling rule includes the same corresponding IP address to be used to build the EDNS Client Subnet option. For Option B, the DNS handling rule includes corresponding Local DNS Server IP address which is also the same as other UEs.

9-15.	The same procedure from step 11 to step 19 as defined in figure 6.2.3.2.2-1 of TS 23.548 [3].

According to the enhancement from SMF, this UE can receive the same EAS IP address as other UEs which satisfy the pre-condition.

6.18.2.2	Direct DNS response to UE

Figure 6.18.2.2-1: Direct DNS response to UE

0-6.	The same procedure as indicated in Figure 6.18.2.1-1 from step 0 to step 6.

7.	SMF determines whether the UE is satisfied with the pre-conditions, according to UE location information and/or FQDN. The SMF can also recover the DNAI, PSA ID, DNN, S-NSSAI of the UE or the UE's PDU Sessions. If the UE location information in the scope of pre-defined UE location, and the FQDN is also in the scope of pre-defined scope, the SMF determines to discover the same EAS to this UE as other UEs in group. SMF decides to have a direct DNS response to the UE.

8-9.	The same procedure as indicated in Figure 6.18.2.1-1 from step 8 to step 9. But SMF invokes Neasdf_DNSContext_Update Request (DNS message handling rules) to EASDF to directly respond to the UE DNS query with the same EAS IP address that other UEs are served.

10.	The EASDF sends the DNS Response(s) to the UE with the same EAS IP address that other UEs are served.

6.18.2.3	EAS Discovery Procedure with Local DNS Server/Resolver

Figure 6.18.2.3-1: EAS Discovery Procedure with Local DNS Server/Resolver

1.	The UE sends a PDU Session establishment request to SMF.

2.	SMF determines whether the UE is satisfied with the pre-conditions, according to UE location information and FQDN. The SMF can also recover the DNAI, PSA ID, DNN, S-NSSAI of the UE or the UE's PDU Sessions. If the UE location information in the scope of pre-defined UE location, and the FQDN is also in the scope of pre-defined scope, the SMF determines to discover the same EAS to this UE as other UEs in group. The SMF determines the DNAI and selects corresponding PSA UPF serves the DNAI, and determines DNS server(s) to ensure a dynamic group uses the same EAS and/or same DNAI.

3.	Optionally, the SMF selects and inserts UL-CL/BP. The SMF configures the UL-CL/BP for DNS Query handling.

4.	The SMF includes the IP address of Local DNS Server in PDU Session Establishment Accept message.

5.	The EAS information (e.g. EAS IP address) is resolved by Local DNS Server and sent the UE.

6.18.3	Impacts on services, entities and interfaces

SMF:

-	determines the pre-conditions of whether the collections of the UE should be configured with the same EAS.

-	determines whether the UE is satisfied with the pre-conditions, according to UE location information and FQDN.

-	decides to have a direct DNS response to the UE or configure the same DNS message handling rule to this UE DNS query, according to the configurations in the pre-conditions.

EASDF:

-	directly responds to the UE DNS query with the EAS IP address that other UEs are served.

AF:

-	sends the indication of EAS correlation.

PCF:

-	creates PCC rule based on information provided in AF request.

UDR:

-	stores common DNS server for the collection of UEs.

>>>> End of Changes <<<<