3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206752

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Key Issue#3: evaluation

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract: Key issue#3 “Local Area Restriction for an LMF and GMLC” has four solutions in the TR. After several meeting discussion, they are ready for evaluation.

1. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71.

# * * * * First change * * * *

7.x	Key Issue #3: Local Area Restriction for an LMF and GMLC

The clause evaluates the solutions for KI#3 as following.

Solution#6, #7, #8, #9 are all addressing key issue#3: Local Area Restriction for an LMF and GMLC. Three aspects “parameter configuration”, “parameter exchange”, and “LMF selection logic” are used to summarize the solutions:

|  | Solution#6 | Solution#7 | Solution#8 | Solution#9 |
| --- | --- | --- | --- | --- |
| Parameter configuration | GMLC: configured with LMF ID | AMF/NRF: Mapping of AF/client ID and LMF ID | AMF: GMLC service area | AMF/UDM: Mapping of UE ID and LMF/GMLC address |
| Parameter exchange | GMLC to AMF: Configured LMF ID | If mapping is on NRF, AMF retrieves the LMF ID | none | If mapping is on UDM, AMF fetches it. |
| LMF selection | AMF, based on provisioned LMF ID | By AMF | AMF, make sure LMF can serve the service area of the GMLC | By AMF |

# * * * * End of changes * * * *