3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2207096

Elbonia, August 17 – 26, 2022	(revision of S2-2206222r02)

Source:	Huawei, HiSilicon

Title:	KI#2: Evaluation and Conclusion

Document for:	Approval

Agenda Item:	9.12

Work Item / Release:	FS_5GSATB / Rel-18

Abstract: An evaluation and conclusion on KI#2 is proposed.

# 1. Introduction/Discussion

This proposal proposes to make an evaluation and conclusion on KI#2 (Support of Satellite Edge Computing via UPF on board).

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-27.

# * * * * First change (All new)* * *

# 7	Overall Evaluation

## 7.x	Evaluation of solutions for Key Issue #2 - Support of Satellite Edge Computing via UPF on board

Solutions #4 (Edge computing part of this solution), #5, #6(Edge computing part of this solution) and #10 describe supporting Satellite Edge Computing via UPF on board.

Session Breakout connectivity model (solutions #4, #5)

In solutions #4 and #5, the on-board UPF acts as UL CL/BP and local PSA. The Session Breakout connectivity model defined in TS 23.548 [6] is supported.

In solution#4 the on-board UPF can be inserted statically (i.e. during PDU Session Establishment procedure) or dynamically (i.e. during the DNS Query process), while in solution#5 the on-board UPF is inserted dynamically based on DNS Query initiated by UE.

For GEO UPF selection, in Solution#4, SMF selects on-board UPF based on DNAI as described in TS 23.548 [6]. The DNAI is derived by SMF based on the GEO satellite ID provided by AMF. Solution#5 requires the UPF onboard to report the gNB ID it connects to the SMF via N4 association, then the SMF can select a UPF connecting with the gNB serving the UE after determining the candidate UPFs using existing mechanism.

Distributed Anchor Point and multiple PDU Sessions connectivity models (solution #6)

In solution#6, the on-board UPF acts as PSA. The Distributed Anchor Point and multiple PDU Sessions connectivity model defined in TS 23.548 [6] are supported.

EAS rediscovery (solution #10)

In solution#10, EAS rediscovery is described. SMF stores EAS located in a GEO satellite or neighbouring GEO satellites on on-board local DNS. In case of EAS change, the local DNS reports this change to SMF. An interface between SMF and local DNS is needed for such reporting.

# * * * * Second change (All new)* * * *

# 8	Conclusions

## 8.x	Conclusion on solutions for Key Issue #2

On-board PSA UPF or ULCL/BP/local PSA UPF selection is performed as following:

-	The SMF determines DNAI based on local configuration and GEO satellite ID received from AMF. SMF performs UPF selection considering the DNAI.

NOTE 1: It’s assumed that a DNAI value is assigned for each GEO satellite ID by the operator. SMF is local configured with mapping relationship between DNAI and GEO satellite ID.

NOTE 2: The selected On-board UPF can be inserted as ULCL/BP/L-PSA reusing existing ULCL/BP insertion procedures defined in TS 23.502[3] or TS 23.548[6]

For Distributed Anchor Point and multiple PDU Sessions connectivity models:

-	Based on GEO satellite backhaul category from AMF, UE PCF may determine and update URSP rule including a appropriate DNN/S-NSSAI for services that allowed to access the services on GEO satellite.

# * * * * End of changes * * * *