SA WG2 Meeting #152e	S2-2207766

# Aug 17th – 26th, 2022; Elbonia               	    (revision of S2-2206213r01)

Source: 	China Telecom

Title: 	KI #1, Sol #20: update to resolve the ENs

Document for: 	Approval

Agenda Item: 	9.13

Work Item / Release:	FS_5WWC_Ph2 / Rel-18

Abstract of the contribution: This contribution proposes to update Solution#20 to resolve the ENs.

1	Discussion

This contribution updates to resolve the ENs.

2 Proposal

It is proposed to update TR 23.700-17 on FS_5WWC_Ph2 as follows.

# * * * * Start of Changes * * * *

6.20.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). (Sub) clause(s) may be added to capture details.

In Rel-18 FS_5WWC_Ph2, the KI#1 aims to provide differentiated service (e.g. QoS and charging) for UE and Non-3GPP devices connected behind a 5G RG. Guided by the existing knowledge in 3GPP (e.g. ProSe), the study for L-3 UE-to-Network Relay without N3IWF in 5G can also be applied to UE connects to the 5G-RG for UE approach.

In this solution, 5G-RG obtains User ID and UE info of devices connected behind a 5G-RG. And 5G-RG sends these information to the SMF, in turn delivered to the PCF. Then the PCF or SMF can make corresponding policy decisions related to e.g. QoS control or charging.

6.20.2	Procedures

#### 6.20.2.1	providing differentiated service for non-3GPP devices connected behind a 5G-RG

Editor's note:	This clause describes high-level procedures and information flows for the solution.

Figure 6.20.2.1-1

1. Non-3GPP device connects to 5G-RG with non-3GPP access network (e.g. establishes WiFi association).

During this procedure, 5G-RG obtains non-3GPP device’s User ID and Non-3GPP device info. The User ID is an identity of the non-3GPP device (e.g. MAC address, or SUCI which could be acquired by Authentication procedure for NSWOF in TS 33.501[9] v17.5.0 Annex S, or SUPI). To acquire SUCI/SUPI of non-3GPP device, it is assumed that non-3GPP device and 5G-RG support 5G NSWO where 5G-RG takes the role of "WLAN AN". In the case of acquiring SUPI of the non-3GPP device, during the authentication procedure for NSWOF in TS 33.501[9] the 5G-RG can add an indication in SWa protocol message to NSWOF and an indication in Nausf_UEAuthentication_Authenticate Request to AUSF for acquiring the SUPI. Then the AUSF can send SUPI as a new parameter in Nausf_UEAuthentication_Authenticate Response to NSWOFand deliver SUPI as a new parameter in SWa protocol message to the 5G-RG. The non-3GPP device info is used to assist identifying the non-3GPP device in the 5GC. For IP PDU Session Type, the non-3GPP device info is non-3GPP device IP info, e.g. TCP/UDP port ranges assigned to non-3GPP device by 5G-RG as NAT in case of IPV4. For Ethernet PDU Session Type, the non-3GPP device info is non-3GPP device Ethernet address.

2. 5G-RG sends Remote UE Report message to the SMF through the AMF. The Remote UE Report message includes Non-3GPP device’s User ID and Non-3GPP device info.

NOTE:	It is assumed that the 5G-RG already establishes a PDU session to simplify the procedures.

3. The SMF sends non-3GPP device’s User ID and Non-3GPP device info to the PCF by initiating the SM Association Modification procedure. The SMF may store the Non-3GPP device’s User ID and Non-3GPP device info. Then SMF may get subscription data for Non-3GPP device from UDM using Nudm_SDM_Get service.

4. The PCF stores the Non-3GPP device’s User ID and Non-3GPP device info. The PCF can get policy information for Non-3GPP device from local configuration (e.g. common policy for Non-3GPP device) or from UDR using Nudr_DM_Query service.

5. The PCF shall authorize policy for the non-3GPP device based on policy information at step 4.

6. The PCF generates and sends the PCC rules related to e.g. QoS or charging for non-3GPP devices connected behind a 5G-RG.

7. The SMF performs charging and QoS provision for the data traffic of the Non-3GPP device via existing mechanisms. And the SMF provides the charging information with corresponding User ID to the CHF. Same CHF session is proposed tobe used to deal with the 5G RG traffic and with the traffic of different devices behind the 5G RG e.g. distinguish different traffic by corresponding User ID. The details of processing will be determined by SA5 WG.

#### 6.20.2.2	differentiated QoS for 5GC-capable UE connected behind a 5G-RG

Figure 6.20.2.2 -1

The solution is based on the architecture as defined in clause 4.10 of TS 23.316[5] which UE connects to the overlay 5G network using the trusted N3GPP for UE approach.

In step 1, UE completes authentication and receives IP configuration during Registration procedure for trusted non-3GPP access in clause 4.12a of TS 23.502[3]. The 5G-RG obtains UE info which is used to assist identifying the UE in the 5GC. For IPv4, 5G-RG as a NAT shall assign TCP/UDP port ranges to individual UE(s) as UE info. .

In step 2 and 3, the 5G-RG (acts as a TNAP) performs step 2 and 3 as in clause 6.20.2.1 with the modification where the UE takes the role of "non-3GPP devices", to report User ID (e.g. MAC address, or SUCI of the UE) and UE info to 5GC.

In step 4, PCF shall authorize QoS to UE, e.g. with the configured default QoS for the UE as described from step 4 to 7 in clause 6.20.2.1. (by configuration) If needed, the 5GC at 5G-RG side can perform differentiated charging for the data flow of this UE following the procedures in 6.20.2.1, although the 5GC at the UE side will perform charging for the UE. This approach can handle the issue of repeat billing at 5G-RG, by excluding the amount of data generated by 5GC-capable UEs behind the 5G-RG.

In step 5 and 6, once the TNGF acquires the UE’s QoS information e.g. UE-AMBR or session-AMBR from 5GC when UE setup PDU session, it sends the corresponding QoS information to 5G-RG through the Ta interface. Then 5G-RG stores the QoS information for the UE and performs PDU session modification procedure to request the QoS for the UE’s data flow identified by UE info. For example, the 5G-RG can request MBR for the data flow of this UE in the PDU session of 5G-RG. The SMF and PCF perform subsequent procedures for PDU Session Modification to authorize and enforce the QoS for the UE’s data flow in the PDU session.

6.20.3	Impacts on Existing Nodes and Functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

5G-RG:

-	 5G-RG sends User ID and UE info to the SMF through the AMF, e.g. by Remote UE Report message

-	 5G-RG receives Qos information of the UE from TGNF and performs PDU session modification procedure to request the QoS for data flow of the UE.

SMF:

-  The SMF sends User ID and UE info to the PCF by initiating the SM Association Modification procedure.

-	The SMF provides the charging information with User ID to the CHF.

-  The SMF may store the User ID and UE info of devices connected behind a 5G-RG.

PCF:

-  The PCF can get policy related to e.g. QoS control, or charging of Non-3GPP device.

-  The PCF stores the User ID and UE info of devices connected behind a 5G-RG.

-  PCF authorizes policy for non-3GPP devices or UE behind the 5G-RG.

TNGF:

-  The TNGF sends the corresponding QoS information of the UE to 5G-RG through the Ta interface.

# * * * * End of changes * * * *