3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206115

Elbonia, Aug17th – 26th, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	Evaluation for FS_5TRS_URLLC KI#3

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract: Providing evaluations of solutions for KI#3.

1. Introduction/Discussion

There are 2 solutions (Sol#18, Sol#19) for FS_5TRS_URLLC KI#3.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-25.

# * * * * First change * * * *

7.x	Key Issue #3: Support for controlling 5G time synchronization service based on subscription

There are 2 solutions (Sol#18, Sol#19) for KI#3. The following technical issues are studied:

1. How to authorize time synchronization service based on UE subscription.

- 	TSCTSF performs time synchronization service configuration (i.e. ASTI and/or (g)PTP time distribution) based on UE's Time Synchronization Subscription Data. (Sol#18, Sol#19)

-	TSCTSF authorizes time synchronization service request based on UE subscription. (Sol#19)

2. How to enforce time synchronization service on a per UE basis based on subscription.

- 	TSCTSF performs time synchronization service configuration (i.e. ASTI and/or (g)PTP time distribution) based on UE's Time Synchronization Subscription Data. (Sol#18)

-	TSCTSF updates the 5G time distribution indication based on UE subscription. (Sol#19)

-	TSCTSF subscribes to Time Synchronization Subscription Data at the UDM. (Sol#18, Sol#19)

-	TSCTSF determines a suitable 5G timing source to provide time synchronization service for the UE. (Sol#19)

3. What parts of time synchronization service require a separate UE subscription (and authorization), if any.

-	UDM stores Time Synchronization Subscription Data and Access Stratum Time Synchronization Service Authorization fields management. (Sol#18)

-	UDM stores Subscription data including time synchronization parameters or timing resiliency parameters. (Sol#19)

# * * * * End of changes * * * *