SA WG2 Meeting #152E eMeeting	S2-2206334

Elbonia, Aug. 17 – 26, 2022 	(revision of S2-20xxxxx)

Source:	vivo, OPPO

Title:	FS_AIMLsys KI#3 Evaluations and Proposed Interim Conclusions

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: This contribution is present the evaluation for KI#3 and to propose the interim conclusions

Background

The intent of this PCR is to present the evaluation for the set of solutions for KI#3 and to propose an interim conclusion.

Discussions

There are 7 solutions which have been proposed to address KI#3, i.e. Solution#6, Solution#7, Solution#8, Solution#28, Solution#32, Solution#33, and Solution#34. Among these solutions, Solution#28 is proposed as the overall architecture framework and will only be further considered when all the KIs are getting close to be concluded.

- Solution #6 proposes that 5GC NWDAF provides new FL analytics on expected latency for candidate FL members, with input parameters (e.g. Candidate FL members, Local model size, Aggregated model size, etc.) provided by AI/ML application server. It is consumed by AI/ML application server to do FL members selection. User consent checking for each member from UDM should be done by NWDAF before exposing the analytics.

- Solution #7 proposes that 5GS provide the supported UL/DL data rate to AI/ML application server to make the decision on AI/ML operation, e.g. to determine the splitting point for AI/ML model splitting, or to determine the AI/ML model and the size of parameter for AI/ML model downloading. It requires the RAN node to estimate the supported UL/DL data rate based on the channel condition between RAN node and UE, the bandwidth available for the UE and other factors, and then the RAN sends data rate monitoring results to UPF over user plane, which will be exposed to AI/ML application server by UPF via SMF.

- Solution #8 addresses both KI#2 and KI#3, it proposes that 5G NF/NWDAF provides 5GC information or analytics to UE(KI#2) or AF(KI#3) for the purpose of triggering decisions on AI/ML operation or application adaptation. For KI#3, AF subscribes for network information or data analytics from 5GC for the UE, and NF/NWDAF notifies AF about the non-AF Session/PDU Session specific network information or data analytics.

- Solution #32 proposes that 5GC NWDAF provides enhanced "DN Performance", "Observed Service Experience" analytics to assist AF to make AI/ML decisions for split inference. New inputs and new outputs for these existing analytics IDs are defined in this solution, for example, Statistics/predictions on traffic rate for UL, on packet delay for UL and on packet loss rate for UL.

- Solution #33 proposes a mechanism to enable the network operator to have the control for which UE is allowed for participation in the Application layer AI/ML operation. The 5G system can expose the network authorization status of the UE to the AF for the UE participation in the corresponding Application AI/ML operation. Such awareness of the network authorization status for the UE can assist the AF's decision for the UE selection for the corresponding Application AI/ML operation. Network authorization status for the UE is newly defined and stored as subscription information of UE in the UDM.

- Solution #34 proposes that NWDAF can expose the prediction/statistics of Application AI/ML traffic transmission status(User Data Congestion, Observed Service Experience, NF load and QoS flow level data for the Application AI/ML traffic) to the AF. But this solution does not clarify what’s the usage of these prediction/statistics by AF.

Proposals

It is proposed to add the following text to TR 23.700-80.

***** Start of Changes (all new text) *****

7 	Evaluation

7.3	Key Issue#3

There are 7 solutions which have been proposed to address KI#3, i.e. Solution#6, Solution#7, Solution#8, Solution#28, Solution#32, Solution#33, and Solution#34. Among these solutions, Solution#28 is proposed as the overall architecture framework and will only be further considered when all the KIs are getting close to be concluded.

- Solution #6 proposes that 5GC NWDAF provides new FL analytics on expected latency for candidate FL members, with input parameters (e.g. Candidate FL members, Local model size, Aggregated model size, etc.) provided by AI/ML application server. It is consumed by AI/ML application server to do FL members selection. User consent checking for each member from UDM should be done by NWDAF before exposing the analytics.

- Solution #7 proposes that 5GS provide the supported UL/DL data rate to AI/ML application server to make the decision on AI/ML operation, e.g. to determine the splitting point for AI/ML model splitting, or to determine the AI/ML model and the size of parameter for AI/ML model downloading. It requires the RAN node to estimate the supported UL/DL data rate based on the channel condition between RAN node and UE, the bandwidth available for the UE and other factors, and then the RAN sends data rate monitoring results to UPF over user plane, which will be exposed to AI/ML application server by UPF via SMF.

- Solution #8 addresses both KI#2 and KI#3, it proposes that 5G NF/NWDAF provides 5GC information or analytics to UE(KI#2) or AF(KI#3) for the purpose of triggering decisions on AI/ML operation or application adaptation. For KI#3, AF subscribes for network information or data analytics from 5GC for the UE, and NF/NWDAF notifies AF about the non-AF Session/PDU Session specific network information or data analytics.

- Solution #32 proposes that 5GC NWDAF provides enhanced "DN Performance", "Observed Service Experience" analytics to assist AF to make AI/ML decisions for split inference. New inputs and new outputs for these existing analytics IDs are defined in this solution, for example, Statistics/predictions on traffic rate for UL, on packet delay for UL and on packet loss rate for UL.

- Solution #33 proposes a mechanism to enable the network operator to have the control for which UE is allowed for participation in the Application layer AI/ML operation. The 5G system can expose the network authorization status of the UE to the AF for the UE participation in the corresponding Application AI/ML operation. Such awareness of the network authorization status for the UE can assist the AF's decision for the UE selection for the corresponding Application AI/ML operation. Network authorization status for the UE is newly defined and stored as subscription information of UE in the UDM.

- Solution #34 proposes that NWDAF can expose the prediction/statistics of Application AI/ML traffic transmission status(User Data Congestion, Observed Service Experience, NF load and QoS flow level data for the Application AI/ML traffic) to the AF. But this solution does not clarify what’s the usage of these prediction/statistics by AF.

***** Next Changes (all new text) *****

8 	Conclusions

8.3	Key Issue#3

Proposal for the interim conclusions for KI#3:

-	Minimize 5GS signalling and processing overheads when exposing 5GC information to AF;

-	The user consent checking is also required before the UE related information or data analytics are exposed to the 3rd party.

***** End of Changes *****