3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206605

Elbonia, Aug 17 - 26, 2022	(revision of S2-220xxxx)

Source:	KDDI

Title:	KI#4, Sol#12: Update to resolve EN.

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: This contribution proposes update of solution#12 for KI#4.

1. Discussion

In SA2#151E, following Editor's Notes was captured in Solution#12:

2.	PCF may initiate the PDU Session modification procedure as defined in clause 4.3.3.2 of TS 23.502 [3]. The PCF generates appropriate PCC rules based on the information from AF as mentioned in step 1, e.g. 5QI, PER and PDB. The PDU Set level packet handling/treatment requirements may be considered during the PCC rule generation. The PCC rules may also include the detection rules of service data flow, PDU Set level packet handling/treatment policy, PDU Set identification rules. The PCF sends the PCC rules to SMF.

Editor's note:	The kind of PDU Set detection rules that PCF generates is FFS. The relationship between these rules and the detection mechanisms in 6.12.3.2 (except for periodicity) needs to be clarified.

In the step 2 above, there is no clear description for PDU Set identification rules as shown in clause 6.12.3.2. Therefore, it is proposed to update the PDU Set identification rules and remove the Editor's Notes.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# *** Start of Changes ***

6.12.3.1	PDU Set integrated packet handling

High level procedure of PDU Set integrated packet handling can be shown as following.

Figure 6.12.3.1-1: High-level Procedure of PDU Set integrated packet handling

As shown in Figure 6.12.3.1-1, a schematic diagram of the modification process of the PDU session for the XR service is given. The process includes the following steps:

0.	The UE establishes a PDU Session as defined in clause 4.3.2.2.1 of TS 23.502 [3]. A network slice type for XR service can be used for such a PDU Session.

1.	Optionally, as defined in clause 4.15.6.6 of TS 23.502 [3], the AF may invoke the Nnef_AFsessionWithQoS_Create request to set up an AF session with required QoS. In the step 1, the AF may send the following information to PCF:

Flow description of the target media service data flows for PDU Set handling.

Burst periodicity.

The PDU Set level packet handling/treatment requirements. The PDU Set level packet handling/treatment requirements are optional and may include e.g. PDU Set Error Rate, PDU Set Delay Budget, Maximum PDU Set Loss Rate/Number, Media protocol info (e.g. which RFC is used in user data)  etc.

2.	PCF may initiate the PDU Session modification procedure as defined in clause 4.3.3.2 of TS 23.502 [3]. The PCF generates appropriate PCC rules based on the information from AF as mentioned in step 1, e.g. 5QI, PER and PDB. The PDU Set level packet handling/treatment requirements may be considered during the PCC rule generation. The PCC rules may also include the detection rules of service data flow, PDU Set level packet handling/treatment policy, PDU Set identification rules. The PDU Set identification rules may contain RTP header identification method as shown in clause 6.12.3.2.1, and/or RTP payload identification method as shown in clause 6.12.3.2.2 (H.264), clause 6.12.3.2.4 (H.265) and clause 6.12.3.2.5 (H.266). The PCF sends the PCC rules to SMF.

3.	SMF generates the QoS profiles and N4 rules based on the PCC rules from PCF, which may include the packet handling/treatment policy. SMF sends the N4 rules to UPF via the N4 rule, which may include the identification and marking rule for PDU Set. Besides, SMF also sends the QoS profiles to the RAN node via AMF, and instructs RAN to perform PDU Set integrated QoS handling.

NOTE: Based on Media protocol info included in the PDU Set level packet handling/treatment requirements or based on pre-configuration, SMF generates N4 rules accordingly to instruct UPF to identify the PDU Sets of corresponding protocols as shown in clause 6.12.3.2.

4.	Based on received N4 rules or locally configuration on the UPF, the UPF identifies the PDU Set and marks PDU Set info in the GTP-U layer in the DL packets, including start/end indication of the PDU Set and the PDU Set ID. Details of how to identify the PDU Set are in clause 6.12.3.2.

5.	The RAN identifies the PDU Set based on the PDU Set info in GTP-U header and transmits PDUs within the PDU Set in an integrated manner, e.g. The RAN may drop the PDU Set as a whole in case of poor network condition, and execute packet handling/treatment policy.

NOTE: The details of RAN behaviour is up to RAN WG.

# *** End of Changes ***