SA WG2 Meeting #S2-152E	S2-2206896

17 – 26 August , 2022, E-meeting

Source:	Xiaomi

Title:	evaluation for key issue#2

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	5G_AIMLsys/Rel-18

Abstract of the contribution: This paper proposes the evaluation for key Issue#2

*** Start of changes ***

7	Evaluation

Editor's note:	This clause provides an evaluation of the solutions.

7.3 Evaluation for KI#2 5GC information exposure to UE

Control plane based solutions:

- SMF serving the PDU Session for AI/ML based services/applications provides analytics information obtained from the NWDAF to the UE.(solution#3,#4)

- During registration, based on UE request, AMF subscribes to NWDAF for network analytics, and deliver to UE via NAS.(solution #41)

User plane based solution:

- extend the existing DCAF/AF functionality and architecture as defined in TS 26.531 [7] to expose network data analytics related to the UE to assist the UE’s local Application AI/ML operation. (solution#2)

- AI/ML translator (may be integrated into DCAF or AF) is proposed for translating (mapping) the Application layer AI/ML related requests received from UE(s) to the requests for 5GC (i.e., from AI/ML request to 5GC request), and converting the information or analytics produced at 5GC to AI/ML assistance information for UE(s) (solution30)

Both Control plane and User plane:

- During the Registration procedure, the AMF checks whether the UE is allowed to request/subscribe to Network Exposure events, which Analytics and Network Exposure information the UE is allowed to collect, and applicable information delivery mechanism (i.e., UP, CP or both)(solution#5).

- AF subscribes to network analytics/network information or data analytics for the UE. Then SMF obtained the network analytics information delivers to UE via NAS, or user plane via AF(solutions#8)

*** End of changes ***