SA WG2 Meeting #S2-152e(e-meeting)	S2-2205653

17 - 26 August 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Ericsson

Title:	KI#5, New Sol, solution for UE location service via mobile IAB-node

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR/Rel-18

Abstract of the contribution: Introduction of a new solution for KI#5.

# 1. Introduction

This paper introduces a new solution for KI#5

# 2. Proposal

This paper proposes the following new solution to TR 23.700-05 clause 6.

* Start of change *

## 6.x	Solution #x: Solution for UE location service via mobile IAB-node

### 6.x.1	General

This solution relates to KI#5 "Support of location services for UEs accessing via a mobile base station relay".

### 6.x.2	Functional descriptions

This solution is based on the following principles:

-	NG-RAN holds the 5G-GUTI allocated by the AMF handling the mobile IAB-node (e.g. AMF provides the info to NG-RAN as part of the Registration procedure).

-	When a UE access network via a mobile IAB-node, the NG-RAN (i.e., IAB-donor gNB) provides the 5G-GUTI of the mobile IAB-node, together with ULI, as part of the NGAP message (e.g. NGAP Initial UE message) to AMF which handles the UE. The AMF stores the 5G-GUTI of the IAB-node as part of the UE context.

-	When AMF triggers location request towards LMF for the UE, the AMF provides also the 5G-GUTI of the IAB-node, together with the cell information from the additional ULI (as proposed in solution#9) if it’s available.

-	When LMF determines the location of UE, it also retrieves the location of mobile IAB-node based on the 5G-GUTI/Cell information provided by the AMF.

### 6.x.3	Procedures

#### 6.x.3.1	Location service for UE served by a mobile IAB-node

Figure 6.x.3-1: Positioning procedure for UE accessing via mobile IAB node

1.	The IAB-node (the IAB-UE part) is registered to the network and properly configured to provide access service for UE. NG-RAN node receives the 5G-GUTI to IAB-UE from AMF serving the IAB-UE (i.e. the AMF-IAB) as part of the NGAP message (e.g. Initial context setup request or UE context modification request) where 5G-GUTI can be included (5G-S-TMSI/UE Identity Index value is already defined in TS 38.413). It shall also be possible for the AMF-IAB to provide only 5G-S-TMSI and NG-RAN formulates the 5G-GUTI based on GUAMI and the 5G-S-TMSI.

2.	The UE is camped on the cell served by the mobile IAB-node and initiates the registration procedure. For the UE accessing the mobile IAB-node, the NG-RAN includes the 5G-GUTI and ULI information of the mobile IAB-node as additional information in NGAP message for the UE. The AMF serving the UE (i.e. AMF-UE) store the information and finish the UE registration procedure.

NOTE: In case of 5G-GUTI update for the IAB-node (e.g. in case of IAB-node mobility with AMF change), the NG-RAN provides the updated IAB-node 5G-GUTI info to the AMF-UE for all UEs connected via the IAB-node (detailed messages needs to by synched with the solution for KI#2 and KI#3. using a NG-AP message associated to the UEs served by the IAB-UE.

3.	The AMF-UE initiates the location reporting procedure for the UE, as defined in the 5GC-MT-LR procedure or the 5GC-MO-LR procedure of TS 23.273 [4].

4.	The AMF-UE selects LMF for the UE and invokes Nlmf_Location_DetermineLocation Request service operation towards the LMF including the LCS Correlation identifier for UE, the 5G-GUTI and ULI information of the IAB-node.

5.	The LMF performs one or more of the positioning procedures towards the UE via the AMF-UE as described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [4].

6.	The LMF finds the AMF-IAB based on the GUAMI based on the 5G-GUTI received in step 4 via NRF, and initiates the location reporting procedure for IAB-node/IAB-UE based on the 5G-GUTI received in step 4. The UE positioning procedure in step 5 and IAB-UE positioning procedure in step 6 may can be performed at the same time.

7.	The LMF determines the location of UE considering the location information received in step 5 for the UE and the location information received in step 6 for the IAB-UE. Then the LMF provides the UE location to the AMF-UE serving the UE.

NOTE: How the LMF determines the location of UE by taking the location of the IAB-UE into account is up to LMF implementation.

### 6.x.4	Impacts on services, entities, and interfaces

LMF:

-	Support receiving 5G-GUTI/cell information of IAB node from AMF when LCS for UE is requested.

-	Support using the 5G-GUTI to determine the serving AMF and trigger location estimation of the IAB-node.

-	Support determining the UE location based on the position information of the UE and the location estimated for the IAB-node.

AMF:

-	Support receiving and storing 5G-GUTI/cell information from NG-RAN when UE is accessing via mobile IAB-node.

-	Support providing the 5G-GUTI/cell information of the IAB-node to LMF when send LCS request for a UE accessing via mobile IAB-node.

-	Support LCS related message delievery from LMF based on 5G-GUTI.

NG-RAN:

-	Support providing the 5G-GUTI/cell information of the IAB-node to the AMF when UE is accessing via mobile IAB-node.

-	Retain in IAB-UE context the UEs' AMFs that need to be updated with the 5G-GUTI of the IAB-UE when this changes.

* End of changes *