3GPP TSG SA WG2 Meeting #152-e	S2-2205671r02

# August 17th – 26th, 2022, Electronic meeting

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3667 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NSAGs and their priorities based on S-NSSAIs in both the Allowed NSSAI and the Pending NSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_slice-Core |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When the UE NAS provides to the UE AS the NSAGs and their priorities, the NSAGs and their priorities are based on the S-NSSAIs in the Allowed NSSAI as an input to cell reselection according to the latest 23.501. However, this logic may not work in case where the UE holds the Pending NSSAI for 3GPP access. If the UE reselects to a cell considering the network slice availability based only on the NSSAIs in the Allowed NSSAI while the NSSAA for pending S-NSSAI(s) is ongoing, the UE may end up in a target cell that does not support one or more of the pending S-NSSAI(s) which have just been made allowed after successful NSSAA procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to add S-NSSAI(s) in the Pending NSSAI as an input to cell reselection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | S-NSSAI(s) that requires the NSSAA procedure may not be used by the UE after the Network Slice based cell reselection procedure takes place. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.4.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF 1st CHANGE

5.3.4.3.4	Network Slice based cell reselection

When one or more S-NSSAI(s) are associated with NSAG(s), the UE may perform Network Slice based cell reselection as described in TS 38.300 [27], TS 38.304 [50], TS 38.331 [28] and TS 24.501 [47].

When providing NSAG Information to the UE, the AMF should also provide the NSAG priority information for the NSAGs provided in the NSAG Information. The AMF determines the NSAG priority information based on operator's policy. If the UE has received NSAG priority information from the AMF, the UE shall use the NSAG priority information provided by the AMF for cell reselection as described below. If the UE has not received any NSAG priority information from the AMF, the UE shall not use Network Slice based cell reselection at all.

The UE NAS provides to the UE AS the NSAGs and their priorities as received from the AMF for the S-NSSAIs in the Allowed NSSAI and the S-NSSAIs of the 3GPP access in the Pending NSSAI as input to cell reselection. Except when the UE intends to register with a new set of S-NSSAIs with a Requested NSSAI different from the current Allowed NSSAI, in which case the UE NAS provides to the UE AS layer the NSAGs and their priorities as received from the AMF for the S-NSSAIs in the Requested NSSAI and optionally the S-NSSAIs in the Pending NSSAI of the 3GPP access if stored in the UE, and this may trigger a cell reselection, before sending the Registration Request including the new Requested NSSAI.

In case that the NSSAA procedure fails, the UE NAS may provide to the UE AS layer the NSAGs and their priorities for the S-NSSAIs in the Requested NSSAI that has received from the AMF by the UE Configuration Update Command message during the NSSAA procedure.

NOTE:	How the UE NAS provides the NSAGs priorities to UE AS is based internal UE interface, and not specified.

END OF CHANGE