SA WG2 Meeting #S2-152E	S2-2205925

17 – 26 August, 2022, Electronic meeting

Source:	Nokia, Nokia Shanghai Bell

Title:	Updates to solution 19 for key issue 1

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_MBS2 / Rel-18

Abstract of the contribution: this paper proposes updates for solution 19 for key issue 1

1.	Discussion

Most editor´s notes for solution 19 point to RAN dependencies. It is more appropriate to document such dependencies as NOTEs as the solution will likely not be updated but the dependencies will be considered during the evaluation.

2		Proposal

It is proposed to include the following changes in TR 23.700-47.

*** 1st Change ***

6.19	Solution #19: Procedures for Transmission mode tor inactive data reception

6.19.1	Introduction

This solution addresses Key Issue #1.

Editor's note:	Whether/how the RAN-centric option A works and whether it is adopted depend on the discussion and decision in RAN WGs.

6.19.2	Functional description

This solution builds on top of solution 1.

UEs that transition to RRC inactive mode can move freely in their RAN notification area without notifying the RAN nodes. Their location is thus not known at cell level. At the moment only the RAN node serving a UE is aware that it is participating in an MBS multicast session. Additional RAN nodes not handling UEs within the multicast session but within RAN notification areas of those UEs may need to be made aware of the multicast session and perform procedures for the multicast session, such as transmitting data of the multicast session and managing the related the shared delivery path.

Editor's note: 	Whether additional RAN nodes not handling UEs within the multicast session need to be made aware of the multicast session and perform procedures for the multicast session is FFS.

This solution describes two option to accomplish that with CN impacts:

Option A: RAN nodes inform their peers serving parts of RAN notification areas of UEs in an MBS session about the MBS session. The peer RAN nodes request assistance information from the CN via shared delivery establishment.

Option B: The MB-SMF informs all RAN nodes in a service area of an MBS session via the AMF about the MBS session

NOTE 2:	The RAN-centric option A is in scope of RAN WGs to a large extent. Whether this option is selected thus depends on RAN WG.

As service areas are optional for multicast MBS sessions, all RAN nodes in a PLMN are possible affected by Option B.

RRC_inactive UEs can move to cells where the transmission mode for RRC connected state is applied and then need to transition to the RRC connected state to receive MBS data.

The current MBS session activation procedures use paging for the MBS session and trigger RRC-inactive UEs to transition to RRC-connected state. For the delivery mode for RRC_inactive reception, RRC-inactive UEs should be made aware that the MBS multicast session is activated, but remain in RRC-inactive state for multicast data reception.

6.19.3	Procedures

Editor's note:	It is not assumed all the RAN node within the same RNA area have the same capability. It needs to be verified whether there are gaps in the procedures for inhomogeneous deployment.

6.19.3.1	Moving a UE to RRC Inactive state and providing assistance information to additional RAN nodes (Option A)

Figure 6.19.3.1-1: NG-RAN node moves a UE in 5MBS session to RRC Inactive state

0.	UE joined an MBS session and receives MBS data while in RRC connected state

1.	NG-RAN 1 uses assistance information to decide whether to apply the delivery mode for RRC-inactive reception for an MBS session, and whether to move one or several UEs in MBS session to RRC inactive state.

2.	RAN internal procedures.

3.	NG-RAN 1 determines that part of the RAN notification areas of the UEs is served by RAN 2. It informs RAN 2 that handling of the MBS session for inactive UEs is required in that area.

NOTE 1:	Depending on RAN decision, assistance information for MBS might be transferred in that step.

4.	The NG-RAN 2 establishes shared delivery with MB-SMF. The MB-SMF transmits assistance information as defined in solution 1for inactive reception in the shared delivery response.

5.	RAN internal procedures at NG-RAN 2 to handle MBS session.

NOTE 2:	Transfer of assistance information and shared delivery to additional RAN nodes have CN impacts.

6.19.3.2	MBS service activation

Figure 6.19.3.2-1: MBS service activation

1.	When triggered according to step 11 of Figure 7.2.5.2-1 of TS 23.247 [4], AMF sends NGAP activation request message to NG-RAN nodes.

2.	Related RAN procedures. RAN nodes decides whether to deliver in mode for RRC inactive reception, and otherwise will perform RAN paging. For the delivery mode for RRC_inactive reception, RRC-inactive UEs should be made aware that the MBS multicast session is activated, but remain in RRC-inactive state for multicast data reception.

3.	NG-RAN 1 determines that part of the RAN notification areas of UEs in MBS session is served by NG-RAN 2. It requests NG-RAN 2 to perform RAN paging for MBS session,

NOTE 1:	Depending on RAN decision, assistance information for MBS might be transferred in that step.

Editor's note: 	Whether a RAN node can request another RAN node to perform RAN paging for MBS session is FFS.

4.	If not yet done, the NG-RAN 2 establishes shared delivery with MB-SMF. The MB-SMF transmits assistance information (as defined in solution 1) for inactive reception in the shared delivery response.

5.	RAN internal procedures at NG-RAN 2 similar to step 2.

6.19.3.3	Triggering MBS service announcement by MB-SMF (Option B)

NOTE 1:	Unlike the broadcast session start, the present procedure is only used to make the RAN nodes aware that an MBS multicast session enabling delivery for inactive rerception is ongoing. RAN nodes can then decide whether to transmit data and select the delivery mode

￼

Figure 6.19.3.3-1: Triggering MBS service announcement by MB-SMF (Option B)

1.	Multicast MBS session suitable for delivery mode for inactive reception is created at MB-SMF.

Editor's note: 	It is FFS how an multicast MBS session suitable for delivery mode for inactive reception is created, i.e. what are the differences from the existing MBS session creation procedure.

2-5.	MB-SMF informs RAN nodes in service area of multicast session via AMFs of multicast session and asks them to announce multicast session.

Editor's note: 	It is FFS whether and how the MB-SMF can inform the NG-RAN without being triggered by MBS session join and establishment procedure.

6. RAN decides delivery mode for MBS session in a cell and whether to deliver data.

Editor's note: 	It is FFS whether and how the NG-RAN node can decide to deliver multicast data without performing MBS session establishment procedure.

7.	RAN related procedures

NOTE 2:	The steps 6 to 7 are to be decided by RAN WGs.

6.19.4	Impacts on services, entities and interfaces.

Functional entities defined in clause 5.3.2 of TS 23.247 [4] are reused.

In addition to impacts for solution 1:

NG-RAN:

-	Decide transmission mode for a multicast session in a cell.

-	Receive assistance information either aspart of request for service announcement or within shared delivery Establishment.

UE:

-	Receive MBS data in indicated transmission mode while MBS data is active.

MB-SMF:

-	inform RAN nodes in service area of multicast session (Option B).

-	provide assistance information as part of shared delivery establishment.