3GPP TSG-WG SA2 Meeting #152E e-meeting  	S2-2206077

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

# Title:	[DRAFT] LS on LS on AS-NAS layer interactions for MBS

# Response to:	LS (S2-2205426) on LS on AS-NAS layer interactions for MBS from RAN2

# Release:	Release 17

# Work Item:	5MBS, NR_MBS-Core

Source:	[Huawei to be] SA2

To:	RAN2, CT1

Cc:

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	Raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None.

1. Overall Description:

SA2 would like to thank RAN2 for LS on AS-NAS layer interactions for MBS, and SA2 would like to provide the following response.

RAN2 is currently reviewing the procedures in RAN2 specifications related to MBS and one of the doubts that arose during the process was related to whether some of the information should be passed to upper layers or not. Currently, RAN2 assumes that AS will indicate the following events/information related to MBS session: - When the UE receives a Paging message including a TMGI for activation of a multicast MBS session which the UE has previously joined, the UE will forward the TMGI to upper layers (for both UE in RRC_IDLE and RRC_INACTIVE states). - When the user plane resources are established for either an MBS broadcast or an MBS multicast session, the UE will notify upper layers about this (and include TMGI to identify the session). - When the user plane resources of either an MBS broadcast or an MBS multicast session are released, the UE will notify upper layers about this (and include TMGI to identify the session). RAN2 respectfully asks CT1 and/or SA2 to confirm whether the indications from AS to upper layers as mentioned above are needed and/or whether there are any other events or information concerning MBS at AS which the upper layers should be informed about.

SA2 agrees that the TMGI information is needed at NAS layer when the CM-IDLE UE receives a Paging message including a TMGI for activation of a multicast MBS session.

- For the UEs in CM-IDLE state, they need to send the service request message including the associated PDU session ID(s) of the TMGI after receiving the paging message including TMGI.

SA2 also agrees that for a UE in RRC_INACTIVE state the AS layer needs to provide the TMGI information to upper layers when receiving a Paging message including a TMGI for activation of a multicast MBS session so that client applications can take proper actions towards the user.

For scenario 2 and 3, SA2 assumes that AS layer needs to indicate to the related upper layer per received TMGI, so that client applications may take proper actions towards users. SA2 hasn’t seen other events or information to be notified from AS layer towards upper layer.

However, SA2 would like to point out that in the join request (presumably initiated by the application), a source specific IP multicast address may be used as identifier of an MBS multicast session instead of an TMGI. In this case a TMGI will be assigned to the MBS session in addition and used in subsequent signalling. An application might not be aware of the TMGI, but only the IP source specific multicast address as identifier for the MBS session unless it is being made aware of the mapping.

2. Actions:

To RAN2 group:

ACTION:   SA2 asks RAN2 group to take the above information into consideration

To CT1 group:

ACTION: 	SA2 asks CT1 group to take the above information into consideration and check if there is any gap that needs to be addressed.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #153e   	Oct 10 – 14, 2022 				Elbonia

TSG-SA2 Meeting #154   	Nov 14 – 18, 2022 				TBD