SA WG2 Meeting #S2-152e(e-meeting)	S2-2206623

17 - 26 August 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	CATT

Title:	KI #5, Sol #7: Update sol#7 to remove EN

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR

Abstract of the contribution: this pCR is proposed to update solution #7 to solve FFS.

# 1. Discussion

Solution#7 includes the following FFS:

Editor's note:	It is FFS how to handle the case when the UE and the Mobile base station relay are served by different PLMNs.

This pCR is proposed to solve the FFS and clarify the details of the step 7.

# 2. Proposal

It is proposed to update solution #7 to solve FFS.

* First change *

## 6.7	Solution #7: Network based Positioning of mobile base station relay for location services

### 6.7.1	General

This solution relates to KI#5 "Support of location services for UEs accessing via a mobile base station relay". It proposes a solution where the UE and the mobile base station relay/VMR uses the legacy location procedures and positioning protocols described in the specifications TS 23.273 [4], TS 37.355 [13] and NRPPa in TS 38.455 [14].

There are three different cases for a UE location estimation served by a mobile base station relay:

-	A UE performing network assisted GNSS location.

-	A UE performing positioning measurements based on signals from the mobile base station relay and other base stations gNBs.

-	A UE performing positioning measurements only on the mobile base station relay, i.e. no neighbouring gNBs to measure on. The UE may use the Sidelink positioning procedures investigated in the FS_Ranging_SL captured in TR 23.700-86 [15]

This solution does not address the GNSS based positioning estimation case as the GNSS location is not dependent on the exact location of the mobile base station relay. Therefore, GNSS positioning can use the legacy positioning protocol.

### 6.7.2	Functional descriptions

A network node on which UE performs its positioning measurements needs a known position. In case the network node is a mobile base station relay it may change its location, so it is important that the position of the mobile base station relay node is known for the time period the UE performs measurements on the node.

Thus, there are different cases that needs to be taken into account:

-	When a UE performs positioning measurements based on signals from the mobile base station relay and possibly other base stations gNBs the current position of the mobile base station relay needs to be known by the LMF and therefore the LMF may need to request positioning measurements of the mobile base station relay.

-	When a UE only can perform measurements on the mobile base station relay i.e. no or not enough neighbouring gNBs to measure on, it may use the sidelink positioning procedures as investigated in FS_Ranging_SL TR 23.700-86 [15]. This solution will not go into details of this solution as it is handled within FS_Ranging_SL.

To support the UEs location procedures, the Mobile base station relay needs to get a good location estimate. The solution is based on the following principles:

-	Reuse of the LPP TS 37.355 [13] for DL/UL positioning messages to the UE served by the mobile base station relay.

-	It is assumed that the mobile base station relay does not always transmit a Positioning Reference Signal (PRS). Therefore, the donor-IAB RAN node will need to send a control message to the mobile base station relay to turn on the PRS.

-	The donor-IAB RAN node will need to inform the LMF that it has a IAB-DU node that is a mobile base station relay. In Rel-17 the IAB-DU node was assumed to be static at a known location.

-	The LMF will need to perform location determination of the mobile base station relay in case the UE reports positioning data from the mobile base station relay, that the donor-IAB RAN node reported as a mobile base station relay.

-	The Mobile base station relay will then act like a UE using the LPP protocol.

### 6.7.3	Procedures

#### 6.7.3.1	MT_LR for UE served by a mobile base station relay

This procedure on MT-LR positioning consists of two parts, first the procedure for UE positioning from TS 23.273 [4] utilizing the mobile base station relay as a RAN node and then the IAB-UE positioning using the UE positioning procedures in TS 23.273 [4].

Figure 6.7.3.1-1: Procedure for MT_LR for UE served by a mobile base station relay

0.	The AMF serving UE is triggered to initiate location reporting procedure for UE as specified in TS 23.273 [4]. The Mobile Base station relay have registered with the network, and during the registration the doner-IAB RAN node provided the IAB indication to the AMF.

NOTE 1:	The Mobile Base Station Relay and UE may be registered and served by different AMFs.

1.	The LMF, selected by the AMF serving UE, is triggered to initiate location reporting procedure for the UE. The AMF provides the LCS correlation ID and the cell-ID of the serving cell, i.e. the serving cell is the MBSR.

2.	The LMF sends a NRPPa message to the RAN node to turn on the PRS broadcasting. Based on this message the RAN node will instruct all IAB nodes including the mobile base station relay to turn on the PRS broadcasting. The LMF may also query which TRPs that are connected to the RAN node. The response from the RAN node may include an indication of which cell-ID(s) that are associated with a Mobile Base Station Relay.

NOTE 2:	Whether the LMF needs to query RAN node using dedicated signalling i.e. the TRP Information request about the TRPs to receive the cell-ID of any mobile base station relays or the this could be combined with other NRPPa message is up to RAN to specify.

NOTE 3:	The message between the donor-IAB RAN node and the mobile base station relay should be defined by RAN. In the figure above the message is indicated as NRPPa message, as this was the trigger for the donor-IAB RAN node.

3.	The RAN node informs the LMF that at least one of the nodes is a mobile base station relay and includes the cell-ID of mobile base station relay and the 5G-GUTI of the mobile base station relay in the response message.

NOTE 4:	The 5G-GUTI and the indication that a TRP/cell-ID is associated with a mobile base station relay is a new information element that RAN needs to confirm.

4.	The LMF sends a DL positioning message to the UE. The LMF may also have provided the UE with assistance data that includes relevant cells in the area i.e. the Area-ID-CellList.

5.	The UE performs positioning measurements on the mobile base station relay and other neighbouring cells.

6.	The UE send the UL positioning message including the positioning data captured by the UE in step 5. The data set may include one or more cell-ID(s) with positioning data associated with a mobile base station relay.

7.	If the LMF has received positioning data linked to one or more mobile base station cell-ID(s), the LMF resolves the AMF serving the mobile base station relay, from the 5G-GUTI received in step 3, and sends Namf_Location_ProvidePositioningInfo Request to the AMF to trigger the AMF to initiate the location reporting procedure for the mobile base station relay.

Steps 6-9 in 5GC-MT-LR procedure for the regulatory location service in clause 6.1.1 in TS 23.273 are performed. The AMF sends Namf_Location_ProvidePositioningInfo Response to the LMF including the MBSR location estimation. The LMF determines the location of UE using the positioning data set and the estimated mobile base station relay(s) location.

8.	The LMF provides the UE location to the AMF serving the UE.

### 6.7.4	Impacts on services, entities, and interfaces

LMF:

-	Support receiving 5G-GUTI of a mobile base station relay from a RAN node.

-	Support using the 5G-GUTI to determine the serving AMF and trigger location estimation of the mobile base station relay.

-	Support determining the UE location based on the positioning data received by the UE combined with the location estimates of the mobile base station relay(s).

RAN:

-	Support providing the 5G-GUTI of the mobile base station relay to the LMF to indicate that the cell-ID/TRP is a mobile base station relay.

* End of change *