3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206724r01

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	CATT

Title:	KI#3: Evaluation and conclusion

Document for:	Approval

Agenda Item:	9.12

Work Item / Release:	FS_5GSATB / Rel-18

Abstract: An evaluation and conclusion on KI#3 is proposed.

1. Introduction/Discussion

This contribution proposes an evaluation and conclusion on KI#3 (Support of Local Data Switching via UPF on-board).

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-27.

# * * * * First change * * * *

7	Overall Evaluation

7.x	Evaluation of solutions for Key Issue #3: Support of Local Data Switching via UPF on-board

There are 3 candidate solutions (solutions #4, #6, #7) proposed to solve how to enable local data switching via UPF on-board:

-	Solution #4 requires the UPF onboard to act as a UL-CL/BP and an additional PSA, so that UE to UE traffic can be routed to the additional PSA on the satellite, and then forwarded to the target UE. The SMF determines whether the two UEs which requests UE-UE communication are severed by same DNAI mapped from the GEO satellite ID provided by AMF without creating group. If the two UEs are severed by same DNAI, the SMF inserts the on-board UPF as UL CL/BP and local PSA for the two UEs according to the DNAI. When different SMFs are used for the two UEs, the UDR is involved in above procedure to judge whether two UEs are under same DNAI.

-	Solution #6 requires the UPF onboard to act as a PSA which is pre-configured with IP address pool per DNN, and the UPFs onboard are configured with particular PDR and FAR pairs, so that the UE to UE traffic can be local routed automatically without creating group. It’s assumed that the same SMF is used and inter-GEO UE-UE communication is supported.

-	Solution #7 also proposes the UPF onboard to act as a UL-CL/BP and an additional PSA, but requires to support QoS flow level traffic switching at the UL-CL/BP, and establish N19 tunnel between two additional PSAs, so that the UE to UE traffic will be routed to target UE belonging to same 5GLAN VN group according to the PDR and FAR. The same SMF is assumed in this solution.

# * * * * Second change * * * *

8	Conclusions

8.x	Conclusions on Support of Local Data Switching via UPF on-board

It is recommended that :

-	The UE to UE traffic can be locally routed by UPF on-board to the target UE without traversing back to the satellite gateway on the ground;

-	Only a single SMF case has been considered in this specification;

-	The UPF on-board can act as a PSA with configured IP address pool to enable local data switching as specified in solution #6, where SMF creates a UPF-level N4 rule on the on-board UPF to enable local data switching, and PCF determines URSP rule;

-	The UPF on-board can act as a UL-CL/BP and an additional PSA to enable local data switching at either QoS flow level or IP flow level;

-	N19 tunnel can be established between two UPFs on-board different satellites to locally switch UE to UE traffic.

# * * * * End of changes * * * *