3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206724r02

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	CATT, Huawei, Hisilicon

Title:	KI#3: Evaluation and conclusion

Document for:	Approval

Agenda Item:	9.12

Work Item / Release:	FS_5GSATB / Rel-18

Abstract: An evaluation and conclusion on KI#3 is proposed.

# 1. Introduction/Discussion

This contribution proposes an evaluation and conclusion on KI#3 (Support of Local Data Switching via UPF on-board).

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-27.

# * * * * First change * * * *

# 7	Overall Evaluation

## 7.x	Evaluation of solutions for Key Issue #3: Support of Local Data Switching via UPF on-board

There are 3 candidate solutions (solutions #4, #6, #7) proposed to solve how to enable local data switching via UPF on-board:

-	Solution #4 requires the UPF onboard to act as a UL-CL/BP and an additional PSA, so that UE to UE traffic can be routed to the additional PSA on the satellite, and then forwarded to the target UE. The SMF determines whether the two UEs which requests UE-UE communication are severed by same DNAI mapped from the GEO satellite ID provided by AMF without creating group. If the two UEs are severed by same DNAI, the SMF inserts the on-board UPF as UL CL/BP and local PSA for the two UEs according to the DNAI. When different SMFs are used for the two UEs, the UDR is involved in above procedure to judge whether two UEs are under same DNAI.

-	Solution #6 requires the UPF onboard to act as a PSA which is pre-configured with IP address pool per DNN, and the UPFs onboard are configured with particular PDR and FAR pairs, so that the UE to UE traffic can be local routed automatically without creating group. It’s assumed that the same SMF is used and inter-GEO UE-UE communication is supported.

-	Solution #7 also proposes the UPF onboard to act as a UL-CL/BP and an additional PSA, but requires to support QoS flow level traffic switching at the UL-CL/BP, and establish N19 tunnel between two additional PSAs, so that the UE to UE traffic will be routed to target UE belonging to same 5GLAN VN group according to the PDR and FAR. The same SMF is assumed in this solution.

# * * * * Second change * * * *

# 8	Conclusions

## 8.x	Conclusions on Support of Local Data Switching via UPF on-board

It is concluded that :

Only a single SMF is supported for Local Data Switching and N19 via UPF on-board, i.e. both UEs are served by the same SMF.

On-board PSA UPF or ULCL/BP UPF selection is performed as following (Same mechanism can be reused to KI#2):

-	For each GEO satellite ID, a DNAI value is assigned by the operator. SMF is configured with DNAI corresponding to each GEO satellite ID. The SMF determines DNAI based on the configuration and GEO satellite ID received from AMF.

-	On-board UPF selection is done based on the DNAI as current UPF selection defined in TS 23.501 [2] and TS 23.502[3].

For on-board UPF acting as PSA case:

-	A combination of DNN/S-NSSAI is assigned by the operator to the services deployed on GEO satellite. Based on GEO satellite backhaul category from AMF, PCF determines URSP rule including the DNN/S-NSSAI for such services. PCF sends the URSP rules to UEs allowed to access the services on GEO satellite. (Same mechanism can be reused to KI#2)

-	The UE to UE traffic may be locally routed by UPF on-board to the target UE without traversing back to the satellite gateway on the ground;

-	The UPF on-board may act as a PSA with configured IP address pool to enable local data switching as specified in solution #6, where SMF creates a UPF-level N4 rule on the on-board UPF to enable local data switching;

-	The UPF on-board may act as a UL-CL/BP and an additional PSA to enable local data switching;

-	N19 tunnel may be established between two UPFs on-board different satellites to locally switch UE to UE traffic.

# * * * * End of changes * * * *