3GPP TSG SA WG2 Meeting #152-e	 S2-2206736r01

# Electronic Meeting, Aug. 17 – 26, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3697 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Access Network selection for 5G NSWO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Broadcom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NSWO_5G, TEI17 |  |  |  |  |  | Date: |  |  | 2022-08-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As presented in more detail in S2-2206732, it is not possible for a UE to determine whether a WLAN supports AAA interworking with EPC and/or 5GC in a PLMN. Therefore, when the UE wants to connect to a WLAN using its 3GPP credentials, it cannot decide whether it should initiate the “Authentication and Key Agreement” procedure specified in TS 33.402, or the “NSWO authentication” procedure specified in TS 33.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to define a new PLMN list that can be advertised by a WLAN access network: - A PLMN List-5, which includes PLMNs with which "AAA connectivity to 5GC" is supported. A WLAN access network supports "AAA connectivity to 5GC" with a PLMN when it deploys an AAA function that can connect with a NSWOF in this PLMN. It is also proposed to define how the UE selects a WLAN access network and a PLMN before initiating the 5G NSWO authenticatio procedure (aka "WLAN connection using 5G credentials without 5GS registration"). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE cannot determine which WLANs support 5G NSWO and may initiate connection with a WLAN using 5G credentials which will fail. More details can be found in S2-2206732. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.12b (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** For Information *****

### 6.3.12	Trusted Non-3GPP Access Network selection

[…]

### 6.3.12a	Access Network selection for devices that do not support 5GC NAS over WLAN

[…]

***** Start of Changes *****

### 6.3.12b	Access Network selection for 5G NSWO

In addition to the PLMN lists specified in clause 6.3.12 and in clause 6.3.12a, a WLAN access network may also advertise the following PLMN list:

-	A PLMN List-5, which includes PLMNs with which "AAA connectivity to 5GC" is supported. A WLAN access network supports "AAA connectivity to 5GC" in a PLMN when it deploys an AAA function that can connect with a NSWOF in this PLMN. The NSWOF supports "WLAN connection using 5G credentials without 5GS registration", as defined in clause 4.2.15.

When the UE wants to connect to a WLAN access network using the "WLAN connection using 5G credentials without 5GS registration" procedure, the UE may retrieve the PLMN List-5 advertized by each discovered WLAN access network and may consider this list for selecting the WLAN access network to connect to. For example, if the UE identifies that the HPLMN is included in the PLMN List-5 advertized by a WLAN access network, the UE may select this WLAN access network to connect to using the "WLAN connection using 5G credentials without 5GS registration" procedure.

***** End of Changes *****