SA WG2 Meeting #152e	S2-2206739

# August 17th – 26th, 2022; Elbonia          	  		(revision of S2-220)

Source: 	Lenovo

Title: 	Conclusions for KI#4: KI#4: Influencing UPF and EAS (re)location for collections of UEs

Document for: 	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_eEDGE_Ph2 / Rel-18

Abstract of the contribution: Conclusions for KI#4: Influencing UPF and EAS (re)location for collections of UEs

- Discussion

Void.

2 Proposal

It is proposed to update TR 23700-48 as follows

Start change

7	Evaluation

Editor's note:	This clause will capture the evaluations related to the solutions per KI.

7.x	Evaluation for KI#4: Influencing UPF and EAS (re)location for collections of UEs

For KI#4, the key points to address the issues include:

- 1) How to define and identify a collection of UEs forming a dynamic ad-hoc group that should use the same EAS and/or same local part of DN and/or same DNAI. The dynamic ad-hoc group, agreed between the 5GC and the AF (e.g. as part of a SLA) can be managed only by AF or be synchronized by both AF and 5GC using agreed targeting information.

- 2) How to inform the SMF the information of the same EAS and/or same local part of DN and/or same DNAI to be used. The information of the same EAS and/or same local part of DN and/or same DNAI to be used can be determined by the AF and sent to the 5GC for maintaining and using, or can be determined by the 5GC and maintained within 5GC. For the AF-determined DNAI/EAS, it can be sent to the UE via application layer which is out of scope of 3GPP SA2, it can also be sent to the 5GC for the routing decisions for traffic of PDU Session and the DNS handling rule decision.

Following solutions address KI 4

| 14: Group Management | 58 | reuse the 5GVN group mechanism and consider inter-UDM case (the UDR assigns the Internal Group ID to ensure the unique of the internal group ID.) |
| --- | --- | --- |
| 15: Selection of common DNAI | 62 | SCMF (Session Collection Management Function) is introduced for making the decision for and EAS (re)location for collections of UEs |
| 16: Selecting the same EAS/DNAI for collection of UEs | 70 | the SMF make the decision that the UE is associated with the collection of UEs and use the same EAS or same DNAI, and the case when multiple SMFs are involved for the SMF synchronizing the DNAI/EAS with that in the UDR. Correlation ID is sent from AF and delivered to the SMF via PCC rule. |
| 17: Application layer EAS selection for collections of UEs | 73 | AF retrieves the candidate DNAI(s)for each UE of a list of UEs (via PCF and then interact with SMF) and the AF decides the target DNAI and EAS and AF notifies selected EAS via application layer, and target DNAI to 5GC. |
| 18: Discovery of the same EAS for collections of UEs | 75 | the pre-condition, configured to the SMF reusing AF request without any change, is used by the SMF to associate the PDU session with the collection of UEs using the same DNAI/EAS. |
| 19: Influencing UPF and EAS (re)location for collections of UEs | 79 | Dynamic group management with Group type and collection of UEs IDs forming the group or group property such as all users served by the same EAS. |
| 34: Selecting the same EAS/DNAI for collection of UEs | 116 | based on solution 16, EDI with external group ID per DNAI and updating the EDI with the selected DNAI/EAS, the external ID works as correlation ID in solution 16. |
| 35: Providing dedicated (re)location information as traffic routing information | 119 | the selected DNAI/EAS is informed to the SMF as traffic routing information. |
| 36: Providing dedicated (re)location information as EAS Deployment information | 123 | the selected DNAI/EAS informed to the SMF as EDI information. |
| 37: (Re)location of same EAS and coordination across UEs | 126 | based on solution 19, proposes the interaction between the SMF and UDM for the same EAS ID information alignment. |

In 3GPP TS 23.548, the mechanism to support “Edge Relocation Triggered by AF” is defined. The AF may invoke the AF request targeting an individual UE address procedure as described in TS 23.502 [3] clause 4.3.6.4 (the AF influence on traffic routing procedure), due to EAS relocation.

In 3GPP TS 23.548 and 23.502, The solution for “EAS (Re-)discovery over Session Breakout Connectivity Model ” is described. The SMF interacts with EASDF and provides the ECS option or the Local DNS server address, related to candidate DNAI(s) for that FQDN for the UE location, as part of the rules to handle DNS queries from the UE to the EASDF, the EASDF sends DNS message to the local DNS server or sends DNS message to the centralized DNS server with ECS option. The rules to handle DNS queries from the UE can be updated based on the notified DNS Query with certain FQDN, UE mobility or PSA relocation. For the selection of the candidate DNAI for a FQDN for the UE, the SMF may consider the UE location, network topology, EAS Deployment Information and related policy information for the PDU Session provided as defined in TS 23.503 [4] clause 6.4 or be preconfigured into the SMF. The EAS Deployment Information used to construct the rule can be set per group.

Thus, to reuse the existing mechanism as much as possible and solve the issue for KI#4, for 1) to define and identify a collection of UEs:

- if the collection of UEs is defined and identified by AF, if the interaction between AF and 5GC is per UE, and the correlation information can be used to associate all the UEs within the same collection;

- if the collection of UEs is defined and defined and identified by both AF and 5GC, the dynamic group management between the AF and the 5GC and within the 5GC are required. The interaction between AF and 5GC can be per collection of UEs. Wherein, the group managing dynamically with the extending 5GVN group management mechanism can be applied. Alternatively, to save the complexity to update each user subscription while the group member changes dynamically, the Targeting UEs information used for KI#3 can be applied.

- UE list, or group ID or any UE and Spatial Validity Condition could be used for UE collection definition of specific application, which is identified by FQDN or Application ID.

For 2) to inform the SMF the information of the same EAS and/or same local part of DN and/or same DNAI to be used,  the following ways should be included:

- the SMF retrieves the information from the PCF via pcc rule, which is received from the AF via traffic routing information per UE or per the collection of UE as described above.

- the SMF retrieves the information from the EAS Deployment Information for the collection of UE as described above.

- the SMF selects the target DNAI and stores it, or the SMF gets the target EAS using the existing EAS discovery procedure and stores it.

- the DNAI/EAS, stored in the SMF for other PDU sessions within the collection, is applied to the PDU session with the correlation information.

- the SMF retrieves/updates the target DNAI/EAS from/to the UDM.

2nd change

8	Conclusions

Editor's note:	This clause will capture the conclusions of the study.

8.x	Conclusions for KI#4: Influencing UPF and EAS (re)location for collections of UEs

KI#4 shall be concluded by the three aspects: UE collection definition, 5GC selection of common EAS/DNAI, AF selection of common EAS/DNAI.

UE collection definition.

The UEs in the UE collection are identified by a UE list, or ,group ID or any UE may be used. and Optionally, Spatial Validity Condition, or Target UEs with service information may could be used to limit the UEs in the UE collection to a specific area.

UEs in the UE collection shall use the same application. 5GC identifies the application based on the FQDN(s) used in the DNS query.

For group ID case, the dynamic group management is used to identify the UEs in the UE collection. For dynamic group management the existing NEF service is enhanced or a new NEF service is defined. UDR is used to store the group id and the UE identities in the group.

Nnef_ParameterProvision service is used for 5G VN UE group management, and the service is enhanced for dynamic 5G VN group management.

5GC selection of common EAS/DNAI.

All the solutions above are based on Rel-17 EAS discovery procedure, and use a 5GC NF to manage the common EAS/DNAI. This is the consensus of the solutions for 5GC selection of common EAS/DNAI.

For this aspect, it is proposed that:

- 5GC selection of the common EAS/DNAI for UE collection shall be supported.

- UDR is used for maintaining storing the common EAS/DNAI for UE collection.

- Add the parameter indication of EAS/DNAI correlation in Traffic Influence (defined in clause 5.6.7 of TS 23.501 [x]) to indicate the UE collection assessing the same EAS/DNAI.

-	The SMF identifies the collection of UEs and decides the common DNAI/EAS for them.

- For option C, the procedures in TS 23.548, The SMF select the common DNS server to resolve the FQDN in DNS query from for collection of UEs.

- If a common EAS IP address is required by the application, the EASDF responses directly with the IP address to the DNS Query.

AF selection of common EAS/DNAI.

As an option, AF could determine the common EAS/DNAI and provide the common EAS/DNAI to 5GS. For this aspect, it is proposed that:

- AF selection of the common EAS/DNAI for UE collection shall be supported.

- AF determined DNAI/EAS for UE collection shall be sent to the 5GC for the routing decisions for traffic of PDU Session and the DNS handling rule decision.

End of change