3GPP TSG-SA WG2#152E e-meeting	S2-2206879r011

Elbonia, 17 – 26 August 2022	(was S2-220xxxx)

Source:	Qualcomm Incorporated, AT&T, Charter Communications, Lenovo

Title:	KI#5: Evaluation and conclusions

Document for:	Discussion/Approval

Agenda Item:	9.19

Work Item / Release:	FS_EDGE_Ph2/Rel-18

Abstract of the contribution: This paper proposes evaluations and conclusions for KI#5 (GSMA OPG impacts and improvements for EHE operated by separate party).

1.	Discussion

This paper summarizes some of the solutions addressing Key Issue #5 and proposed a way forward.

2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-48:

# >>>>BEGINNING OF CHANGES<<<<

7	Evaluation

Editor's note:	This clause will capture the evaluations related to the solutions per KI.

7.x	Key Issue #5

The solutions addressing Key Issue #5 can be split into two categories: 1) solutions that enable the discovery of an EAS deployed in another PLMN than the one serving the UE, and 2) solutions that facilitate the edge relocation between PLMNs.

Solutions to enable the discovery of an EAS deployed in another PLMN than the one serving the UE.

This category of solutions encompasses Solution 20, Solution 21, Solution 22, Solution 38 and Solution 40. All solutions assume that the involved PLMNs are part of an GSMA OP federation and/or that there is a controlled inter-PLMN IP connectivity between the PLMN serving the UE and the PLMN hosting the EAS.

Solution 20 is based on the usage of the newly defined Global EASDF which can directly resolve the DNS query sent by the UE to discover the shared EAS (which can be deployed in any PLMN) or can first resolve the DNS query sent by the UE to discover the EASDF able to provide the IP address of the shared EAS. Both options require the UE to be configured with the Global EASDF’s URL and to resolve it to obtain the Global EASDF’s IP address. Because of that these two options are inconsistent with the Rel-17 EAS discovery procedure described in TS 23.548 clause 6.2.3.2.2 (which, on the contrary, assumes that the EASDF’s IP address is provided by the SMF via ePCO during PDU session establishment/modification). In addition, the second option is incompatible with the Rel-17 EAS discovery because it requires the UE to obtain the EASDF’s IP address by sending a DNS query to the Global EASDF, while the Rel-17 EASDF in the PLMN has no role to play.

Solution 22 describes three alternative options: Option 0 (based on the SMF configuration to know the EAS deployment information of the EAS running on other PLMN's edge infrastructure, e.g., IP address range(s)/FQDN(s)), Option 1 (based on the usage of the Shared EASDF which stores the EDI of each EAS running in each PLMN sharing Edge Nodes and Option 2 (which relies on inter-PLMN inter-EASDF communication to resolve the DNS Query for an EAS hosted by another PLMN). Option 2 requires storing in the NEF/UDR/SMF/EASDF an additional filtering information (PLMN ID and DNS server address/FQDN filter) for the EAS' deployed in another PLMN – such information is essential to locate the EAS during the EAS discovery procedure. Solution 22 has no impact on the UE, but, depending on the option, it affects the CN entities to different degrees.

Solution 38 relies on the AF to provide extended EAS Deployment Information (EDI) which includes N6 traffic routing information to the PLMN hosting the EHE. After having been translated by the NEF of the original PLMN and sent to the PLMN serving the UE, the extended EDI is stored in the UDR of the PLMN serving the UE. The SMF of the serving PLMN can then use such EDI to find the EAS hosted in the other PLMN. Similarly to Solution 22, this solution has no UE impacts.

Solution 40 has a similar concept to Option 2 of Solution 22, in that it forwards the DNS query from the EASDF of the PLMN serving the UE to the EASDF of the PLMN hosting the EHE, if the former cannot find it. Differently from Solution 22 though, the inter-EASDF communication takes place via SMF and NEF. In addition, Solution 40 proposes the use of additional information (S-NSSAI, DNN, UE location and External Group ID) to allow the SMF in the PLMN hosting the EHE to find the proper EASDF.

Solution 21 ensures that an EAS deployed by a certain operator in a VPLMN’s EHE can only be discovered and accessed by the UE of that operator while roaming in local breakout to the VPLMN. To do this, the solution the AF to provide, together with the EAS Deployment Information, the PLMN ID of the EAS owner so that the SMF can match it with the UE’s HPLMN ID when the EAS discovery is triggered. This solution, however, does not seem to address the ENS (Edge Node Sharing) deployment which allows OP B to deploy Edge Computing applications in the OP A's EHE, without restricting the access to it to only OP B’s UEs.

Solutions to enable the EAS relocation for shared EHE.

Solution 23 proposes to support the scenario of EAS relocation between EHE deployed by a source EHE provider to another EHE deployed by a target EHE provider, even in scenarios when EHEs are operated by different service providers. It needs the NRF or NEF maintains parameters for each AF, e.g. EHE domain it supports, service area, etc. Then SMF selects suitable target AF. The impact is introduced on 5GC and no impact on UE.

Solution 39 proposes to support the scenario EAS relocation between EHEs deployed in different PLMNs. The AF sends corresponding PLMN ID to network to indicate the EAS should be relocated to the target PLMN, and based on retrieval of PLMN ID, SMF selects UPF to access target EAS via controlled IP network. The impact is introduced on 5GC and no impact on UE.

# >>>>NEXT CHANGE<<<<

8	Conclusions

Editor's note:	This clause will capture the conclusions of the study.

8.x	Key Issue #5

To enable the discovery of an EAS deployed in another PLMN than the one serving the UE (e.g., an EAS deployed in a shared EHE),:

Editor's note: It is FFS based on what assumption and what information related to the edge application in a shared EHE (the supported FQDNs and/or the AF related information for sol#23, and/or the EASDF information for sol#22 and #40, and/or the DNS servers and/or the DNAI list for sol#38, and/or the EAS IP ranges for each PLMN for sol#39) is provided to the serving PLMN.

NOTE: For both EAS Discovery Procedure and Edge Relocation, the principal should be kept aligned for what information related to the edge application in a shared EHE is provided to the serving PLMN. The corresponding solution is decided based on what information related to the edge application in a shared EHE is provided to the serving PLMN.

EAS Deployment Provision

- UE’s serving PLMN gets the information to support UE visit the shared EHE.

Editor's note: It is FFS based on what assumption and what information related to the edge application in a shared EHE (the supported FQDNs and/or the AF related information for sol#23, and/or the EASDF information for sol#22 and #40, and/or the DNS servers and/or the DNAI list for sol#38, and/or the EAS IP ranges for each PLMN for sol#39) is provided to the serving PLMN.

Editor’s note: It is FFS which NF performs the translation of specific information (e.g. DNAI) from different PLMNs if required.

EAS Discovery Procedure

- EAS discovery procedure defined in TS23.548 [3] will be reused.

Editor’s note: It is FFS whether all the methods defined in TS23.548 will be reused in this GSMA OPG scenario.

Traffic transmission between two PLMNs

- When the traffic transmitted between two PLMNs, the UPF in serving PLMN should support DSCP marking to guarantee the low latency transmission.

Edge Relocation

Editor’s note: It is FFS how to support the Edge Relocation in GSMA OPG scenario.

NOTE: For both EAS Discovery Procedure and Edge Relocation, the principal should be kept aligned for what information related to the edge application in a shared EHE is provided to the serving PLMN. The corresponding solution is decided based on what information related to the edge application in a shared EHE is provided to the serving PLMN.

# >>>>END OF CHANGES<<<<