3GPP SA WG2 Meeting #152E	S2-2207103

Elbonia, August 17 - 26, 2022	(was S2-2205522)

Source:	Ericsson

Title:	KI#1, 4: Update to solution 16

Document for:	Discussion/Approval

Agenda Item:	9.17

Work Item / Release:	FS_NG_RTC

Abstract of the contribution: This contribution proposes an update to solution 16.

# 1. Proposal

It is proposed to accept the following changes to TR 23.700-78.

FIRST CHANGE

## 6.16	Solution #16: Solution for IMS Media Support for DC Server

### 6.16.1	Description

In this solution, the MRF is enhanced to support the IMS DC feature. To provide the DC Signalling Function (DCSF) with the enhanced MRF Egress information for Bootstrap data as an example, as well as receive the DCSF bootstrapping addressing information, the IMS AS interacts with the DCSF for that purpose.

Figure 6.16.1-1 depicts a high level architecture diagram for the enhanced MRF solution showing the architecture of the IMS for supporting the IMS Data Channel feature, illustrating the interworking between IMS and other entities needed to support DC feature in this option.

In this architecture, the IMS MRF is enhanced to support additional functionality related to DCSF and other data channel media. This functionality is referred to as Data Channel Media Function (DCMF) while maintaining backward compatibility, and reusability. MRFs that are expected to support DC feature shall be enhanced for that purpose.

NOTE:	Not all deployed MRFs needs to be enhanced to support DC feature.

In this architecture, the '3rd Party External Web Application' provides content relayed over HTTP only (IF8/IF1) while the '3rd Party WebRTC enabled Application' provides application media content relayed over established WebRTC 1.0 RTCDataChannels (IF10). The '3rd Party External Web Application' is consequently representing a standard web server which extends bootstrap media with external content. The '3rd Party WebRTC enabled Application' on the other hand, establishes peer media connectivity using WebRTC data channels.

Figure 6.16.1.1-1includes the following interfaces:

-	IF1 between the enhanced MRF and DCSF for media transport related to bootstrapping. What is transported over IF1 is only relevant to the UE. IF1 is HTTP based.

-	IF10 between the enhanced MRF and 3pty Web RTC Applications. IMS is transparent to the data channel payload media carried on this interface.

-	IF6 between the IMS AS and DCSF for managing DC feature related to IMS data channel including media control. This interface hast to be extensible and support vendor specific information with no impact on interoperability. IF6 is service based.

-	IF8 between the DCSF and External Web servers. Interface is used to access existing web application servers that could be accessed by the downloaded Web application over the bootstrapping data channels. The interface has to be extensible and support vendor specific elements.

-	DCSF includes multiple functions. The control function for DCSF decides usage of data channel media within the sessions, interacting with both the IMSAS and third-party WebRTC applications. The Bootstrap media function terminates HTTP bootstrap channels established with call sessions, hereby taking the role of a standard web server, potentially acting as a proxy for HTTP resources hosted by external third-party webservers. The DC Application repository includes bootstrap media content fetched by TS 26.114 [6] UE's.

-	MRF (MRFC/MRFP) includes both legacy media resources (e.g. Voice and Video) and data channel media function (DCMF) resources.

IF1 and IF6 are in scope of the TR.

IF8 and IF10 are out of scope of the TR.

Nimsas implements services supported by the IMS AS.

Ndcsf implements services supported by the DCSF.

Editors note:	Nimsas, and Ndcsf are FFS.

Figure 6.16.1-1: IMS DC Enhanced MRF Architectural Option

### 6.16.2	Procedures

#### 6.16.2.1	IMS Session Procedures

The call flow in Fig 6.16.2.1-1 illustrates the various interactions within an IMS DC related session related to the DC feature. Interactions triggered by UEs related to the actual initiation of bootstrapping are not included.

Figure 6.16.2.1-1 DC Channel Establishment

The following is a brief description of the steps in the call flow:

-	In step 1, a DCMTSI UE registers in IMS after successfully establishing an IMS PDU session.

- 	In step 2, the UE establishes an audio IMS session with a DCMTSI UE of the terminating network, i.e. both UE’s have indicated support for the IMS DC feature by including the feature tag +sip.app-subtype="webrtc-datachannel” in the Contact header

-	In step 3, the originating user initiates upgrade of the established session to add data channel content, which results in the DCMTSI UE sending a session re-negotiation request with local and remote data channel streams added to the SDP as per TS 26.114 [6]. It is here assumed that both the originating and terminating networks provides IMS DC content.

-	In step 4, the IMS AS notifies the DCSF about a session modification request event of served user including information about offered data channel streams. DCSF apply policies and check for applicable content that may require bootstrapping.

Editor’s Note: The exact nature of that interaction is FFS

-	In step 5, the DCSF instructs the IMS AS on how each offered data channel stream of served user shall be handled, i.e. passed-on to the terminating network, removed or be anchored in the enhanced MRF. In this example, the DCSF requests IMS AS to anchor the stream including local bootstrapping and pass-on the stream including remote bootstrapping.

Editor’s Note: The exact nature of this interaction is FFS.

-	In step 6, the IMS AS selects an enhanced MRF supporting the DCMF feature and requests the MRF to allocate resources for the specified data channel stream.

-	In step 7, the IMS AS informs DSCF that anchoring of the local bootstrap stream was successful, so session modification procedure can continue.

Editor’s Note: The exact nature of that interaction is FFS.

-	In step 8, the session modification procedure is continued. The SDP is adjusted as per DCSF request, to exclude the data channel stream anchored in the MRF before the session re-negotiation request is sent to the terminating network.

-	In step 9, the terminating network responds to the session modification request. In this call flow it is also assumed that the terminating network offers remote bootstrapping, which means that the offered data channel stream for remote bootstrapping is accepted.

-	In step 10, the IMS AS notifies the DCSF of the session modification request answer event, including data channel stream information of the SDP answer.

Editor’s Note: The exact nature of that interaction is FFS.

-	In step 11, The DCSF may at this point depending on the remote answer, request the IMS AS to update the enhanced MRF with additional instructions on how DC streams shall be handled within the session.

-	In step 12, the IMS AS updates the SDP towards the UE as per DCSF request, so that the IMS AGW is updated with correct media anchoring information related to the bootstrap data streams.

-	In step 13, the SIP ACK is sent to the target.

As can be seen, as examples, a) the local Bootstrap data of served user traverses through enhanced MRF to the DCSF, b) the remote Bootstrap of served user, offered by the Terminating NW, bypasses the enhanced MRF

-	In step 14, the remote user may take the initiative to upgrade the session with DC content, which would result in the remote DCMTSI UE, initiating a session modification, adding local and remote bootstrapping streams.

#### 6.16.2.2	IMS AS - DC Server Interaction for Media control

Editor's note:	This interface is FFS.

### 6.16.3	Impacts on existing nodes and functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

END OF CHANGES