3GPP TSG-SA2 Meeting #154	S2-2210532

# Toulouse, France, Nov 14 - 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3786 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CAG based control of UE access to 5GS via MBSR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to FS_VMR KI#7 conclusion in TR 23.700-05, the existing CAG mechism can be used to control the access of UE via MBSR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description that CAG Identifier is used to control the access of UE via MBSR (i.e. mobile IAB-node) and existing CAG mechanism can be used for managing UE's access to MBSR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Control of UE access to 5GS via MBSR cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35.x (new), 5.35.x.x (new), 5.35.x.x.1 (new), 5.35.x.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_VMR and the WI code for the work is not assigned yet & will replace WI code DUMMY when available |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.35.x	Support for Mobile Base Station Relay

5.35.x.x	CAG based control of UE access to 5GS via MBSR

5.35.x.x.1	General

When a UE connects to 5GS via a MBSR (a mobile IAB-node), the access control of the UE is managed by CN, RAN and UE. During the CM-CONNECTED mobility, the access control via MBSR (mobile IAB-node) is managed by CN and RAN.

RAN and CN shall support the UE access control based on the CAG identifier associated with the MBSR cell and the allowed CAG identifiers for the UE that supports CAG functionality.

UEs supporting CAG may select the cells covered by a MBSR based on the configured MBSR CAG identifier using the existing CAG mechanism defined in clause 5.30.3.4. The legacy UE(s) that support CAG shall also support to access the MBSR cell based on the CAG identifier.

The enhanced Allowed CAG list including time duration and location information may be provided to UE and AMF for enforcement to make sure UE not accessing the MBSR cell outside of the time duration or geographic area, e.g. if the time when a certain CAG is allowed for a UE is up or UE is out of the geographic area, the CAG for the UE is revoked from the network.

For the UEs that do not support CAG (i.e. the UE is not able to understand the broadcasted CAG information by the cell) but support the use of IAB architecture as defined in clause 5.35.1, they may select the cell as normal and rely on RAN and CN to control the IAB-node access using network side CAG control logic.

NOTE:	RAN and CN are allowed to use not only CAG mechanism but also the other existing mechanism e.g. forbidden Tracking Area.

5.35.x.x.2	CAG identifier of MBSR

If the MBSR is pre-configured with the PLMN list in which the MBSR (i.e. mobile IAB-node) is allowed to operate as an IAB node, the corresponding CAG Identifier per PLMN shall also be configured in the MBSR either during the communication with the serving PLMN OAM or pre-configured and announced by the cells covered by the MBSR.

The CAG identifier value of MBSR cell shall be different from the CAG identifier of normal cell and unique within the scope of a PLMN ID.

* * * * End of Changes * * * *