3GPP TSG-SA WG2 Meeting #156E	S2-2304026r01

# Electronic, April 17 - 21, 2023	                                                       (merge of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0728 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2- Removing Editor´s Note |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 04/07/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Conclusions of KI#2 states that a new Analytics ID will be discussed to be supported to monitor traffic in one or multiple PDU Session for the purpose to monitor if the UE enforces the URSP Rules as instructed by the PCF. However, the current NWDAF role is to provide analytics without knowing how the consumer will use them, as such each Analytics ID can be used in multiple procedures. The proposal is to generalize this Analytics ID to report the UE(s) run traffic as indicated by the consumer, and then the consumer knows how to use the Analytics ID. Editor´s Note on the Traffic Descriptors that can be provided by the consumer is extended with Application ID and Domain Descriptor. The Application ID is known at the UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove Editor´s Note on KI#2 to allow that the consumer can provide a Traffic Descriptor including an Application ID and a Domain Descriptor. Refer to the Traffic Descriptor provided by the consumer, since the NWDAF does not know if a URSP Rule was sent to the UE or not. The NWDAF provides analytics on traffic detected on the PDU Session, with further categorization whether it matches the Traffic Descriptor, and then the consumer knows what this Traffic Descriptor was used for. Extend the input data to list the information that Nupf_EventExposure provides when subscription to event “UserDataUsageMeasurest” is requested on PDU Session level, and request to report per Flow information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions of KI#2 are not specified. Editor´s Note remains |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.20, 6.20.1, 6.20.2, 6.20.3, 6.20.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * *First change * * * *

## 6.20	PDU Session traffic analytics

### 6.20.1	General

This clause specifies the procedure for an NWDAF to provide statistics on whether traffic of UEs via one or multiple PDU sessions is according to the information provide by the service consumer.

The NWDAF collects traffic flow information of UE traffic via PDU session(s) established for a specific S-NSSAI and/or DNN and provides statistics of UEs that route traffic according to the information provided by the service consumer (e.g., Traffic Descriptor, S-NSSAI, DNN)  and UEs that route traffic which is not expected according to the information provided by the service consumer (e.g., Traffic Descriptor, S-NSSAI, DNN)

The assumption is that Traffic Descriptors are known (e.g. known flow description), so that there are associated Packet Detection Rule(s) for the known traffic configured in the UPF.

NOTE 1:	How traffic from different applications over the same S-NSSAI/DNN and/or PDU session is discerned is implementation-specific.

The service consumer may be a PCF.

The consumer of these analytics includes in the request:

-	Analytics ID = "PDU Session traffic".

-	Target of Analytics Reporting: UE, group of UE or any UE.

-	Traffic Descriptor.

NOTE 2:	Application Id, IP Descriptions or Domain Descriptors are  applicable.

-	Analytics Filter Information containing:

-	Area of Interest (i.e. the location of UEs where PDU Session traffic is monitored);

-	S-NSSAI;

-	DNN;

-	An Analytics target period indicates the time period over which the analytics are requested.

-	Optionally, maximum number of objects.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

### 6.20.2	Input Data

NWDAF collects input data from the SMF and/or UPF. The detailed data are described in Table 6.20.2-1.

Table 6.20.2-1: Input data to be collected per PDU Session

| Information | Source | Description |
| --- | --- | --- |
| SUPI | SMF, UPF | UE ID for the UE that established PDU sessions to an S-NSSAI/DNN. |
| S-NSSAI | SMF, UPF | The S-NSSAI for which the PDU session is established. |
| DNN | SMF, UPF | The DNN for which the PDU session is established. |
| PDU Session Information |  | The detected Service Data Flow, volume measured and Application related information in a PDU Session |
| > Packet Filter Set | SMF, UPF | Packet Filter Set for the detected Service Data Flow as defined in clause 5.8.2.4.2 in TS 23.502. |
| > UL Data volume | SMF, UPF | Measured UL data traffic volume for the Service Data Flow for the duration of the PDU Session |
| > DL Data volume | SMF, UPF | Measured DL data traffic volume for the Service Data Flow for the duration of the PDU Session. |
| > Application Identifier | SMF, UPF | If available, the Application Identifier is reported |
| > URL list | SMF, UPF | If available, list of URLs extracted from the inspected user plane packets in the Service Data Flow |
| > Domain Name list | SMF, UPF | If available, list of domain names extracted from the inspected user plane packets in the Service Data Flow |

NOTE:	Care needs to be taken with regards to load and major signalling caused when requesting Any UE. This could be achieved via utilization of some event filters (e.g. Area of Interest), a specific DNN,S-NSSAI or sampling ratio as part of Event Reporting Information.

Editor's note:	Additional input data from AF and other Data Sources are FFS.

The NWDAF collects input data from the UPF either indirectly via the SMF, or directly from the UPF using the "UserDataUsageMeasures" event exposure event as described in clause 4.15.4.5 of TS 23.502 [3]. Further details about input parameters are described in Table 4.15.4.5.1 of TS 23.502 [3].

Editor's Note: 	Whether the NWDAF subscribes from the SMF/UPF to report any traffic via the PDU session or report only traffic which is not expected according to the information provided by the service consumer (e.g., Traffic Descriptor) depends on the conclusions in the normative work of the UPEAS work.

### 6.20.3	Output Analytics

The NWDAF collects input data from the UPF, via SMF when the request is for a UE or a group of UEs or directly from the UPF when possible (see TS 23.501 5.8.2.17) if the request applies for any UE and NWDAF provides analytics of UEs about the traffic they route over a PDU session of specific S-NSSAI/DNN according to the Traffic Descriptor provided by the consumer and traffic they route over a PDU Session that is not according to the Traffic Descriptor provided by the consumer, and information about that traffic

The output analytics is shown in Table 6.20.3-1.

Table 6.20.3-1: PDU Session traffic statistics

| Information | Description |
| --- | --- |
| List of SUPIs or SUPI, | Identifies a SUPI, or a list of SUPIs for which analytics are provided. |
| S-NSSAI | Identifies the Network Slice for which analytic information is provided. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| Traffic matching the Traffic Descriptor | Identifies traffic that matches Traffic Descriptor provided by the consumer in those PDU Sessions identified by the S-NSSAI and DNN above and the volume. |
| > Traffic Descriptor | IP Flow descriptor containing 3-tuple, server side (destination address, port, and protocol) or Application ID or Domain descriptor. |
| > Volume | measures of data volume exchanged (UL, DL and/or overall) and/or number of packets exchanged (UL, DL and/or overall) for the Traffic Descriptor within the PDU Session. |
| Traffic which does not match Traffic Descriptor | Identifies traffic that does not match Traffic Descriptor provided by the consumer in those PDU Sessions identified by the S-NSSAI and DNN above and the volume. |
| > Traffic Descriptor | IP Flow descriptor containing 3-tuple, server side (destination address, port, and protocol) or Application ID or Domain descriptor. |
| > Volume | measures of data volume exchanged (UL, DL and/or overall) and/or number of packets exchanged (UL, DL and/or overall). |

NOTE:	Predictions are not provided

### 6.20.4	Procedures

The procedure for deriving PDU Session traffic  is shown below.

Figure 6.20.4-1: NWDAF providing URSP enforcement analytics

1.	The Consumer NF (e.g., the PCF) requests or subscribes to the NWDAF to request URSP Enforcement analytics. The consumer includes within analytics filter the expected traffic via a PDU session according to the Traffic Descriptors  and the S-NSSAI/DNN of the PDU session. It is assumed that there are associated Packet Detection Rule(s) for the expected traffic listed in the analytics request, i.e. the expected traffic is known.

2.	The NWDAF determines to collect data, either directly from the UPF or indirectly via the SMF and identifies the SMF and/or UPF to retrieve input data according to the S-NSSAI/DNN and UE information in the analytics request. The NWDAF retrieves data from the UPF where the UE has established a PDU session to the DNN/S-NSSAI included in the analytics request.

3.	The NWDAF collects traffic information using the "UserDataUsageMeasures" event of UPF event exposure service. The NWDAF requests volume measurements for the PDU Session with per flow granularity.

4.	The NWDAF derives analytics indicating a list of UEs and the traffic they route according to the provided information provided by the consumer (i.e. Traffic Descriptor, S-NSSAI and DNN), including its volume and the traffic they route that it is not expected according to the information provided by the consumer (i.e. Traffic Descriptor, S-NSSAI and DNN) including its volume .

5.	The NWDAF notifies URSP enforcement analytics.

# * * * * End of changes * * * *