3GPP TSG-SA2 Meeting #156 (e-meeting)	S2-2304033r04

# Elbonia, April 17th – 21st, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0732 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solving EN in the Procedure for Maintaining Federated Learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Solving the EN in clause 6.2C.2.3: “What service will be used by Server and Client NWDAFs to terminate Federated Learning operation is FFS.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | To solve the EN, the following changes are made: - Added Nnwdaf_MLModelTraining_Notify service operation with Termination Request to step 1b for Client NWDAF(s) informing the Server NWDAF its leaving of the Federated Learning process. - Added Nnwdaf_MLModelTraining_Unsubscribe service operation to step 4 for Server NWDAF sending terminate request to the Client NWDAF(s) which will be removed from the Federated Learning process. - Update clause 7.10.3 accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for maintaining Federated Learning is incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.2 (duplicated, corrected to 6.2C.2.3), 7.10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

6.2C.2.3	Procedures for Maintaining Federated Learning Processes

This clause specifies how to maintain a Federation Learning process in FL execution phase, including Server NWDAF triggers reselection, addition, or removal of Client NWDAF(s), discovers new Client NWDAF(s) via NRF, and Client NWDAF(s) joins or leaves Federated Learning process dynamically.

In Federated Learning execution phase, Server NWDAF monitors the status changes of Client NWDAF(s), and may reselects Client NWDAF(s) based on the updated status, availability, and/or capability, etc.

NOTE 1:	Server NWDAF checks if there is a need to carry on the FL execution phase and then reselects FL members for the next iteration if needed.

Figure 6.2C.2.3-1: Procedure of Server NWDAF reselects Client NWDAF(s), Client NWDAF(s) Join or Leave Federated Learning Process Dynamically in Federated Learning execution phase

The procedure for Server NWDAF reselecting Client NWDAF(s), Client NWDAF(s) joining or leaving Federated Learning process dynamically is as follows:

0.	Server NWDAF registers into NRF about the Federated Learning procedure with the following parameters:

-	Analytics ID.

1a.	Server NWDAF may get the updated status of current Client NWDAF(s) via NRF by using Nnrf_NFManagement service (as in clause 5.2.7.2 of TS 23.502 [3]) in the Federated Learning execution phase.

Server NWDAF may subscribe to NRF for notifications of status changes of the current NWDAF(s) (Client NWDAFs 1…N) by invoking an Nnrf_NFManagement_NFStatusSubscribe service operation. NRF notifies the Server NWDAF the status changes of the current Client NWDAF(s) by invoking Nnrf_NFManagement_NFStatusNotify service operation(s).

The status of a current Client NWDAF could be availability changes, capability changes (e.g. it will not support FL anymore, etc.).

1b. The current Client NWDAF(s) may inform Server NWDAF that it is leaving the Federated Learning process by invoking Nnwdaf_MLModelTraining_Notify service operation with Termination Request and cause code (reason for leaving, e.g. high NF load, time availability changes).

1c.	Server NWDAF may get the information of the new Client NWDAF(s) dynamically via NRF by subscribing to the event that a new Client NWDAF registers (Nnrf_NFManagement_NFStatusSubscribe service as in clause 5.2.7.2 of TS 23.502 [3]).

1d.	NWDAF may subscribe for NF load analytics of the Client NWDAF(s).

1e.	Client NWDAF(s) may report status of FL training including accuracy of local model and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension, etc.).

Editor's note:	It is FFS whether and what and how (i.e. using which service) Training Input Data Information is provided from the Client NWDAF to the Server NWDAF.

When the Model Accuracy Check Flag is present in the Nnwdaf_MLModelTraining_Subscribe request or Nnwdaf_MLModelTrainingInfo_Request service operation, Server NWDAF gets the Model Accuracy of the global ML Model calculated by Client NWDAF(s) using the local training data as the testing dataset. If the Model Accuracy calculated by a Client NWDAF is much different from that calculated by other Client NWDAFs or that by Server NWDAF itself when it has enough data, Server NWDAF may force this Client NWDAF to quit current Federated Learning process by Nnwdaf_MLModelTraining_Unsubscribe request service operation.

2.	Server NWDAF checks Client NWDAF(s) status based on the received information, and may determine whether reselection of Client NWDAF(s) for the next round(s) of Federated Learning is needed based on the received information from step 1.

NOTE 2:	Several examples of the factors that the Server NWDAF can consider to reselect the Client NWDAF(s) are updated status of Client NWDAF reported by NRF is different than the criteria were initially used for selecting the client; characteristics of local training dataset is different than global validation dataset and accuracy of the local model is lower than the accuracy of the global model; the load of the Client NWDAF (from the NF load Analytics or from the client NWDAF directly) is high; the Server NWDAF receives leave request from the Client NWDAF; the Client NWDAF reports the status of FL Training within the maximum response time.

3.	[If re-selection is needed as judged in step 2] If step 1c is not performed, Server NWDAF may discover new candidate Client NWDAF(s) via NRF by using Nnrf_NFDiscovery services as in clause 5.2.7.3 of TS 23.502 [3]. Server NWDAF reselects Client NWDAF(s) from the current Client NWDAF(s) and the new candidate Client NWDAF(s) (found in steps 1c or 3). For the new candidate Client NWDAF(s), the interaction between Server NWDAF and Client NWDAF(s) is same as the selection procedure described in clause 6.2C.2.1. The adding / deleting Client NWDAF(s) may happen at the end of each iteration.

4.	Server NWDAF sends termination request by invoking Nnwdaf_MLModelTraining_Unsubscribe service operation or Nnwdaf_MLModelTrainingInfo_Request service operation with Correlation Termination Flag to the Client NWDAF(s), optionally indicating the reason, e.g. FL Client NWDAF is unselected by the FL Server NWDAF for the FL process, or the FL process is suspended, etc. And FL server may also send the updated global ML model information to the unselected FL client NWDAF with recommend operations for it to be qualified in participation of FL training in the next cycles. Client NWDAF(s) terminates operations for the Federated Learning process if receive termination request from the Server NWDAF and may perform further action based on the updated global ML model information to be qualified in participation of FL training in the next cycles.

NOTE 3:	In the case of high load, the Client NWDAF can e.g. decline new service request. In the case of low accuracy, the Client NWDAF can gather new local training data.

* * * Next Changes * * *

7.10.3	Nnwdaf_MLModelTraining_Unsubscribe service operation

Service operation name: Nnwdaf_MLModelTraining_Unsubscribe

Description: Terminate NWDAF ML model training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: Cause code (e.g. FL Client NWDAF is unselected by the FL Server NWDAF for the FL process, or the FL process is suspended or finished, etc.). Final aggregated ML model information (if FL has finished) or updated aggregated ML model information (if FL is suspended)

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

* * * End of Changes * * *