3GPP TSG-SA2 Meeting #156-e	S2-2304069r01

# Online, , 17th Apr 2023 - 21st Apr 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3959 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Forwarding of UE reporting of URSP rule enforcement between SM-PCF and UE-PCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  |  |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2#155, the support of UE reporting of URSP rule enforcement for deployments where the PCF serving the PDU session is not the same as the PCF serving the UE was left FFS. Furthermore, concerns were raised regarding using parameter provisioning, with some views expressed that reusing the procedure already present for application detection would be preferred |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to reuse the existing procedure in 4.16.14.2.1 for forwarding the information between SM-PCF and UE-PCF. For clarity reasons, it was decided to write a new clause. Adding the required information to the existing clause or writing a new clause based on describing changes to the existing clause would result in convoluted text that would not be clear to the reader, worst case breaking existing text |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality in deployments where the PCF serving the PDU session is not the same as the PCF serving the UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.X (New), 5.2.5.1, 5.2.5.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 23.503 CR 0947 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 4.16.X |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.16.x	Management of URSP rule based on awareness of URSP rule enforcement

4.16.x.1	General

The procedure for management of URSP rule depending on the URSP rule enforcement needs to let the PCF for the UE be aware of URSP rule enforcement.

The policy control decisions in the PCF for the UE may be performed with or without UE assistance as defined in clause 6.1.6.

The content of this clause applies to both non-roaming and HR roaming cases. The PCF for the UE represents the H-PCF in roaming cases.

If PCF for the UE and PCF for the PDU Session are the same PCF, then step 5, 6 and 8 in Figure 4.16.x.2-1 are not performed.

If the PCF for the UE and the PCF for the PDU Session are different PCFs, then the PCF for the UE is informed when the PCF for the PDU Session receives URSP rule enforcement.

4.16.X.2	Procedures for management of URSP rule based on awareness of URSP rule enformcement

This procedure applies when the PCF for the PDU Session receives URSP rule enforcement during PDU Session Establishment or PDU Session Modification procedure.

Figure 4.16.X.2-1: Management of URSP rule based on awareness of URSP rule enforcement

1.	The UE registers to 5GC, as described in clause 4.2.2.2.2. During the UE Policy Association Establishment procedure, If the UE indicates capability of support to report URSP rule enforcement to network, the PCF for the UE request the AMF to indicate the future PCF(s) for the PDU Session to notify the URSP rule enforcement.

2.	The PCF for the UE registers its information to the BSF by invoking Nbsf_Management_Register service operation.

3.	The UE initiates PDU Session Establishment/Modification procedure based on URSP matching result. If the Traffic Descriptor of the matched URSP rule contains Connection Capabilities and the UE is capable of reporting URSP rule enforcement to the network, the UE includes the corresponding Connection Capabilites within the PDU Session Establishmen or Modification request message towards the SMF.

4. The SMF initiates SM Policy Association Establishment or Modification procedure as defined in clasue 4.16.4 or 4.16.5.1 respectively. The PDU Session parameters and the UE reported Connection Capabilities is included the request message, if available.

5. Based on the received Connection Capabilities or local policies, the PCF for the PDU Session may determine to contact with the PCF for the UE to inform the URSP rule enforcement information.

If the PCF ID of the PCF for the UE is not available yet, it invokes Nbsf_Management_Discovery to the BSF to retrieve the PCF ID of the PCF for the UE. Alternatively, if the PCF for the PDU Session receives the request sent by the PCF for the UE in step 1 via AMF and SMF, then step 5 is skipped and step 6B will be performed.

6A. The PCF for the PDU Session invokes Npcf_PolicyCombination_Create or Npcf_PolicyCombination_Update request to the PCF for the UE, by including URSP rule enforcement information in the request message, which may contain Connection Capabilities reported from the UE if available, detected application events if available and the PDU Session parameters, e.g. DNN, S-NSSAI, SSC mode, etc.

6B. The PCF for the PDU session invokes Npcf_PolicyAuthorization_Notify request to the PCF for the UE indicated by the PCF binding information received from the SMF to notify the URSP rule enforcement report, by including the same URSP rule enforcement information as depicted in step 6A.

Based on the received URSP rule enforcement inforamtion, the PCF for the UE checks the the PDU Session parameters is consistent with the list of URSP rules sent to the UE, and provides one or more traffic descriptor corresponds to the URSP rule enforcement to the PCF for the PDU Session as input for PCC rule generation.

7a. If the PCF for the UE determines to adjust the URSP rules based on the request message, it triggers UE Policy Association modification procedure as defined in clause 4.16.12.2, and updates the URSP rules in the UE.

7b. Based on the traffic descriptor in the response message, the PCF for the PDU Session generates PCC rules as defined in clause 6.1.6.1 of TS 23.503[20].

8. When the PCF for the PDU Session receives updated Connection Capabilities from the UE, or it receives newly detected application event, it invokes the Npcf_PolicyCombination_Update request to the PCF for the UE.

When the PDU Session is released, then the PCF for the PDU Session invokes the Npcf_PolicyCombination_Delete request to the PCF for the UE.

# * * * * Next change * * * *

5.2.5.1	General

The following table illustrates the PCF Services.

Table 5.2.5.1-1: NF services provided by PCF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Npcf_AMPolicyControl | Create | Request/Response | AMF |
|  | Update | Request/Response | AMF |
|  | UpdateNotify | Subscribe/Notify | AMF |
|  | Delete | Request/Response | AMF |
| Npcf_Policy Authorization | Create | Request/Response | AF, NEF, TSCTSF |
| (NOTE 1) | Update | Request/Response | AF, NEF, TSCTSF |
|  | Delete | Request/Response | AF, NEF, TSCTSF |
|  | Notify | Subscribe/Notify | AF, NEF, NWDAF, PCF, TSCTSF |
|  | Subscribe |  | AF, NEF, NWDAF, PCF, TSCTSF |
|  | Unsubscribe |  | AF, NEF, NWDAF, PCF, TSCTSF |
| Npcf_SMPolicyControl | Create | Request/Response | SMF |
|  | UpdateNotify | Subscribe/Notify | SMF |
|  | Update | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
| Npcf_BDTPolicyControl | Create | Request/Response | NEF |
|  | Update | Request/Response | NEF |
|  | Notify |  | NEF |
| Npcf_UEPolicyControl | Create | Request/Response | AMF, V-PCF |
|  | Update | Request/Response | AMF, V-PCF |
|  | UpdateNotify | Subscribe/Notify | AMF, V-PCF |
|  | Delete | Request/Response | AMF, V-PCF |
| Npcf_EventExposure | Subscribe | Subscribe/Notify | NEF, NWDAF |
|  | Unsubscribe |  |  |
|  | Notify |  |  |
| Npcf_AMPolicyAuthorization | Create | Request/Response | AF, NEF, TSCTSF |
|  | Update | Request/Response | AF, NEF, TSCTSF |
|  | Delete | Request/Response | AF, NEF, TSCTSF |
|  | Notify | Subscribe/Notify | AF, NEF, 5G DDNMF |
|  | Subscribe |  | AF, NEF, 5G DDNMF |
|  | Unsubscribe |  | AF, NEF, 5G DDNMF |
| Npcf_PDTQPolicyControl | Create | Request/Response | NEF |
|  | Update | Request/Response | NEF |
|  | Notify |  | NEF |
| Npcf_PolicyCombiniation (NOTE 1) | Create | Request/Response | PCF |
|  | Update | Request/Response | PCF |
|  | Delete | Request/Response | PCF |
| NOTE: In the Npcf_PolicyAuthorization and Npcf_PolicyCombination operations, PCF is a consumer when the PCF for the UE and the PCF for the PDU session are different. |  |  |  |

# * * * * Next change * * * *

5.2.5.X	Npcf_PolicyCombination Service

5.2.5.X.1	General

Service description: This service allows the PCF for the PDU Session interact with the PCF for the UE for combined policy decision scenario when the PCF for the UE and PCF for the PDU Session are separately deployed, which includes the following functionalities:

-	triggers the check for UE reported Connection Capabilities, and retrieves the traffic descriptor of the traffic allowed to occur.

5.2.5.X.2	Npcf_PolicyCombination_Create service operation

Service operation name: Npcf_ PolicyCombination _Create

Description: This service is to create the combined policy association in the PCF for the UE.

Inputs, Required: SUPI.

Inputs, Optional: Connection Capabilities, DNN, S-NSSAI, SSC mode, PDU Session Type, Acceees Type, RSN and/or PDU Session Pair ID, detected application event.

Outputs, Required: Combined Policy Association ID, One or more traffic descriptor of the traffic allowed to occur in the PDU Session.

Outputs, Optional: None.

5.2.5.X.3	Npcf_ PolicyCombination _Update service operation

Service operation name: Npcf_ PolicyCombination _Update

Description: This service is to update the combined policy association in the PCF for the UE.

Inputs, Required: Combined Policy Association ID.

Inputs, Optional: Connection Capabilities, DNN, S-NSSAI, SSC mode, PDU Session Type, Acceees Type, RSN and/or PDU Session Pair ID, detected application event.

Outputs, Required: None

Outputs, Optional: None.

5.2.5.X.4	Npcf_ PolicyCombination _Delete service operation

Service operation name: Npcf_ PolicyCombination _Notify

Description: This service is to terminate the combined policy association in the PCF for the UE.

Inputs, Required: Combined Policy Association ID.

Inputs, Optional: None.

Outputs, Required: Success or Failure.

Outputs, Optional: None.

# * * * * End of changes * * * *