3GPP TSG SA WG2 Meeting #156-E	S2-2304111r01

# Electronic meeting, April 17-21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3965 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addressing Editor’s note related to AF requested QoS for a group of UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is an Editor’s Note in 4.15.6.14: Editor's note: The scenario without TSCTSF can be revisited based on the progress of other work items. Whether an additional signalling route for the direct PCF case is to be supported is FFS. No work in other work items seems to impact the conclusions for GMEC. There is a discussion on aligning the northbound NEF API, but that discussion is not dependent on this EN. It is therefore proposed to remove the EN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove EN by clarifying: NEF determines whether to provide the AF request QoS information for a group to TSCTSF or PCF based on operator configuration, i.e. described in step 3 in clause 4.15.6.6. same for PCF. When the NEF or PCF determines to use TSCTSF for non-TSC case, then there is no need to use separate signalling route for non-TSC case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Remaining open issue |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

4.15.6.14	Procedures for AF requested QoS for a UE or group of UEs not identified by a UE address

Figure 4.15.6.14-1: Procedure for AF requested QoS for a UE or group of UEs not identified by a UE address

0.	When a new PDU Session is established, based on local configuration associated with the DNN/S-NSSAI, the PCF determines if the PDU Session needs involvement of TSCTSF. If the TSCTSF is used the PCF invokes Npcf_PolicyAuthorization_Notify service operation to the TSCTSF discovered and selected as described in clause 6.3.24 of TS 23.501 [2]. The Npcf_PolicyAuthorization_Notify service operation includes the UE address of the PDU Session and DNN/S-NSSAI. If the TSCTSF is not used, the PCF subscribes to notifications for Application Data from UDR.

NOTE:	In the case of private IPv4 address being used for the UE, the DNN and S-NSSAI are required for session binding in the PCF.

The PCF registers to BSF as described in TS 23.503 [20]. When the TSCTSF is used, the TSCTSF invokes a Npcf_PolicyAuthorization_Create request message to the PCF and stores the DNN, S-NSSAI and IP address as received from PCF and SUPI as received from BSF and associates them with the AF-session.

1.	The AF sends a request to reserve resources using Nnef_AF_Request_QoS_Create request message (GPSI or External Group ID, AF Identifier, Flow description(s) or External Application Identifier, QoS reference or individual QoS parameters, Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20]), DNN, S-NSSAI) to the NEF. Optionally, a period of time or a traffic volume for the requested QoS can be included in the AF request. The AF may, instead of a QoS Reference, provide the individual QoS parameters. Regardless, whether the AF request is formulated using a QoS Reference or Individual QoS paramaters, the AF may also provide one or more of the parameters that describe the traffic characteristics as described in clause 6.1.3.23 or 6.1.3.23a of TS 23.503 [20]. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as described in clause 6.1.3.22 of TS 23.503 [20].

2.	This step is the same as step 2 in clause 4.15.6.6.

3.	This step is the same as steps 2-4 in clause 4.15.6.10.

The NEF determines whether to invoke the TSCTSF or not, as described in step 3 in clause 4.15.6.6. If the NEF determines to invoke TSCTSF, steps 5-9 are executed and steps 10-12 are skipped. Otherwise steps 5-9 are skipped and steps 10-12 are executed. The procedure then continues in step 13.

4.	This step is the same as step 3a in clause 4.15.6.6 with the difference that Internal Group ID or SUPI is provided instead of UE address.

5.	The TSCTSF determines the individual group members using Nudm SDM Group Identifier translation service. The TSCTSF also determines which of these group members have active PDU Sessions matching the DNN/S-NSSAI and determines the relevant UE address. The TSCTSF manages the AF request QoS information targeting for a group as defined in TS 23.501 [2].

6.	For each PDU Session, the TSCTSF invokes Npcf_PolicyAuthorization service. This step is the same as step 3b in clause 4.15.6.6.

7.	The PCF replies to the TSCTSF. This step is the same as step 4a in clause 4.15.6.6.

8.	The TSCTSF replies to the NEF. This step is the same as step 4b in clause 4.15.6.6.

9.	This step is the same as step 6 in clause 4.15.6.6 with the difference that it is executed for all PDU Sessions identified in step 6 above.

10.	If the NEF determines to not invoke the TSCTSF, the NEF uses the Nudr_DM service to store the information related to the Internal Group ID or SUPI in UDR. The information is stored as Application Data in UDR. If the AF requested for notifications of Resource allocation status or other events, the NEF includes the information required for reporting the event, including the Notification Target Address pointing to the NEF or AF and the Notification Correlation ID containing the AF Transaction Internal ID.

11.	The UDR notifies the PCF(s) that have subscribed.

12.	The PCF(s) identifies the active PDU Sessions associated with the data received from UDR. The PCF(s) manages the AF request QoS information targeting for a group as defined in TS 23.501 [2].

13. The NEF replies to the AF. This step is the same as step 8 in clause 4.15.6.6.

14. When an event condition is met, e.g. that the establishment of the transmission resources corresponding to the QoS update succeeded or failed, the PCF sends a notification to TSCTSF or NEF as applicable. This step is the same as steps 7-8 in clause 4.15.6.6.

**** End of Changes ****