3GPP TSG-SA WG2 Meeting #156E (e-meeting)	S2-2304180r01

# Elbonia, 17 – 21 April, 2023												 		 (revision of S2-xxxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4227 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IAB authorization change handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, HiSilicon, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The authorization change handling is still FFS currently. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the DP S2-2304179, it’s proposed to introduce the following change: -Specify that network triggers Deregistration with re-registration required when IAB authorization chnages for a registered IAB-UE. -Add mechanism to trigger the Deregistration from network side when IAB authorization changes to not authorized. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN remains unsolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

### 5.35A.4	MBSR authorization

For a MBSR, the subscription information stored in the HPLMN indicates whether it is authorized to operate as MBSR, and the corresponding location and time periods.

When MBSR roaming is supported, a roaming agreement between VPLMN and HPLMN regarding MBSR operation is in place, and the AMF can make use of subscription data for authorization of MBSR in VPLMN. The MBSR (IAB-DU) can use IAB-node integration procedure or inter-IAB-donor gNB mobility procedure to integrate into VPLMN to provide service.

The MBSR(IAB-UE) is assumed to be configured with preferred PLMN lists and forbidden PLMNs by the HPLMN for the MBSR operation.

Editor's note:	Configuration mechanism will be further described based on development of clause 5.35A.2.1.

When the MBSR (IAB-UE) performs initial registration with the serving PLMN, it indicates the request to operate as a MBSR as described in clause 5.35A.1. The AMF authorizes the MBSR based on the subscription information, and provides MBSR authorized indication to NG-RAN as specified in registration procedure in TS 23.502 [3]. The MBSR establishes the connection to OAM system using the configuration information for MBSR operation.

Editor's note:	The detailed support of inter-IAB-donor gNB mobility is FFS and needs coordination with RAN WGs.

NOTE 1:	How the MBSR obtains the configuration information for MBSR operation is described in clause 5.35A.2.1.

After registered to the 5G system, the MBSR remains in CM-CONNECTED state if the MBSR operation is authorized. The MBSR establishes the connection to OAM system using the configuration information for MBSR operation.

If the MBSR operation is not authorized (e.g. due to location or time limitation), the AMF may reject the MBSR (IAB-UE)’s registration or de-register the MBSR. Or the AMF may initiate UE Context setup procedure or UE Context Modification procedure by providing MBSR-Operation allowed indication with the value set to “not authorized” to the NG-RAN, but the the MBSR is still registered.

The AMF of the MBSR can indicate to the MBSR (IAB-UE) that it is not allowed to act as an MBSR node as part of registration procedure. The AMF may provide the indication either in a Registration Accept (if the PLMN allows the MBSR (IAB-UE) to be registered in the PLMM) or in a Registration Reject (if the PLMN does not allow the MBSR (IAB-UE) to be registered in the PLMN). When MBSR authorization state changes for a MBSR (IAB-UE) registered in network (either authorized, or not authorized), The network triggers Deregistration procedure with re-registration required. If the authorization state changes from authorized to not authorized, the AMF triggers Deregistration procedure based on local configuration (e.g. based on the expiration of a timer started by AMF when NG-RAN is informed the IAB-node is non-authorized).

NOTE 2:	The AMF delays the MBSR de-registration to allow the IAB-donor gNB to move all connected UEs via MBSR to other cells as specified in clause 8.9.10 of TS 38.401 [42].

NOTE 3:	The mechanism applies to both roaming and non-roaming MBSR operations.

****************** END CHANGES ***************