3GPP TSG-WG SA2 Meeting #156-e	S2-2304319

# Elbonia, April 17 – 21, 2023		(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0213 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update functional entity clauses for group message delivery and power saving functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For group message delivery and UEs using power saving functions,the enhancement to the functional entities are not reflected in clause 5.3.3 accordingly. Besides, the following NOTE in clause 5.3.2.11 needs an update to refer to the SA4 spec. Similar update is also needed in clause 5.3.2.12 NOTE 1: MBSF functionality related to service and MBS data handling (e.g. encoding) is to be determined with SA WG4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the description for group message delivery and power saving Update the notes in clauses 5.3.2.11 and 5.3.2.12 to refer to SA4 spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing the features of group message delivery and UE using powering saving functions in TS 23.247. Misalignment between SA2 and SA4 specs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.8, 5.3.2.9, 5.3.2.10, 5.3.2.11, 5.3.2.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Second change * * * *

#### 5.3.2.8	UE

In addition to the functions defined in TS 23.501 [5], the UE may perform the following functions to support MBS:

-	Reception of multicast data using PTM/PTP in RRC_CONNECTED state.

-	If supported, reception of multicast data using PTM in RRC_INACTIVE state.

-	Reception of broadcast data using PTM.

-	Handling of incoming MBS QoS flows.

-	Support of signalling for joining and leaving a Multicast MBS session.

-	MBS resource management support at AS layer.

-	Reception of notification in CM-IDLE state and CM-CONNECTED with RRC _INACTIVE state for multicast data transmission.

-	Support of the aforementioned functions for  UE using power saving functions .

NOTE:	UE functionality for MBS security is provided in TS 33.501 [20].

# * * * * Third change * * * *

#### 5.3.2.9	AF

The AF performs the following functions to support MBS:

-	Requesting multicast or broadcast service from the 5GC by providing service information including QoS requirement to 5GC.

-	Instructing MBS session operation towards 5GC if needed.

-	Interacting with NEF for MBS related service exposure.

-	Interacting with NEF for group message delivery to the UE(s).

# * * * * fourth change * * * *

#### 5.3.2.10	NEF

In addition to the functions defined in TS 23.501 [5], the NEF performs the following functions to support MBS:

-	Providing an interface to AFs for MBS procedures including service provisioning, MBS session and QoS management.

-	Interacting with AF and NFs in 5GC, e.g. MB-SMF for MBS session operations, determination of transport parameters.

-	Selection of MB-SMF to serve an MBS Session.

-	Support of provisioning the optional MBS Session Assistance Information related to the reception of multicast MBS data in RRC_INACTIVE state.

-	Interacting with AF and MBSF/MBSTF for group message delivery.

# * * * * Fifth change * * * *

#### 5.3.2.11	MBSF

The MBSF performs the following functions to support MBS:

-	Service level functionality to support MBS, and interworking with LTE MBMS

-	Interacting with AF and MB-SMF for MBS session operations, determination of transport parameters, and session transport.

-	Selection of MB-SMF to serve an MBS Session.

-	Controlling MBSTF if the MBSTF is used.

-	Determination of destination IP multicast address for the MBS session if IP multicast address is sourced by MBSTF.

-

NOTE 1:	MBSF functionality related to service and MBS data handling (e.g. encoding) is specified in TS 26.502 [18].

NOTE 2:	MBSF functionality for MBS security is provided in TS 33.501 [20].

# * * * * Sixth change * * * *

#### 5.3.2.12	MBSTF

The MBSTF performs the following functions to support MBS if deployed:

-	Media anchor for MBS data traffic if needed.

-	Sourcing of IP Multicast if needed.

-	Generic packet transport functionalities available to any IP multicast enabled application such as framing, multiple flows, packet FEC (encoding).

-	Multicast/broadcast delivery of input files as objects or object flows.

NOTE 1:	MBSTF functionality related to MBS data handling (e.g. encoding) is specified in TS 26.502 [18] .

NOTE 2:	MBSTF functionality for MBS security is provided in TS 33.501 [20].

# * * * * End of changes * * * *