3GPP TSG-WG SA2 Meeting #156E 	S2-2304427r03

# April 17th – 21th, 2023	(Revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0755 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Federated Learning among Multiple NWDAFs in TS 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Lenovo, Ericsson, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_ph3 |  |  |  |  |  | Date: |  |  | 2023-04-05 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There seems no extra criteria, so the following EN can be removed: Editor's note: Additional criteria for FL server to discover FL clients from the NRF (e.g. data availability) is FFS. Particularly for maximum response time, in step 3, saying like the below: If the Client NWDAF is not able to complete the training within the maximum response time, the Client NWDAF notifies the Server NWDAF on the delay event before the maximum response time elapses. Therefore, the two ENs regarding maximum response time in step3 and 4 can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove 3 Editor’s Notes in clause 6.2C which already are resolved after SA2#155 meeting. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete functionality defined in Rel-18 eNA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.1, 6.2C.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.2C	Federated Learning among Multiple NWDAFs

6.2C.1	General

This clause specifies how NWDAF containing MTLF can leverage Federated Learning technique to train an ML model.

6.2C.2	Procedures

6.2C.2.1	Registration and Discovery procedure for Federated Learning

Figure 6.2C.2.1-1: Registration and Discovery procedure for Federated Learning

1-3.	NWDAF containing MTLF as FL Server NWDAF or Client NWDAF registers to NRF with its NF profile, which includes NWDAF NF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF, Service Area, FL capability type information (i.e. FL server or FL client) and Time interval supporting FL as described in clause 5.2.

4-6.	NWDAF containing MTLF determines ML model requires FL based on Analytic ID, Service Area/DNAI or data can not be obtained directly from data producer NF (e.g. due to privacy reasons).

If the NWDAF containing MTLF can not perform as Server NWDAF, the MTLF first discovers and selects FL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, Model filter information as defined in TS 23.288 [5], FL capability Type (i.e. FL server), check if the selected Server NWDAF is currently executing an Federated Learning procedure for the Analytics ID, Time Period of Interest, Service Area.

Once the FL Server NWDAF (the requested or the selected one) is determined, it discovers and selects other NWDAF(s) containing MTLF as Client NWDAF(s) from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, FL capability Type (i.e. FL client), Service Area, NF type(s) of data sources from which the Client NWDAF is able to collect data for FL training, Time Period of Interest, Interoperability Indicator.

7.	Server NWDAF sends Federated Learning preparation request to the Client NWDAF(s) with Interoperability information. In the preparation request, the following parameters may be added: Available data requirement, availability time requirement.

Editor's note:	Other parameters in the preparation request and the definition of "Available data requirement" and "availability time requirement" are FFS.

8.	Client NWDAF(s) decides whether to join the Federated Learning process based on implementation. Example criteria used by a Client NWDAF(s) may be based on its availability, computation and communication capability, and Interoperability information.

9.	Client NWDAF(s) sends response to FL Server NWDAF indicating if it will join the FL procedure.

10.	FL Server NWDAF conducts selection of Client NWDAF(s).

END of CHANGES