3GPP TSG-WG SA2 Meeting #156-e	S2-2304719r14

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4383 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve ENs for support of PDU Set handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia?, Nokia Shanghai Bell?, Samsung?, OPPO, MediaTek, Tencent, Tencent Cloud, Qualcomm Incorporated, Xiaomi? Lenovo? vivo, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It’s proposed to resolve the following ENs for support of PDU Set handling. - Clarify the usage of PSIHI and remove the following EN. Editor’s NOTE: Usage of PSIHI is FFS. Editor's Note: The PSER definition may be subject to change if RAN2 provides any feedback on that. Based on the LS from RAN2 (R2-2302010), the PSER only defines an upper bound for a rate of non-congestion related PDU Set losses and the usage of PSER is up to network implementation. Therefore, it’s proposed to remove the EN, update the PSER definition and also add a NOTE for clarification. - “Protocol Description” is used for per packet detection for PDU Set identification, therefore it should be moved to the Packet Detection Information part of the PDR. Editor's Note: The need for AN PSDB and definition of AN PSDB is FFS. The AN PSDB is used for the gNB to indicate the upper bound for the delay that a PDU Set may experience for the transfer between the UE and the NG-RAN. The PSDB is equal to the sum of the AN PSDB and the CN PDB and RAN should derive the AN PSDB based on the CN PDB accordingly. - The following EN is removed by quote to SA4 TS. Editor’s Note: How to document the UPF derivation of the PDU Set Information from the RTP/SRTP header or payload is FFS in coordination with SA4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve the existing ENs for support of PDU Set handling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ENs are not addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.7.1.2, 5.7.7, 5.8.2.4.2, 5.8.5.3, 5.37.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS); Stage 2".

[6]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface: Stage 3".

[7]	IETF RFC 7157: "IPv6 Multihoming without Network Address Translation".

[8]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[9]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[10]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[11]	ITU-T Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[12]	ITU-T Recommendation Q.65: "The unified functional methodology for the characterization of services and network capabilities".

[13]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS): Stage 3".

[14]	IETF RFC 3736: "Stateless DHCP Service for IPv6".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 22.173: "IMS Multimedia Telephony Service and supplementary services; Stage 1".

[17]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[18]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[19]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[20]	IETF RFC 7542: "The Network Access Identifier".

[21]	3GPP TS 23.002: "Network Architecture".

[22]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows; Stage 2".

[23]	3GPP TS 23.221: "Architectural requirements".

[24]	3GPP TS 22.153: "Multimedia priority service".

[25]	3GPP TS 22.011: "Service Accessibility".

[26]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[27]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[28]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[29]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[32]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[33]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[35]	3GPP TS 33.126: "Lawful Interception Requirements".

[36]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[37]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[38]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[39]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[40]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[41]	3GPP TS 32.240: "Charging management; Charging architecture and principles".

[42]	3GPP TS 38.401: "NG-RAN Architecture description".

[43]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[44]	IETF RFC 4960: "Stream Control Transmission Protocol".

[45]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[46]	3GPP TS 23.041: "Public Warning System".

[47]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[48]	3GPP TS 24.502: "Access to the 5G System (5GS) via non-3GPP access networks; Stage 3".

[49]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[50]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[51]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[52]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[53]	Void.

[54]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[55]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[56]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[57]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[58]	3GPP TS 29.510: "5G System: Network function repository services; Stage 3".

[59]	3GPP TS 29.502: "5G System: Session Management Services: Stage 3".

[60]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2) ".

[61]	3GPP TS 23.380: "IMS Restoration Procedures".

[62]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[63]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[64]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs".

[65]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[66]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[67]	3GPP TS 32.290: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[68]	3GPP TS 32.255: "5G Data connectivity domain charging; Stage 2".

[69]	3GPP TS 38.306: "NR; User Equipment -UE) radio access capabilities".

[70]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access -E-UTRA); User Equipment -UE) radio access capabilities".

[71]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[72]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[73]	IETF RFC 2865: "Remote Authentication Dial In User Service (RADIUS)".

[74]	IETF RFC 3162: "RADIUS and IPv6".

[75]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[76]	3GPP TS 26.238: "Uplink streaming".

[77]	3GPP TR 26.939: "Guidelines on the Framework for Live Uplink Streaming (FLUS)".

[78]	International Telecommunication Union (ITU), Standardization Bureau (TSB): "Operational Bulletin No. 1156"; http://handle.itu.int/11.1002/pub/810cad63-en (retrieved October 5, 2018).

[79]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[80]	3GPP TS 24.250: "Protocol for Reliable Data Service; Stage 3".

[81]	IETF RFC 8684: "TCP Extensions for Multipath Operation with Multiple Addresses".

[82]	IETF RFC 8803: "0-RTT TCP Convert Protocol".

[83]	IEEE Std 802.1CB-2017: "IEEE Standard for Local and metropolitan area networks-Frame Replication and Elimination for Reliability".

[84]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[85]	WiFi Alliance Technical Committee, Hotspot 2.0 Technical Task Group: "Hotspot 2.0 (Release 2) Technical Specification".

[86]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[87]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[88]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[89]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[90]	BBF TR-124 issue 5: "Functional Requirements for Broadband Residential Gateway Devices".

[91]	BBF TR-101 issue 2: "Migration to Ethernet-Based Broadband Aggregation".

[92]	BBF TR-178 issue 1: "Multi-service Broadband Network Architecture and Nodal Requirements".

[93]	BBF TR-456 issue 2: "AGF Functional Requirements".

[94]	BBF WT-457: "FMIF Functional Requirements".

Editor's note:	The reference to BBF WT-457 will be revised when finalized by BBF.

[95]	IEEE Std 802.1Qcc-2018: "IEEE Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[96]	Void.

[97]	IEEE Std 802.1AB-2016: "IEEE Standard for Local and metropolitan area networks -- Station and Media Access Control Connectivity Discovery".

[98]	IEEE Std 802.1Q-2018: "IEEE Standard for Local and metropolitan area networks--Bridges and Bridged Networks".

[99]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[100]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[101]	3GPP TS 29.274: "Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[102]	3GPP TS 23.632: "User Data Interworking, Coexistence and Migration; stage 2".

[103]	3GPP TS 29.563: "5G System (5GS); HSS services for interworking with UDM; Stage 3".

[104]	IEEE Std 802.1AS-2020: "IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications".

[105]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[106]	IEEE Std 802.11-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[107]	IEEE Std 1588-2008: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems".

[108]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[109]	3GPP TS 24.193: "Access Traffic Steering, Switching and Splitting; Stage 3".

[110]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[111]	3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios; Stage 1".

[112]	3GPP TR 38.824: "Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)".

[113]	IEEE: "Guidelines for Use of Extended Unique Identifier (EUI), Organizationally Unique Identifier (OUI), and Company ID (CID)", https://standards.ieee.org/content/dam/ieee-standards/standards/web/documents/tutorials/eui.pdf.

[114]	3GPP TS 32.256: "Charging Management; 5G connection and mobility domain charging; Stage 2".

[115]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[116]	3GPP TS 38.415: "PDU Session User Plane Protocol".

[117]	3GPP TS 24.535: "Device-side Time-Sensitive Networking (TSN) Translator (DS-TT) to network-side TSN Translator (NW-TT) protocol aspects; Stage 3".

[118]	3GPP TS 32.274: "Charging Management; Short Message Service (SMS) charging".

[119]	3GPP TS 23.008: "Organization of subscriber data".

[120]	3GPP TS 38.314: "NR; Layer 2 measurements".

[121]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[122]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[123]	3GPP TS 32.254: "Charging management; Exposure function Northbound Application Program Interfaces (APIs) charging".

[124]	3GPP TS 33.535: "Authentication and Key Management for Applications based on 3GPP credentials in the 5G System (5GS)".

[125]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[126]	IEEE Std 1588: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems", Edition 2019.

[127]	ST 2059-2:2015: "SMPTE Standard - SMPTE Profile for Use of IEEE-1588 Precision Time Protocol in Professional Broadcast Applications".

[128]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[129]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[130]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[131]	IEEE Std 802.3: "Ethernet".

[132]	3GPP TS 29.561: "5G System; Interworking between 5G Network and external Data Networks; Stage 3".

[133]	3GPP TS 29.513: "Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[134]	3GPP TS 23.558: "Architecture for enabling Edge Applications (EA)".

[135]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[136]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[137]	GSMA NG.116: "Generic Network Slice Template".

[138]	IETF RFC 3948: "UDP Encapsulation of IPsec ESP Packets".

[139]	3GPP TS 24.539: "5G System (5GS); Network to TSN translator (TT) protocol aspects; Stage 3".

[140]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic bootstrapping architecture".

[141]	3GPP TS 33.223: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) Push function".

[142]	3GPP TS 23.540: "Technical realization of Service Based Short Message Service; Stage 2".

[143]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[144]	3GPP TS 29.525: "5G System; UE Policy Control Service; Stage 3".

[145]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository Services for Subscription Data; Stage 3".

[146]	IEEE Std P802.1Qdj-d0.3: "IEEE Draft Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment XX: Configuration Enhancements for Time-Sensitive Networking".

[147]	3GPP TS 23.700-05: "Study on architecture enhancements for vehicle-mounted relays".

[X]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

# * * * * 2nd change * * * *

#### 5.7.1.2	QoS Profile

A QoS Flow may either be 'GBR' or 'Non-GBR' depending on its QoS profile. The QoS profile of a QoS Flow is sent to the (R)AN and it contains QoS parameters as described below (details of QoS parameters are described in clause 5.7.2):

-	For each QoS Flow, the QoS profile shall include the QoS parameters:

-	5G QoS Identifier (5QI); and

-	Allocation and Retention Priority (ARP).

-	Optionally the PDU Set QoS Parameters as described in clause 5.7.7.

-	For each Non-GBR QoS Flow only, the QoS profile may also include the QoS parameter:

-	Reflective QoS Attribute (RQA).

-	For each GBR QoS Flow only, the QoS profile shall also include the QoS parameters:

-	Guaranteed Flow Bit Rate (GFBR) - UL and DL; and

-	Maximum Flow Bit Rate (MFBR) - UL and DL; and

-	In the case of a GBR QoS Flow only, the QoS profile may also include one or more of the QoS parameters:

-	Notification control;

-	Maximum Packet Loss Rate - UL and DL.

NOTE:	In this Release of the specification, the Maximum Packet Loss Rate (UL, DL) is only provided for a GBR QoS Flow belonging to voice media.

Each QoS profile has one corresponding QoS Flow identifier (QFI) which is not included in the QoS profile itself.

The usage of a dynamically assigned 5QI for a QoS Flow requires in addition the signalling of the complete 5G QoS characteristics (described in clause 5.7.3) as part of the QoS profile.

When a standardized or pre-configured 5QI is used for a QoS Flow, some of the 5G QoS characteristics may be signalled as part of the QoS profile (as described in clause 5.7.3).

# * * * * 3rd change * * * *

### 5.7.7	PDU Set QoS Parameters

#### 5.7.7.1	General

PDU Set QoS Parameters are used to support PDU Set based QoS handling in the NG-RAN. The followings are the PDU Set specific QoS characteristics:

1.	PDU Set Delay Budget (PSDB).

2.	PDU Set Error Rate (PSER).

3.	PDU Set Integrated Handling Information (PSIHI).

For a QoS Flow supporting PDU Set based QoS handling, SMF indicates NG-RAN to activate the PDU Set based QoS handling, the QoS Profile includes the PDU Set QoS Parameters described in this clause (see clause 5.7.1.2). The PCF determines the PDU Set QoS Parameters based on information provided by AF and/or local configuration. The PDU Set QoS parameters are sent to the SMF as part of PCC rule. The SMF sends them to NG-RAN as part of the QoS Profile.

If the NG-RAN receives PDU Set QoS Parameters and supports them, it applies PDU Set QoS Parameters as described in this clause.

NOTE 1: Each PDU Set QoS parameters can be optional.

Editor's note:	[XRM] The applicability and details of PDU Set handling in uplink direction is pending RAN WG's progress.

#### 5.7.7.2	PDU Set Delay Budget

The PDU Set Delay Budget (PSDB) defines an upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, i.e. the duration between the reception time of the first PDU (at the N6 termination point for DL or the UE for UL) and the time when all PDUs of a PDU Set have been successfully received (at the UE for DL or N6 termination point for UL). PSDB applies to the DL PDU Set received by the PSA UPF over the N6 interface, and to the UL PDU Set sent by the UE.

NOTE:	To enable support for PSDB, it is required that a maximum inter arrival time between the first received PDU and the last received PDU of a PDU Set complies with SLA. This maximum inter arrival time does not exceed PSDB. NG-RAN behaviour when the SLA is not fulfilled is out of scope of this specification.

A QoS Flow is associated with only one PDU Set Delay Budget. The value of the PDU Set Delay Budget is the same in UL and DL. PSDB is an optional parameter that may be provided by the PCF. The provided PSDB can be used by the NG-RAN to support the configuration of scheduling and link layer functions.

When the PSDB is available, the PSDB supersedes the PDB.

#### 5.7.7.3	PDU Set Error Rate

The PDU Set Error Rate (PSER) defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). Thus, the PSER defines an upper bound for a rate of non-congestion related PDU Set error and losses.

NOTE 1:	In this Release, a PDU Set is considered as successfully delivered only when all PDUs of a PDU Set are delivered successfully.

NOTE 2:	How RAN enforces PSER is up to RAN implementation.

A QoS Flow is associated with only one PDU Set Error Rate. If the PSER is available, the usage of PSER supersedes the usage of PER. The value of the PDU Set Error Rate is the same in UL and DL.

#### 5.7.7.4	PDU Set Integrated Handling Information

The value of PDU Set Integrated Handling Information (PSIHI) indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side.

# * * * * 4th change * * * *

##### 5.8.2.4.2	Traffic Detection Information

The SMF controls the traffic detection at the UP function by providing detection information for every PDR.

For IPv4 or IPv6 or IPv4v6 PDU Session type, detection information is a combination of:

-	CN tunnel info.

-	Network instance.

-	QFI.

-	IP Packet Filter Set as defined in clause 5.7.6.2.

-	Application Identifier: The Application Identifier is an index to a set of application detection rules configured in UPF.

For Ethernet PDU Session type, detection information is a combination of:

-	CN tunnel info.

-	Network instance.

-	QFI.

-	Ethernet Packet Filter Set as defined in clause 5.7.6.3.

In this Release of the specification for Unstructured PDU Session Type, the UPF does not perform-QoS Flow level traffic detection for QoS enforcement.

Traffic detection information sent by the SMF to the UPF for a PDU Session may be associated with Network instance for detection and routing of traffic over N6. In the case of IP PDU Session Type, Network Instances can, e.g., be used by the UPF for traffic detection and routing in the case of different IP domains or overlapping IP addresses. In the case of Ethernet PDU Session Type, different Network Instances can e.g. be configured in the UPF with different ways to handle the association between N6 and the PDU Sessions.

Based on SMF instructions, UPF may identify the PDU Sets, according to the Protocol Description in PDR, to derive the PDU Set Information for DL traffics and send it to RAN via DL GTP-U header of each PDU identified as belonging to a PDU Set. The PDU Set Information, is described in clause 5.37.5. The PDU Set identification can be done by UPF implementation or by detecting RTP/SRTP header or payload. The details of the RTP/SRTP headers, header extensions and/or payloads used to identify PDU Sets are defined in TS 26.522 [X].

# * * * * 5th change * * * *

#### 5.8.5.3	Packet Detection Rule

The following table describes the Packet Detection Rule (PDR) containing information required to classify a packet arriving at the UPF. Every PDR is used to detect packets in a certain transmission direction, e.g. UL direction or DL direction.

Table 5.8.5.3-1: Attributes within Packet Detection Rule

| Attribute |  | Description | Comment |
| --- | --- | --- | --- |
| N4 Session ID |  | Identifies the N4 session associated to this PDR. NOTE 5. |  |
| Rule ID |  | Unique identifier to identify this rule. |  |
| Precedence |  | Determines the order, in which the detection information of all rules is applied. |  |
| Packet | Source interface | Contains the values "access side", "core side", "SMF", "N6-LAN", "5G VN internal". | Combination of UE IP address (together with Network instance, if necessary), CN tunnel info, |
| Detection | UE IP address | One IPv4 address and/or one IPv6 prefix with prefix length (NOTE 3). | packet filter set, application identifier, Ethernet PDU Session |
| Information. NOTE 4. | Network instance (NOTE 1) | Identifies the Network instance associated with the incoming packet. | Information and QFI are used for traffic detection. Source interface identifies the |
|  | CN tunnel info | CN tunnel info on N3, N9 interfaces, i.e. F-TEID. | interface for incoming packets |
|  | Packet Filter Set | Details see clause 5.7.6. | where the PDR applies, e.g. from access side (i.e. up-link), |
|  | Application identifier |  | from core side (i.e. down-link), |
|  | QoS Flow ID | Contains the value of 5QI or non-standardized QFI. | from SMF, from N6-LAN (i.e. the |
|  | Ethernet PDU Session Information | Refers to all the (DL) Ethernet packets matching an Ethernet PDU session, as further described in clause 5.6.10.2 and in TS 29.244 [65]. | DN), or from "5G VN internal" (i.e. local switch). |
|  | Framed Route Information | Refers to Framed Routes defined in clause 5.6.14. | Details like all the combination possibilities on N3, N9 interfaces are left for stage 3 decision. |
|  | Protocol Description | Indicates service protocol used by the flow, e.g. H.264/RTP, SRTP (NOTE 8). |  |
| Packet replication and detection carry on information | Packet replication skip information NOTE 7 | Contains UE address indication or N19/N6 indication. If the packet matches the packet replication skip information, i.e. source address of the packet is the UE address or the packet has been received on the interface in the packet replication skip information, the UP function neither creates a copy of the packet nor applies the corresponding processing (i.e. FAR, QER, URR). Otherwise the UPF performs a copy and applies the corresponding processing (i.e. FAR, QER, URR). |  |
| NOTE 6 | Carry on indication | Instructs the UP function to continue the packet detection process, i.e. lookup of the other PDRs. |  |
| Outer header removal |  | Instructs the UP function to remove one or more outer header(s) (e.g. IP+UDP+GTP, IP + possibly UDP, VLAN tag), from the incoming packet. | Any extension header shall be stored for this packet. |
| Forwarding Action Rule ID (NOTE 2) |  | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |  |
| Multi-Access Rule ID (NOTE 2) |  | The Multi-Access Rule ID identifies an action to be applied for handling forwarding for a MA PDU Session. |  |
| List of Usage Reporting Rule ID(s) |  | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. |  |
| List of QoS Enforcement Rule ID(s) |  | Every QoS Enforcement Rule ID identifies a QoS enforcement action that has to be applied. |  |
| Protocol Description |  | Indicates service protocol used by the flow e.g. H.264/RTP, SRTP (NOTE 8). |  |
| NOTE 1: Needed e.g. if: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or AN node in different IP domains. - UPF "local switch", N6-based forwarding and N19 forwarding is used for different 5G LAN groups. NOTE 2: Either a FAR ID or a MAR ID is included, not both. NOTE 3: The SMF may provide an indication asking the UPF to allocate one IPv4 address and/or IPv6 prefix. When asking to provide an IPv6 Prefix the SMF provides also an IPv6 prefix length. NOTE 4: When in the architecture defined in clause 5.34, a PDR is sent over N16a from SMF to I-SMF, the Packet Detection Information may indicate that CN tunnel info is to be locally determined. This is further defined in clause 5.34.6. NOTE 5: In the architecture defined in clause 5.34, the rules exchanged between I-SMF and SMF are not associated with a N4 Session ID but are associated with a N16a association. NOTE 6: Needed in the case of support for broadcast/multicast traffic forwarding using packet replication with SMF-provided PDRs and FARs as described in clause 5.8.2.13.3.2. NOTE 7: Needed in the case of packet replication with SMF-provided PDRs and FARs as described in clause 5.8.2.13.3.2, to prevent UPF from sending the broadcast/multicast packets back to the source UE or source N19/N6. NOTE 8: Not for PDR matching. It may be provided to assist PDU Set identification when PDU Set Identification marking applies to the PDR. |  |  |  |

# * * * * 6th change * * * *

### 5.37.5	PDU Set based QoS Handling

#### 5.37.5.1	General

A PDU Set is comprised of one or more PDUs carrying an application layer payload such as, e.g. a video frame or video slice. The PDU Set based QoS handling by the NG-RAN is determined by PDU Set QoS parameters specified in clause 5.7.7 and PDU Set information provided by the PSA UPF as described in clause 5.37.5.2.

Editor's note:	The applicability and details of PDU Set handling in uplink direction is pending RAN WG's progress.

In addition to the PDU related service information, the AF may provide PDU Set related assistance information for dynamic PCC control. One or more of the following PDU Set related assistance information may be provided to the NEF/PCF using the AF session with required QoS procedures in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3].

-	PDU Set QoS parameters as described in clause 5.7.7

-	Protocol Description: Indicates protocol and payload type used by the service data flow.

AF provided PDU Set QoS Parameters and Protocol Description may be used in determining the QoS Profile by the PCF and identifying the PDU Set information by the PSA UPF.

Editor's note:	Whether a standardized S-NSSAI (SST) is defined for XRM or whether non-standardized S-NSSAI is used is FFS.

#### 5.37.5.2	PDU Set Information and Identification

To support PDU Set based QoS handling, the PSA UPF identifies PDUs that belong to PDU Sets and determines the below PDU Set Information which it sends to the NG-RAN in the GTP-U header. The PDU Set information is used by the NG-RAN for PDU Set based QoS handling as described above.

The PDU Set Information comprises:

-	PDU Set Sequence Number.

-	Indication of End PDU of the PDU Set.

-	PDU Sequence Number within a PDU Set.

-	PDU Set Size in bytes.

-	PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow.

The NG-RAN may use the PDU Set Importance within a QoS Flow for PDU Set level packet discarding in presence of congestion.

Editor's note:	Whether and how PDU Set Importance can span across QoS Flows is FFS.

NOTE 2:	The PDU Set Information can be different for different PDU Sets within a QoS Flow.

The SMF instructs PSA UPF to perform PDU Set marking and may provide the PSA UPF the Protocol Description indicating the header, extension header (e.g. RTP/SRTP) and payload type (e.g. H.264) used by the service data flow. The Protocol Description may be received in the PCC rule, based on information provided by the AF or by PCF local policies as described in clause 5.37.5.1.

PSA UPF can identify the PDU Set Information using the Protocol Description and the received RTP/SRTP headers or using implementation specific means. The details of the RTP/SRTP headers, header extensions and/or payloads used to identify PDU Set Information are defined in TS 26.522 [X].

For each DL PDU received on N6 for which PDU Set based QoS handling is indicated from the SMF, the PSA UPF applies the rules for PDU Set identification and provides PDU Set Information which is available to the RAN in the GTP-U header.

# * * * * End of changes * * * *