3GPP TSG-WG SA2 Meeting #156-e  	S2-2304905r01

# Elbonia, April 17-21, 2023		(revision of S2-2300xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0801 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for analytics collection from MDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GP. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Implement the conclusion for KI#10: Interactions with MDAS/MDAF in clause 8.10 of TR 23.700-81. - NWDAF, as an MDA MnS Consumer, reuse the MDA functional overview and service framework as defined in TS 28.104 [15]. NWDAF discover MDAS/MDAF by MnS Discovery service producer, and subscribes to MDAS/MDAF by MDA Request service operation, Solutions #74 is adopted as baseline for normative work. - NWDAF supports MDA service framework as defined in TS 28.104 [15] acting as an MnS consumer performing the MDA service discovery as defined in TS 28.537 [28] as described in Solutions 73, 74. According to the conclusion of KI#10, a new procedure that the NWDAF collects analytics from MDAF is needed to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedure for analytics collection from MDAF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure is unknown on how the NWDAF consumes analytics from the MDAF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.2.X (new), 6.2.X.1 (new), 6.2.X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	Void.

[6]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[7]	3GPP TS 28.550: "Management and orchestration; Performance Assurance".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[10]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[11]	ITU-T Recommendation P.1203.3: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport - Quality integration module".

[12]	3GPP TS 38.215: "NR; Physical layer measurements".

[13]	Void.

[14]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[17]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes".

[18]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[19]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[20]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; stage 2".

[21]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); stage 2".

[22]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[23]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[24]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[25]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[26]	3GPP TS 29.503: "Unified Data Management Services; Stage 3".

[27]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[28]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[29]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[30]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[31]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[32]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[33]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[34]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[35]	3GPP TS 22.071: "Technical Specification Group Systems Aspects; Location Services (LCS)".

[36]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[37]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[38]	GSMA TS.06: "IMEI Allocation and Approval Process".

[39]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[40]	ITU-T Y.1540: "Internet protocol data communication service - IP packet transfer and availability performance parameters".

[41]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRMs). Integration Reference Point (IRP): Information Service (IS)".

[42]	3GPP TS 32.422: "Subscriber and equipment trace: Trace control and configuration management".

[43]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

[xx]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[yy]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

* * * * Second Change (all new) * * * *

6.2.X	Analytics Collection from MDAF

6.2.X.1	General

The MDA functional overview and service framework in Figure 5.1-1 as defined in TS 28.104 [xx] is used by NWDAF to trigger the MDA MnS to request analytics from the MDA Management Function.

Before NWDAF requests analytics from the MDA Management Function, the NWDAF firstly discovers the MDA Management Function via the MnS discovery service producer as defined in clause 5 of TS 28.537 [yy].

6.2.X.2	Procedure for analytics collection from MDAF

Figure 6.2.X.2-1: Procedure for analytics collection from MDAF

Precondition: Initially MDAF(s) or MDA MnS producers register their capabilities, i.e., MnS information or MnS profile as described in clause 5 of TS 28.537 [yy] to a MnS discovery service producer. The MnS discovery service producer may contain all or partial information related to the capabilities of MDA MnS producer.

1.	NWDAF discovers the MDA Function from the MnS discovery service producer by sending a MnS producer discovery service operation. The service operation may include the following parameters:

-	requestedMDAType: indicates a specific MDA capability such as Slice coverage analysis, Mobility performance analysis as defined in clause 7.2 of TS 28.104 [xx];

-	Area of Interest;

-	Network Slice information (i.e. NetworkSliceInfo including a DN (Distinguished Name) of the NetworkSlice managed object relating to the network slice instance associated to the S-NSSAI and NSI ID if available as defined in TS 28.541 [22]).

2.	The MnS discovery service producer feedback the relevant Mns information of the MDA Function.

NOTE: MnS information refer to the information used by the consumer to discover the procedure of specific Management Services and to derive the addresses of the Management Services.

- NWDAF uses the addresses received and issues a request to specific MDA Function(s) to discover the capabilities related to computing, latency, etc., which rely on the current workload.

- Each MDA Function provides the request capabilities in response.

Editor Note: Whether the NWDAF can request MDA capabilities from MDAF should be aligned with SA5.

- NWDAF selects the optimal MDA Function, which can fulfill the desired time schedule to obtain the analytics on time.

- The NWDAF sends analytics request to the selected optimal MDA Function by triggering a MDARequest service operation as defined in clause 9.3.2 of TS 28.104 [xx]. The service operation may include the following parameters:

-	requestedMDAOutputs: MDAOutputPerMDAType(type of analytics) and MDAOutputIEFilters (which assist in indicating explicitly the analytics output requested from the set of MDA outputs and the conditions, considering i) threshold cross, for reporting numeric outputs and ii) timeout, when an output is no longer needed);

-	reportingMethod: method for analytics output (it can only be notification-based reporting since file-based, and streaming are not supported in NWDAF).

-	reportingTarget: indicates the reporting target of the MDA outputs (in case the MDA MnS consumer is different from the one that issued the request).

-	analyticsScope: Area of Interest, Network Slice information, etc.;

-	startTime: indicates the start time of the analytics requested by the MnS consumer;

-	stopTime: indicates the stop time of the analytics requested by the MnS consumer.

-	analyticsWindow: indicates the time duration related to MDA output. It can be in the past, when the analytics is statistics, or in the future for a prediction.

7.	The MDA Function provides the analytics to the NWDAF by triggering an MDAReporting service operation.

* * * * End of Change * * * *