3GPP TSG-WG SA2 Meeting #156-e	S2-2304959r02

# Elbonia, April 17 – 21, 2023		(revision of S2-2302781)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3918 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Event exposure enhancement for enhanced NSACF architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The hierarchical NSAC architecture to support multiple service area has been added to TS23.501. The related impact to NSAC exposure also need be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add the event exposure enhancment for the hierarchical NSAC architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC cannot support the event exposure service operation for the hierarchical NSAC architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.10.1, 4.15.3.2.10.x (new), 5.2.21.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.15.3.2.10	Number of UEs and PDU Sessions per network slice notification procedure

4.15.3.2.10.1	Non-hierarchical NSAC architecture

This procedure depicts the case of an AF subscribing to receive the registered number of UEs, or the number of PDU sessions in a specific S-NSSAI. The procedure handles the case when there is a single NSACF in the PLMN responsible for the S-NSSAI (single Service Area) or when there are multiple NSACFs responsible for the S-NSSAI in the PLMN (multiple Service Areas).

Figure 4.15.3.2.10.1-1: Number of UEs and PDU Sessions per network slice notification procedure

1.	To subscribe or unsubscribe for the number of UEs or the number of PDU Sessions per network slice notification with the NEF, the AF sends Nnef_EventExposure Subscribe/Unsubscribe Request (Event ID, Event Filter, Event Reporting information, S-NSSAI) message to the NEF. The Event ID parameter defines the subscribed event ID, i.e. Number of Registered UEs or Number of Established PDU Sessions. The Event Filter parameter defines the S-NSSAI, in case of a trusted AF or AF-Service-Identifier as defined in TS 29.522 for an untrusted AF, for which reporting is required. The Event Reporting information parameter defines the mode of reporting, which includes threshold reporting with a threshold value or periodic reporting with included periodicity time interval. The S-NSSAI is the slice for which the subscription is requested

The AF may request one-time reporting or immediate reporting.

NOTE 1:	When immediate reporting but not for one-time reporting is requested, the subscription is maintained after returning the report to the AF. When one-time reporting is requested, the subscription is terminated right after returning the report to the AF.

Notifications related to the threshold based subscriptions behave as follows:

-	A single notification is sent only when the number of registered UEs or the number of established PDU Sessions reaches the threshold. A single notification is sent every time there is a change from being below the threshold to reach the threshold.

-	A single notification is sent only once when the number of registered UEs or the number of established PDU Sessions go below the threshold after reaching it. A single notification is sent every time there is a change from reaching the threshold to coming down below the threshold.

2.	The NEF confirms with Nnef_EventExposure_Subscribe/Unsubscribe Response message to the AF. This message may include the event reporting, if available in the NEF and immediate reporting or one-time reporting was requested by the AF. In the case of Untrusted AF, the NEF includes the AF-Service-Identifier corresponding to the S-NSSAI in the returned notification.

If immediate reporting or one-time reporting is requested, step 2 occurs after step 5 and the subscription response contains the immediate or one-time report. For the case of one-time reporting, no subscription is created at the NEF/NSACF.

3.	The NEF may query the NRF to find the NSACF(s) responsible for the requested S-NSSAI. If needed, the NEF translates the AF-Service-Identifier to the corresponding S-NSSAI prior to performing the query.

4.	If the NEF has not already subscribed to the event from the NSACF for the requested S-NSSAI, the NEF initiates the request Nnsacf_SliceEventExposure_Subscribe/Unsubscribe Request (Event ID, Event Filter, Event Reporting information, immediate reporting, S-NSSAI) to all the NSACFs supporting the requested S-NSSAI. The NEF stores the AF requested Event Reporting Information. If multiple NSACFs are selected for the requested S-NSSAI, the NEF may set the Event Reporting Information to periodic in its request to the NSACFs. If single NSACF is selected, the NEF sets the Event Reporting Information identical to the received request from the AF. The NEF also sets the Event ID and Event Filter identical to the received request from the AF

NOTE 2:	The period chosen is selected by the NEF based on its internal logic.

5.	The NSACF(s) confirms with Nnsacf_SliceEventExposure_Subscribe/Unsubscribe Response message to the NEF. This message may include the event reporting if available at NSACF and immediate reporting or one-time reporting was requested by the NEF.

6.	When the reporting condition for a subscribed event is fulfilled, the NSACF triggers a notification towards the NEF.

7.	The NSACF sends the Nnsacf_SliceEvent Exposure_Notify (Event ID, Event Reporting information) message to the NEF. If the subscription is for event based notification (e.g. based on the monitored event reaching a threshold value), the Event Reporting information parameter contains confirmation for the event fulfilment. If the subscription is for periodic notification or for immediate reporting, the Event Reporting information parameter provides information for the current number of UEs registered with a network slice (e.g. represented in percentage of the maximum number of the UEs registered with the network slice) or information for the current number of PDU Sessions on a network slice (e.g. represented in percentage of the maximum number of the UEs established on the network slice).

8.	When a single NSACF is returned from the discovery procedure, the NEF sends the Nnef_EventExposure_Notify (Event ID, Event Reporting information) message since the reporting condition is fulfilled. In the case of Untrusted AF, the NEF includes the AF-Service-Identifier corresponding to the S-NSSAI in the returned notification.

9.	When multiple NSACFs are selected for the requested NSSAI the NEF performs the aggregation from reporting NSACF(s) and maintain the overall usage of the S-NSSAI for the selected NSACFs as long as the subscription is active.

NOTE 3:	If multiple NSACFs are selected for the requested S-NSSAI, the NEF continuously updates the aggregated information to be able to fulfil the incoming subscription request from the AF.

10.	When multiple NSACFs are selected for the requested S-NSSAI and when the reporting condition for a subscribed event by the AF is fulfilled, the NEF sends Nnef_EventExposure_Notify (Event ID, Event Reporting information) message towards the AF. In case of untrusted AF; the NEF includes the AF-Service-Identifier corresponding to the S-NSSAI in the returned notification.

4.15.3.2.10.x	Hierarchical NSAC architecture

The procedure handles the case if the hierarchal NSAC architecture is deployed in the PLMN for the NSAC of an S-NSSAI.

Figure 4.15.3.2.10.x-1: Number of UEs and PDU Sessions per network slice notification procedure in hierarchal NSAC architecture deployment

1. The AF subscribes (or unsubscribe) the number of UEs or the number of PDU Sessions per network slice notification to the NEF. The NEF finds the primary NSACF responsible for the requested S-NSSAI. The procedure are same as steps 1-5 of Clause 4.15.3.2.10.1 with the following differences:

- based on the network configuration the NEF is aware that hierarchical NSAC architecture is deployed at the network. The NEF queries the NRF to find the corresponding primary NSACF responsible for the requested S-NSSAI using the Serving Area information of " Entire PLMN" as querying parameter.

- Due to only the primary NSACF is selected for the requested S-NSSAI, the NEF sets the Event Reporting Information identical to the received request from the AF. The NEF also sets the Event ID and Event Filter identical to the received request from the AF with the aggregation indication. If the aggregation indication is received, the Primary NSACF perform the aggregated value from NSACF(s)

NOTE 1: 	To support the update of local maximum number, it is possible the Primary NSACF has already aware the registered number of UE or established PDU session on each NSACF.

2-6.	The Primary NSACF subscribe (or unsubscribe) the number of UEs or the number of PDU Sessions per network slice notification to all the NSACFs supporting the requested S-NSSAI. The procedure are same as steps 4-7 of Clause 4.15.3.2.10.1 for multiple NSACFs case with the replacement of NEF with the Primary NSACF and the following differences:

- Step 2, based on the received event subscription from NEF, the Primary NSACF may update the previous subscribed event to NSACFs if the slice event exposure has been subscribed before. For example the periodicity information of the subscription is changed. If the slice even exposure has not been subscribed before, the Primary NSACF triggers the slice event subscription to NSACF.

- Step 6,	the Primary NSACF performs the aggregated value from reporting NSACF(s) and itself (if needed) and maintain the overall usage of the S-NSSAI for the indicated Event ID parameter, i.e., the number of UEs registered with a network slice or the number of PDU Sessions established on a network slice, from the selected NSACFs as long as the subscription is active. When the reporting condition for a subscribed event is fulfilled, the NSACF triggers a notification of the event towards AF via the NEF.

NOTE 2:	 For the number of registered UEs per network slice, the aggregated value includes both the number of registered UE at NSACF(s) and the number of registered UE at Primary NASCF. For the number of established PDU sessions per network slice, the aggregated value only includes the number of established PDU Session at the NSACF(s).

NOTE 3: 	It is possible to do the aggregation at the NEF. In this case, the mechanism defined in clause 4.15.3.2.10.1 is used. However, the NEF need maintain the subscription to multi NSACF.

# * * * * Next change * * * *

5.2.21	NSACF services

5.2.21.1	General

The following table illustrates the NSACF services.

Table 5.2.21.1-1: List of NSACF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnsacf_NSAC | NumOfUEsUpdate | Request/Response | AMF, SMF (NOTE 1) |
|  | NumOfPDUsUpdate |  | SMF |
|  | EACNotify |  | AMF |
| Nnsacf_SliceEventExposure | Subscribe | Subscribe/Notify | NEF, NWDAF, AF (NOTE 2), Primary NSACF |
|  | Unsubscribe |  | NEF, NWDAF, AF, Primary NSACF |
|  | Notify |  | NEF, NWDAF, AF, Primary NSACF |
| NOTE 1: If EPS counting is required for the S-NSSAI, the SMF+PGW-C uses the Nnsacf_NumberOfUEs Update services operation and Nnsacf_NumberOfPDUsUpdate at PDN connection establishment procedure. NOTE 2: The AF can access NSACF services via NEF to NSACF or to Primary NSACF depending on the NSAC deployment architecture in case of untrusted AF, or directly in case of trusted AF. |  |  |  |

# * * * * End of changes * * * *