3GPP TSG-WG SA2 Meeting #155 	S2-2304986r02

# Elbonia, April 17 – 21, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1308 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting AR Telephony Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | WID NG_RTC has objective to support AR telephony communication in IMS network, which has been concluded in TR 23.700-87. This CR reflects the TR conclusions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the AR network rendering architecture based on DC architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AC.2.2, and Annex AC.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * *

Annex AC (normative):
Support of Data Channel services in IMS

## AC.2.1	Architecture

### AC.2.1.x Architecture to support AR communication

To satisfy the needs of different AR devices that have different media processing capabilities, IMS needs to support media transmission and media rendering related to AR communication. This clause defines how IMS can be enhanced to support AR communication.

Multiple realization alternatives for network assisted AR are available depending on device capability and what requirements the AR application has on the media transport protocol used between the UE and the network assisted AR function. AR services can be delivered via the following mechanisms:

- The AR application server provides network assisted rendering by having all AR media being transported using an established application data channel between the UE and the AR application server. This scenario represents a standard P2A/A2P application data channel as per clause AC.7.2 and consequently not requiring any further description.

- A Data Channel Media Function (DCMF)/Media Resource Function (MRF) having the Augmented Reality Media Function (ARMF) capability, provides required network assisted AR media processing.

Figure AC.2.1.x-1 shows the architecture to support AR communication.

Figure AC.2.1.x-1: Architecture to support AR communication

NOTE 1:	The DCMF could be an MRF that support DC.

NOTE 2:	The AR Application Server is a specific DC Application Server and is not specified in this release.

NOTE 3:	The MDC2 reference point is not specified in 3GPP R18.

NOTE 4:	Definition of the DC4 and MDC2 reference points will be aligned with the overall data channel architecture.

Editor’s note: Whether a more generalized name for DCSF and DCMF is needed is FFS.

### AC.2.2	Functional entities

#### AC.2.2.1	Data Channel Signalling Function (DCSF)

The DCSF is the signalling control function that provides data channel control logic. The DCSF is not involved in SIP signalling. The DCSF supports the following functionalities:

-	The DCSF receives event reports from the IMS AS and decides whether data channel service is allowed to be provided during the IMS session;

-	The DCSF manages bootstrap data channel and (if applicable) application data channel resources at the DCMF or MRF via the IMS AS;

-	The DCSF supports HTTP web server functionality to download data channel applications (bootstrapping) via DCMF and/or MRF to the UE based on UE subscription.

-	The DCSF downloads data channel applications from the Data Channel Application Repository;

NOTE 1:	The detailed procedures to download data channel applications to the UE are out of scope of this specification.

-	The DCSF interacts with NEF for data channel capability exposure via N33;

NOTE 2:	Interaction with NEF is not specified in this release of the specification.

-	The DCSF interacts with the DC Application Server for DC resource control via DC4/DC3, and for traffic forwarding via MDC3/MDC2.

NOTE 3:	Interaction with the DC Application Server for resource control is not specified in this release of the specification.

#### AC.2.2.2	Data Channel Application Repository (DCAR)

The Data Channel Application Repository stores data channel applications which are retrieved by the DCSF when required.

#### AC.2.2.3	Data Channel Media Function (DCMF) / MRF

The DCMF and MRF provide the media resource management and forwarding of data channel media traffic. The DCMF and MRF provides the following functionalities:

-	The DCMF and MRF manage the data channel media resources (bootstrap and application data channel resources, if applicable) under the control of the IMS AS;

-	The DCMF and MRF terminate the bootstrap data channel from the UE and forward HTTP traffic between the UE and DSCF via MDC1;

-	The DCMF and MRF may anchor the application data channel in P2P scenarios, if required, and forward application data traffic from/to the UEs;

-	The DCMF and MRF relays traffic on the A2P/P2A application data channel between the UE and the DC Application Server via MDC2.

-	The DCMF and MRF optionally receives and stores AR service media handling logic from the DC AS, and provide AR media rendering function based on the AR metadata and audio/video media streams received from UE.

#### AC.2.2.4	IMS AS

The IMS AS is enhanced to support the following functionalities:

-	The IMS AS interacts with the DCSF via DC1 for event notifications;

-	The IMS AS receives the data channel control instructions from the DCSF and accordingly interacts with the DCMF via DC2 or with MRF via Mr'/Cr for data channel media resource management;

-	The IMS AS receives the media control instructions from the DCSF and accordingly interacts with the UE for connecting the UE’s audio/video media termination to the DCMF.

# * * * * Second change * * * *

## AC.x	Data Channel Services Procedures

### AC.x.1	Supporting AR Communication Procedures

### Editor’s note: The procedures may need to further update according the conclusion SA4.

#### AC.x.1.1   Terminal Rendering Procedure

Figure AC.x.1-1 depicts a typical call flow procedure to establish a UE centric AR IMS session from UE-A perspective. The main steps in the call flow are as follows:

1.	UE-A initiates an IMS communication with UE-B, including establishment of bootstrap data channel connections.

2.	The user of UE-A upgrades the IMS session with AR experience. 
UE-A initiates a re-INVITE adding media descriptors required by the AR application to be established E2E, e.g. RTP and/or application data channels.

In this scenario, the UE performs AR media rendering based on the AR media from its local application.

3.	UE-A captures and encodes AR media.

4.	UE-A sends the AR media to the peer through the established media connection(s).

5.	UE-A receives AR media from the peer through established media connection(s).

6.	The UE-A decodes and displays the received  AR media on its screen.

Figure AC.x.1-1: Establishment of UE Centric AR communication session

Editor’s note: This procedure is a standard P2P data channel scenario, whether the call flow is needed is FFS.

#### AC.x.1.2   Network-assisted Rendering Procedure

Figure AC.x.1.2-1 depicts a typical call flow procedure to establish a network assisted AR IMS session from a UE-A perspective, being a device with limited AR capabilities. The main steps in the call flow are as follows:

1.	UE-A initiates an IMS communication with the UE-B, including establishment of bootstrap data channel connections.

2.	The user of UE-A upgrades the IMS session with AR experience. The DCSF/AR Application Server concludes that the UE has limited AR capabilities requiring network assisted media processing.
The downloaded AR application in UE-A, initiates a re-INVITE adding media descriptors required by the network assisted AR application. The DCSF requests IMS AS to have the added media descriptors anchored in the ARMF.

3.	AR data is in this scenario captured by both UE-A and UE-B.

4.	UE-A and UE-B send the captured AR media to the ARMF of the originating network.

5	The ARMF processes, renders and encodes received AR media on behalf of UE-A as per AR application instructions.

6	The rendered AR media is sent to UE-A.

7	UE-A displays the AR media to the user.

Figure AC.x.1.2-1: Establishment of NW Assisted AR communication session

# * * * * End of changes * * * *