SA WG2 Meeting #157	S2-2306584

# Berlin, Germany, May 22nd – 26th, 2023           	  			  			   (revision of S2-2305634)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2087 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New feature for 5G-RG to support NSWO procedure to authorize UE behind RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, [China Telecom,] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For 23.700-17 has concluded that “in order to support NSWO (as defined in clause 5.42 of TS 23.501) for 3GPP UE, 5G-RG shall support the SWa' interface to NSWOF but this aspect is in the scope of BBF and/or Cable Labs specifications.” Besides, in TS 23.501 clause 4.2.15, UE is allowed to use 5G credentials based on NSWO to authenticate to the 5GC to first obtain local IP address and then to use untrusted Non-3GPP access to register to the 5GC. Similarly, when UE is behind RG, the UE should also be able to use NSWO to first authenticate to 5GC. So propose a feature for 5G-RG to support NSWO procedure to authorize UE behind RG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - propose a feature for RG to support NSWO procedure to authorize UE behind RG. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RG(s) do not support 5G NSWO procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10, 4.10d (new), 2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X |  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev02: - Support of NSWO supported for 3GPP UE behind a FN-RG - The description of REV01 was referring to a single deployment (traffic offload at W-AGF ) while other are possible. The possible deployments are anyhow to be specified by BBF and CableLabs - In clause 4.10, add “Support of NSWO for 3GPP UE behind a RG is specified in clause 4.10d” |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 4.10	UE behind 5G-RG and FN-RG

An RG connecting via W-5GAN or NG-RAN access towards 5GC can provide connectivity for a UE behind the RG to access an N3IWF or TNGF. It is assumed that the UE is 5GC capable, i.e. supports untrusted non-3GPP access and/or trusted non-3GPP access. This allows the RG, W-5GAN and the RG's connectivity via 5GC to together act as untrusted/trusted N3GPP access to support UEs behind the RG.

When FN-RG/5G-RG is serving a UE, the control and user plane packets of the UE is transported using a FN-RG/5G-RG IP PDU session and then from PSA UPF of that PDU session to an IWF. A single FN-RG/5G-RG IP PDU session can be used to serve multiple UEs.

Figure 4.10-1 shows the non-roaming architecture for a UE, behind a 5G-RG, accessing the 5GC via TNGF where the combination of 5G-RG, W-5GAN and UPF serving the 5G-RG is acting as a trusted Non-3GPP access network.

NOTE 1:	FN-RG and W-5GAN acting as trusted Non-3GPP access is not considered in this specification as it is assumed that FN-RG does not support EAP-based access control (e.g. 802.1X).

Figure 4.10-1: Non-roaming architecture for UE behind 5G-RG using trusted N3GPP access

The 5G-RG can be connected to 5GC via W-5GAN, NG-RAN or via both accesses. The UE can be connected to 5GC via 5G-RG, NG-RAN or via both accesses.

The TNGF and Ta reference point are defined in TS 23.501 [2].

NOTE 2:	The reference architecture in figure 4.10-1 only shows the architecture and the network functions directly connected to W-5GAN or TNGF, and other parts of the architecture are the same as defined in clause 4.2 of TS 23.501 [2].

NOTE 3:	The reference architecture in figure 4.10-1 supports service based interfaces for AMF, SMF and other NFs not represented in the figure.

NOTE 4:	The two N2 instances in Figure 4.10-1 apply to a single AMF for a 5G-RG which is simultaneously connected to the same 5G Core Network over 3GPP access and W-5GAN.

Support of NSWO for 3GPP UE behind an RG is specified in clause 4.10d.

# * * * * Second change * * * *

## 4.10d	Support of NSWO for 3GPP UE behind a RG

In order to support NSWO defined in clause 5.42 of TS 23.501 [2] for 3GPP UE, RG may support the WLAN Access functionality defined in TS 23.501 [2], clause 4.2.15 and clause 5.42. The WLAN Access functionality includes the support of the SWa' interface to NSWOF as shown in Figure 4.10d-1 and Figure 4.10d-2. In this case the user plane packets of the UE are not transported via the RGs PDU Session and are also not traversing the UE’s 5GC. The user plane for NSWO traffic is offloaded without traversing the 5GC network of the UE.

As specified in clause 4.2.15 of TS 23.501 [2], the SWa’ reference point corresponds to SWa reference point defined in TS 23.402 [X] but where the EAP user ID is a SUCI and not an IMSI.

The SWa’ support in Wireline access network has no impact on 3GPP specifications.

NOTE 1: 	The specification of support of functionalities for NSWO in the wireline access network is under BBF and Cablelabs responsibility. This includes potential support of AAA proxy functionality as shown in clause 4.2.15 of TS 23.501 [2] allowing to carry AAA flows between the RG and the NSWOF over the network access of the RG as well as how offloaded data bypass the 5GC of the UE.

Figure 4.10d-1: Reference architecture to support authentication for Non-seamless WLAN offload over a RG

Figure 4.X.1-2: Service based reference architecture to support authentication for Non-seamless WLAN offload in 5GS

NOTE 2: 	Mechanisms used to forward the traffic of UE in NSWO mode are defined by BBF and Cable Labs specifications.

# * * * * next change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	BBF TR-124 issue 5: "Functional Requirements for Broadband Residential Gateway Devices".

[6]	BBF TR-101 issue 2: "Migration to Ethernet-Based Broadband Aggregation".

[7]	BBF TR-178 issue 1: "Multi-service Broadband Network Architecture and Nodal Requirements".

[8]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[9]	BBF TR-456 issue 2: "AGF Functional Requirements".

[10]	BBF WT-457: "FMIF Functional Requirements".

NOTE:	Technical Report of BBF WT-457 will be TR-457 which will be available when finalized by BBF.

[11]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[12]	BBF TR-177 Issue 1 Corrigendum 1: "IPv6 in the context of TR-101".

[13]	IETF RFC 6788: "The Line-Identification Option".

[14]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[15]	IETF RFC 3315: "Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[16]	IETF RFC 6603: "Prefix Exclude Option for DHCPv6-based Prefix Delegation".

[17]	IETF RFC 3633: "IPv6 Prefix Options for Dynamic Host Configuration Protocol (DHCP) version 6".

[18]	BBF TR-069: "CPE WAN Management Protocol".

[19]	BBF TR-369: "User Services Platform (USP)".

[20]	IETF RFC 3046: "DHCP Relay Agent Information Option".

[21]	IETF RFC 4604: "Using Internet Group Management Protocol Version 3 (IGMPv3) and Multicast Listener Discovery Protocol Version 2 (MLDv2) for Source-Specific Multicast".

[22]	3GPP TR 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[23]	3GPP TS 38.413: "NG RAN; NG Application Protocol (NGAP)".

[24]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[25]	3GPP TS 22.011: "Service accessibility".

[26]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[27]	CableLabs WR-TR-5WWC-ARCH: "5G Wireless Wireline Converged Core Architecture".

[28]	IETF RFC 3376: "Internet Group Management Protocol, Version 3".

[29]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS)".

[30]	BBF TR-198: "DQS:DQM systems functional architecture and requirements".

[31]	3GPP TS 23.203: "Policy and charging control architecture".

[32]	3GPP TS 33.126: "Lawful Interception Requirements".

[33]	IETF RFC 2236: "Internet Group Management Protocol, Version 2".

[34]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[35]	IETF RFC 1112: "Internet Group Management Protocol".

[36]	IETF RFC 2710: "Multicast Listener Discovery Version for IPv6".

[37]	IETF RFC 2010: "Operational Criteria for Root Name Servers".

[38]	BBF TR-470: "5G FMC architecture".

[39]	3GPP TS 29.519: "Policy Data, Application Data and Structured Data for exposure".

[40]	3GPP TS 23.041: "Public Warning System".

[41]	IEEE Publication (2017): "Guidelines for Use of Extended Unique Identifier (EUI), Organizationally Unique Identifier (OUI), and Company ID (CID)". https://standards.ieee.org/content/dam/ieee-standards/standards/web/documents/tutorials/eui.pdf.

[42]	3GPP TS 29.413: "Application of the NG Application Protocol (NGAP) to non-3GPP access".

[43]	Void.

[X]	3GPP TS 23.402: " Architecture enhancements for non-3GPP accesses".

# * * * * End of changes * * * *