3GPP TSG-WG SA2 Meeting #157 	S2-2306777

Berlin, Germany, May 22 – 26, 2023	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Update on Network based SL Positioning for UE with NAS connection

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes some updates on Network based SL Positioning for UE with NAS connection.

1. Introduction/Discussion

This paper proposes some updates on Network based SL Positioning for UE with NAS connection.

1. Change network-assisted operation into network-based operation, and change Network assisted SL Positioning into Network based SL Positioning.

2. Remove SL-MO-LR and SL-MT-LR from clause 5.5.2, since SL-MO-LR/SL-MT-LR is related to location service requests for relative positioning/range/direction between one or more UEs.

3. Update the condition that LMF interacts with GMLC to get Locate UE location, into that the location of Located UE is not known.

4. Update LMF functionality, including the determination of LMF/target UE selecting Located UE, the determination of ranging/SL positioning method, determination of the required positioning QoS of Located UE, and determination of the same scheduled location time.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.586.

# * * * * First change * * * *

4.1	General concept

Ranging based service provides the distance between two UEs or more UEs and/or the direction of one UE (i.e. Target UE) from another UE (i.e. Reference UE) via PC5 operations.

Sidelink Positioning provides absolute location, relative position, or Ranging information of a UE by using PC5 for the positioning. A Located UE can be used to determine the absolute location of a Target UE using Sidelink Positioning.

The operation of Ranging/Sidelink Positioning can be performed with either Network-based Operation or UE-only Operation:

-	In the Network-based Operation, 5GC NF(s) is involved for the service request handling and result calculation.

-	In the UE-only Operation, the service request handling and result calculation are performed by UE.

The Ranging/Sidelink Positioning service request can be initiated by a UE (i.e. SL Positioning Client UE, Target UE, SL Reference UE), a 5GC NF, an LCS Client or an AF.

# * * * * next change * * * *

4.3.1	UE

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the UE may support the following functions:

-	Reporting the following Ranging/SL Positioning related capabilities to 5GC over the N1 reference point:

-	Capability of supporting Ranging/SL Positioning over PC5;

NOTE: Based on Ranging/SL Positioning control, a UE capable of Ranging/SL Positioning may take different roles in the operation, e.g. Target UE, Reference UE, Located UE.

-	Capability of supporting SL Positioning Server UE over PC5.

-	Procedures for Ranging/SL Positioning over PC5;

-	Procedures to Network based SL Positioning;

-	Procedures to Ranging/SL Positioning service exposure;

-	Indicating UE Policy Provisioning Request in UE Policy Container for UE triggered Ranging/SL Positioning Policy provisioning, which requests one or multiple types of policies/parameters as listed below:

-	Policy/parameters for Ranging/SL Positioning over PC5;

-	Policy/parameters for Located UE;

-	Policy/parameters for Target UE in addition to the functions defined in TS 23.273 [8] clause 4.3.5;

-	Policy/parameters for SL Positioning Client UE;

-	Policy/parameters for SL Positioning Server UE;

-	Receiving the Ranging/SL Positioning Policy from 5GC over N1 reference point.

-	Configuration of parameters for Ranging/SL Positioning over PC5. These parameters can be pre-configured in the UE, or, if in coverage, provisioned or updated by signalling over the N1 reference point from the PCF in the HPLMN or over SR1 reference point from the Ranging/SL Positioning Application Server.

# * * * * next change * * * *

### 4.3.2	PCF

In addition to the functions defined in TS 23.501 [2] and TS 23.503 [4], the PCF includes functions to provision the UE with necessary policies and parameters to use Ranging/SL Positioning services, as part of the UE Ranging/SL Positioning Policy information as defined in TS 23.503 [4] clause 4.2.2:

-	Authorization policy and parameters for Ranging/SL Positioning over PC5.

-	Authorization policy and parameters for Network based SL Positioning;

-	Authorization policy and Ranging/SL Positioning service exposure;

The PCF may update the Ranging/SL Positioning to the UE under certain conditions.

When receiving the Ranging/SL Positioning in Npcf_UEPolicyControl_Create Request from the AMF or when receiving the updated subscription data from UDR, the PCF generates the PC5 QoS parameters used by NG-RAN corresponding to a UE as defined in clause 5.4.2 of TS 23.287 [6].

Editor's note:	It is FFS what the Ranging/SL positioning QoS parameters are, whether the Ranging/SL positioning QoS parameters are needed and how to use the Ranging/SL positioning QoS parameters.

# * * * * next change * * * *

4.3.4	UDM

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the UDM performs the following functions:

-	Subscription management for Ranging/SL Positioning over PC5.

-	Subscription management for Network based SL Positioning.

-	Subscription management for Ranging/SL Positioning service exposure.

# * * * * next change * * * *

### 5.3.1	General

Any UE supporting Ranging/SL Positioning, e.g. Target UE, Reference UE, Sidelink Positioning Server UE, shall have a Ranging/SL Positioning layer, which is above the AS layer and handles service requests received from application layer to control the Ranging/SL Positioning operation. The Ranging/SL Positioning layer provides the support of authorization and provisioning as described in clause 5.1, the UE discovery and selection as described in clause 5.2, the Ranging/SL Positioning Protocol (RSPP) defined in TS 38.355 [12], and the protocols between UE and LMF for Ranging/SL Positioning. Ranging/SL Positioning layer supports the Ranging/SL Positioning service sessions for one time or periodic ranging, ranging for distance or direction measurement or both. The Ranging/SL Positioning layer provides the ranging parameters to AS layer, including one time or periodic ranging, ranging for distance or direction measurement or both.

NOTE 1: The Ranging/SL Positioning Protocol (RSPP) mention above corresponds to Sidelink Positioning Protocol (SLPP) in RAN.

Editor's note:	The protocol used between UE and LMF will be decided by RAN and could be an extension of LPP, a new protocol or both.

The RSPP is exchanged over SR5 over PC5-U reference point between UEs (i.e. Target UE, Reference UE, Assistant UE, Located UE and SL Positioning Server UE) to manage the Ranging/SL Positioning service sessions between UEs or among a group of UEs. Multiple UEs may be involved in a single Ranging/Sidelink Positioning session. When the Ranging/SL Positioning service session is among a group of UEs, the RSPP may provide group related identifier information for the PC5 transport layer handling. The transport of the RSPP over PC5-U is specified in clause 5.3.2.  The RSPP supports the following functions:

-	exchange the Ranging/Sidelink Positioning capability;

-	exchange the Ranging/Sidelink Positioning assistant data;

-	exchange Ranging/Sidelink positioning measurement data/result.

NOTE 2:	Group management is out-of-scope of this specification.

NOTE 3: 		RSPP signalling negotiation determines whether measurement results or Ranging/Sidelink Positioning results are exchanged over the SR5.

NOTE 4: 	The Ranging/Sidelink positioning supports the scheduled location time feature.

A LMF shall be involved when at least one of the Target UE and the Reference UE are in the network coverage and the serving network is capable to support ranging functionalities. The details of Network based Ranging/SL Positioning operation is described in detail in clause 5.5. When Network based Ranging/SL Positioning is used, the use of RSPP is necessary for Ranging/SL Positioning control.

A SL Positioning Server UE can be discovered and selected for result calculation, method determination, assistant data distribution and SL Reference UE selection in case of out-of-coverage or for UE-only Operation if the serving network does not support Ranging/SL Positioning. If the LMF capable for Ranging/SL Positioning is reachable by Target UE and/or Reference UE, the LMF can still decide that a SL Positioning Server UE, Target UE or Reference UE executes the result calculation. A SL Positioning Server UE can be co-located with a Target UE or Reference UE.

Editor's note:	Functionalities of the SL Positioning Server UE will be determined by RAN WGs.

# * * * * next change * * * *

### 5.5.2	Network based SL Positioning for UE with NAS connection

A UE with a NAS connection is in RM-REGISTERED state. The UE can enter CM-Connected state by performing UE triggered Service Request for 5GC-MO-LR or performing Network triggered Service Request for 5GC-NI-LR or 5GC-MT-LR. As the Target UE can establish a NAS signalling connection with the AMF, the functionality specified in TS 23.273 [8] for location services can be reused including e.g. 5GC-MO-LR, 5GC-MT-LR and 5GC-NI-LR with the additional functionality captured in this clause.

NOTE 1:	Target UE can establish a NAS signalling connection directly or indirectly via a ProSe L2 UE-to-Network Relay.

-	The Location Service is triggered via the AMF serving the Target UE. The location request comes either from an AF/external client via the GMLC, a 5G NF, or the Target UE.

-	The LTE positioning protocol (LPP) as specified in TS 37.355 [9] is used between the Target UE and the LMF. When LCS procedures are used to estimate the location of the Located UE, LPP is used between Located UE and LMF.

Editor's note:	RAN WGs will determine whether and what enhancements and the subset functionalities of LPP are needed to support Network based SL positioning including an 5GC-MT-LR, 5GC-MO-LR and 5GC-NI-LR.

-	Either the Target UE or LMF determines if UE Positioning using SL positioning will be applied, if 5GC-MO-LR/5GC-MT-LR service is triggered. When LMF determines that UE Positioning using SL positioning is used, LMF may trigger the Target UE to perform the discovery of Located UE(s).

-	The Target UE discovers Located UE(s) for UE Positioning using SL positioning. Optionally, to assist the Target UE the LMF may provide to the Target UE a list of candidate Located UE(s). In this case the LMF maintains the candidate list of Located UE(s) including e.g. the capability and location information of the Located UE(s). The Target UE selects the Located UE(s), and reports the UE identity of the selected Located UE(s) to LMF.

-	The Target UE and Located UE(s) perform ranging/SL positioning. The Target UE includes the UE identity of the Located UE(s) to the LMF together with the Ranging measurement data or estimation result. The LMF may interact with GMLC to get the location of Located UE.

Editor's note:	It is FFS what UE identity of the Located UE that the Target UE provides to the LMF. Depending on which UE identity, the LMF or the GMLC may need to resolve this by query another NF. The security impact will be coordinated with SA3.

-	The LMF uses the location of Located UE(s) together with the ranging/SL positioning measurement data or estimation results reported by Target UE and optionally also by Located UEs to estimate the location of the Target UE.

- 	In case the location of Located UE(s) is not known, the LMF interacts with GMLC a required QoS to get the location of Located UE. The ranging/SL positioning and the positioning of the Located UE(s) can be scheduled with the same time using the scheduled location time, as specified in TS 23.273 [8], to improve the Target UE positioning accuracy.

NOTE 2:	The LMF may need to compensate the time difference of the positioning of Target UE and Located UE with additional information, e.g. velocity.

-	The LMF provides the UE location estimate to the Target UE over LPP, or to the AMF, and AMF forwards the UE location to the GMLC, or to the 5GC NF according to legacy functionality.

# * * * * last change * * * *

4.3.8	LMF

In addition to the functions defined in TS 23.273 [8], the LMF supports the following:

-	The Network based SL Positioning;

-	Trigger Ranging/Sidelink positioning.

-	Exchange the Ranging/Sidelink Positioning capability.

-	Exchange the Ranging/Sidelink Positioning assistant data.

-	Exchange Ranging/Sidelink positioning signal measurement data/result.

-	Support of receiving stored Ranging/Sidelink Positioning capability from AMF and support of providing updated Ranging/Sidelink Positioning capability to AMF.

-	Determine LMF or Target UE to select Located UEs.

-	Determine ranging/SL positioning method based on the positioning QoS requirement, UE’s Ranging/Sidelink Positioning capability.

-	Determine the required QoS for Located UE positioning.

-	Determine the same scheduled location time for the ranging/SL positioning and the positioning of the Located UE(s).

-	Determine the location of the Target UE based on the ranging/SL positioning measurement data or result reported by Target UE and the location of Located UE(s).

-	Interaction with GMLC to get the location of Located UE.

-	Delivering the Ranging/Sidelink positioning service request/response for the Ranging service exposure to UE.

-	Resolve the Application Layer ID of the UE involved in Sidelink Positioning by interacting with 5G DDNMF.

# * * * * End of changes * * * *