3GPP TSG-WG SA2 Meeting #157 	S2-2306819

Berlin, Germany, May 22 – 26, 2023	(revision of S2-23xxxxx)

Source:	CATT

Title:	Resolve ENs on UE Discovery

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes to resolve the ENs on UE Discovery in TS 23.586.

1. Discussion

Regarding on the Ranging/Sidelink Positioning UE Discovery, the following open issues need to be resolved.

1) Identifiers included in Direct Discovery message

As both Model A and Model B discovery as defined in clause 6.3.2 of TS 23.304 are supported for the 5G ProSe capable UEs discovery, the identifiers included in Announcement/Solicitation/Response messages should be reused if applicable and new identifiers should be avoided.

On the other hand, since RSPP metadata information has been introduced to indicate the necessary information (e.g. UE role) for Ranging/Sidelink Positioning UE Discovery, there is no need to include other information, e.g. ProSe identifier, User Info ID.

It is proposed to remove the unnecessary identifiers and related ENs.

2) Criteria for Server UE Discovery

Per the definition of SL Positioning Server UE in TS 23.586, the reason for a Target UE to discover a Server UE is more about Server UE can offer Ranging/SL Positioning functionality rather than Target UE/Server UE not served by network/serving network not support Ranging/SL Positioning.

It is proposed to remove the unnecessary criteria and related ENs.

2. Text Proposal

It is proposed to capture the following changes into TS 23.586.

# * * * * First change* * * *

5.2.1	General

Both Model A and Model B discovery as defined in clause 6.3.2 of TS 23.304 [7] are supported for the 5G ProSe capable UEs discovery (including commercial and public safety use cases). During Ranging/SL Positioning Group Member Discovery, the Application Layer Group ID indicates a Ranging/Sidelink Positioning group that the UE belongs to.

Procedures for V2X communication over PC5 reference point as defined in TS 23.287 [6] are supported for the V2X capable UEs discovery. When unicast mode V2X communication procedure as defined in 6.3.3 of TS 23.287 [6] is used, the Service Type in Layer-2 link establishment procedure indicates "Ranging/Sidelink Positioning"; when broadcast mode or groupcast mode V2X communication procedure as defined in 6.3.1 or 6.3.2 of TS 23.287 [6] is used, Service Type indicates "Ranging/Sidelink Positioning".

The role(s) of the discovered UE (e.g. Target UE, SL Reference UE, SL Positioning Server UE) is (are) included in discovery message (for 5G ProSe capable UE) and unicast link establishment messages (for V2X capable UE) as RSPP specific meta data. More details on this and other information conveyed during discovery can be found in clause 6.4.

The procedures for Ranging/Sidelink Positioning UE Discovery & Selection are defined in clause 6.4.

# * * * * Second change * * * *

5.2.3	SL Positioning Server UE Discovery & Selection

SL Positioning Server UE Discovery & Selection is performed by the Target UE, when it meets one or more of the following criteria:

- The Target UE is not capable of performing SL Positioning Server UE functionalities.

The discovery of SL Positioning Server UE follows the same principles as specified in clause 5.2.1. The UE can indicate its role “SL Positioning Server UE” in its list of supported roles during discovery, if it is authorized to be a SL Positioning Server UE in a given PLMN as per the Authorization and Provisioning for Ranging/SL positioning service as specified in clause 5.1.

The Target UE shall discover and select a SL Positioning Server UEs that are in the same or different serving PLMN of the Target UE and the Reference UE(s).

Editor’s note:	Other criteria for the selection of SL Positioning Server UE is FFS, and can be coordinated with RAN WGs.

# * * * * Third change* * * *

6.4.2.1	Ranging/SL Positioning UE direct discovery

As defined for the Model A and Model B discovery in TS 23.304 [7]:

-	Model A uses a single discovery protocol message (Announcement).

-	Model B uses two discovery protocol messages (Solicitation and Response).

Figure 6.4.2.1-1 illustrates the procedure for Ranging/SL Positioning UE discovery with 5G ProSe capable UE using Model A discovery.

Figure 6.4.2.1-1: Ranging/SL Positioning UE discovery in Model A

1.	The Announcing UE (UE-1) sends a Ranging/SL Positioning Announcement message. The Ranging/SL Positioning Announcement message includes the Type of Discovery Message, security protection element, RSPP metadata information.

The Destination Layer-2 ID used to send the Ranging/SL Positioning Announcement message is configured in clause 5.2.

The Source Layer-2 ID to send the Ranging/SL Positioning Announcement message is self-assigned by the Announcing UE

A Monitoring UE determines the Destination Layer-2 ID for signalling reception based on the configuration in clause 5.2.

A Monitoring UE selects the Announcing UE based on the information received in step1.

Editor's note:	The RSPP metadata information (e.g., the role(s) of the Announcing UE) is included as the metadata in the Announcement message, which value is determined by RAN2.

Editor's note:	How the RSPP metadata information is used to convey that a UE is a “Located UE” needs to be aligned with RAN2.

Figure 6.4.2.1-2 illustrates the procedure for Ranging/SL Positioning UE discovery with 5G ProSe capable in Model B discovery.

Figure 6.4.2.1-2: Ranging/SL Positioning UE discovery in Model B

1.	The Discoverer UE (UE-1) sends a Ranging/SL Positioning Solicitation message. The Ranging/SL Positioning Solicitation message includes the Type of Discovery Message, security protection element, and optionally RSPP metadata information.

The Destination Layer-2 ID used to send the Ranging/SL Positioning Solicitation message is configured in clause 5.2.

The Source Layer-2 ID to send the Ranging/SL Positioning Solicitation message is self-assigned by the Discoverer UE

A Discoveree UE determines the Destination Layer-2 ID for signalling reception based on the configuration in clause 5.2.

Editor's note:	The RSPP metadata information (e.g., the specific Role(s) to be discovered) is included as the metadata in the Solicitation message, which value is determined by RAN2.

2	The Discoveree UE that matches the Ranging/SL Positioning solicitation message (e.g., RSPP metadata information) responds to the Discoverer UE with the Ranging/SL Positioning Response message. The Ranging/SL Positioning Response message includes Type of Discovery Message, security protection element, RSPP metadata information.

The Source Layer-2 ID used to send the Ranging/SL Positioning Response message is specified in clause 5.2.

The Destination Layer-2 ID is set to the Source Layer-2 ID of the received Ranging/SL Positioning Solicitation message.

Editor's note:	The RSPP metadata information (e.g., the role(s) of the Discoveree UE) is included as the metadata in the Announcement message, which value is determined by RAN2.

# * * * * End of changes* * * *