3GPP TSG-WG SA2 Meeting #157 	S2-2307250

Berlin, Germany, 22-26 May 2023

# Title:	[Draft] Reply LS on applicability of IWK to mixture of control plane only PDU session and non-control plane only PDU session

# Response to:	LS on applicability of IWK to mixture of control plane only PDU session and non-control plane only PDU session (C1-232702/S2-2306756)

# Release:	Release 18

# Work Item:	5GProtoc18

Source:	SA2

To:	CT1

Cc:	-

Contact Person:

Name:	Sebastian Speicher

E-mail Address:	speicher AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 would like to thank CT1 for the LS on applicability of IWK to mixture of control plane only PDU session and non-control plane only PDU session and would like to provide the following feedback:

CT1 question 1: Can the additional established PDU session in step 2 be subject to inter-working with EPS given the limitation specified in above text from TS 23.501?

SA2 reply: Yes, the additional PDU session can be subject to inter-working with EPS if the new PDU Session is for a DNN/S-NSSAI for which the subscription data for SMF Selection includes an Invoke NEF indication (i.e. for a PDU Session which will be anchored in NEF).

CT1 question 2: if the answer to Q1 is YES, then does the UE need to do anything during TAU procedure for idle or connected mode interworking to EPS? Or the target MME will take care of this if it happens?

SA2 reply: No additional UE or MME behaviour is needed during Idle/Connected mode mobility to handle this scenario.

2. Actions:

To CT1:

ACTION: 	SA2 kindly asks CT1 to take the above into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #158	21-25 August 2023		Gothenburg, Sweden

TSG-SA2 Meeting #159	9-13 October 2023		Xiamen, P.R. China