3GPP TSG-WG SA2 Meeting #157                                                          S2-2307357

May 22-26, 2023   Berlin

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4681 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update TS23.501 EoDB Time to Next Burst |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-5-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The EoDB marking provided by the UPF is of significantly greater use to the RAN if an indication can be given as to the time to the start of the next burst. The RAN can use this information to better configure CDRX (e.g. for Deep, Light or Micro sleep as per UE Power consumption model – see TR38.840). This also aligns with proposed SA4 encoding of the RTP Header Extension and the SA2 LS response to SA4 and RAN2 provided by S2-230xxxx_Reply_LS_on_EoDB |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description of EoDB is updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Poorer power savings in the RAN and GTP-U HE does not align with SA4 RTP HE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

### 5.37.8	UE power saving management

#### 5.37.8.1	General

The following traffic assistance information may be provided by the CN to NG RAN in order to configure UE power saving management scheme for connected mode DRX:

-	UL and/or DL Periodicity;

-	N6 Jitter Information associated with the DL Periodicity;

-	Indication of End of Data Burst.

#### 5.37.8.2	Periodicity and N6 Jitter Information associated with Periodicity

At AF Session Establishment with QoS, the 5G System may be provided with Periodicity information for NG RAN to configure UE power management for connected mode DRX. The Periodicity information is provided by the AF to the PCF via NEF or directly to the PCF when the AF is trusted.

Editor's note:	The method used to provide the Periodicity information to SMF is FFS.

The PCF should send the Periodicity information received from the AF and includes an indication for SMF to request UPF to perform jitter measurements within PCC Rules, in accordance with the operator's local policies.

Upon reception of PCC rule, the SMF determines the TSCAI and forwards it to the NG-RAN. If the PCC rule includes an indication to perform jitter measurements, the SMF also sends the DL Periodicity to the UPF and requests the UPF to monitor and periodically report the Jitter information associated with the DL Periodicity using the N4 Session Modification procedure, see clause 5.8.5.11. It provides the DL Periodicity in the request to the UPF.

NOTE 1:	How the UPF derives the N6 jitter is implementation dependent.

NOTE 2:	How the UPF derives periodicity when not provided by the AF is implementation dependent.

Editor's note:	The method used to report the Periodicity information from the UPF to the SMF is FFS.

At reception of Jitter measurements from the UPF in the N4 Session Level Report, the SMF includes the Jitter information together with the associated Periodicity in the TSCAI and forwards it to the NG-RAN in an NGAP message, see clause 5.27.2.

NOTE 3:	In order to prevent frequent updates from the UPF, the UPF sends the N6 Jitter Measurement Report periodically or only when the jitter is larger than a threshold.

#### 5.37.8.3	End of Data Burst Indication

An indication of End of Data Burst may be provided to the NG-RAN by the UPF, e.g. to configure UE power management schemes like connected mode DRX.

The PCF may provision the Protocol Description within PCC Rules based on the information provided by the AF and/or the local operator policies.

SMF should request the UPF to detect the last PDU of the data burst and the mark the End of Data burst in the GTP-U header of the last PDU in downlink, according to the PCC rule and/or the local operator policies.

According to the request and information from the SMF, the UPF identifies the last PDU of a Data burst in the DL traffic based on the End indication according to the Protocol Description and provides an End of Data Burst indication to the NG-RAN over GTP-U of the last PDU of a Data burst. The End of Data Burst indication provides the following information about the PDU:

- The PDU is not the Last PDU in a Burst

- The PDU is the last PDU in a Burst, but the time to the next burst is not specified

- The PDU is the last PDU of the burst and an indication of the time (range) to the next burst

Note: Whether the information that “The PDU is not the Last PDU in a Burst” is provided explicitly by a dedicated value of the EoDB indication or implicitly by the absence of EoDB indication in the GTP-U header is determined in Stage 3.

Editor’s Note: The number of time ranges and proposed time range threshold values are pending feedback from RAN WG 2 and SA4.

NOTE:	There can be some packets from the Data Burst received by NG-RAN after the PDU with End of Data Burst Indication if packets are received out of sequence.

# * * * * End of change * * * *