3GPP TSG-SA2 Meeting #157	S2-2308079

# Berlin, Germany, 22-26 May 2023									  (revision of S2-2308075)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0827 | rev | 4 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Geographical Identifier in UE Mobility analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rakuten Mobile, Orange, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the conclusion of Key Issue #9: Enhancement of NWDAF with finer granularity of location information that The following use cases will be addressed in normative work: - Existing analytics IDs that request UE location (i.e. Observed Service Experience, UE mobility, and Dispersion Analytics) are enhanced: … - to support the PSAP resolution use case … Considering the case of PSAP resolution, P-CSCF or IMS AS may request assistance from NWDAF as specified in TS 23.228. Section Y.7 In the case that a given cell belongs to more than one area identified by a Geographical Identifier, based on operator policy, the P-CSCF or an IMS AS may use the UE mobility analytics provided by NWDAF as specified in TS 23.288 [103]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To support the PSAP resolution, the UE Mobility analytics feature should be updated to provide output data with the Geographical Identifier for the P-CSCF or IMS AS to determine appropriate PSAP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PSAP resolution use case is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.7.2.1, 6.7.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## First CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	Void.

[6]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[7]	3GPP TS 28.550: "Management and orchestration; Performance Assurance".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[10]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[11]	ITU-T Recommendation P.1203.3: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport - Quality integration module".

[12]	3GPP TS 38.215: "NR; Physical layer measurements".

[13]	Void.

[14]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[17]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes".

[18]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[19]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[20]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; stage 2".

[21]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); stage 2".

[22]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[23]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[24]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[25]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[26]	3GPP TS 29.503: "Unified Data Management Services; Stage 3".

[27]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[28]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[29]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[30]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[31]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[32]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[33]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[34]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[35]	3GPP TS 22.071: "Technical Specification Group Systems Aspects; Location Services (LCS)".

[36]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[37]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[38]	GSMA TS.06: "IMEI Allocation and Approval Process".

[39]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[40]	ITU-T Y.1540: "Internet protocol data communication service - IP packet transfer and availability performance parameters".

[41]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRMs). Integration Reference Point (IRP): Information Service (IS)".

[42]	3GPP TS 32.422: "Subscriber and equipment trace: Trace control and configuration management".

[43]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

[xx]	3GPP TS 23.228: " IP Multimedia Subsystem (IMS); Stage 2".

## Next CHANGE

#### 6.7.2.1	General

NWDAF supporting UE mobility statistics or predictions shall be able to collect UE mobility related information from NF, OAM and to perform data analytics to provide UE mobility statistics or predictions.

The service consumer may be a NF (e.g. AMF, SMF or AF).

The consumer of these analytics may indicate in the request:

-	Analytics ID = "UE Mobility".

-	Target of Analytics Reporting: a single UE (SUPI) or a group of UEs (an Internal Group ID);

-	Analytics Filter Information optionally containing:

-	Area of Interest (AOI): restricts the scope of the UE mobility analytics to the provided area. If the request is for fine granularity location information (i.e. with a finer granularity than cell), the AOI may be described as shown in clause 5.5 of TS 23.273 [39];

NOTE 1:	For LADN service, the consumer (e.g. SMF) provides the LADN DNN to refer the LADN service area as the AOI.

-	Visited Area(s) of Interest (visited AOI(s)): additional filter to only consider UEs that are currently (i.e. now) in the "AOI" and had previously (i.e. in the "Analytics target period") been in at least one of the Visited AOI(s). If this parameter is provided, the Analytics target period shall be in the past (i.e. supported for statistics only);

-	Linear distance threshold: An event where the UE moves by more than some predefined straight line distance from a previous location as per TS 23.273 [39];

-	an optional list of analytics subsets that are requested (see clause 6.7.2.3).

NOTE 1:	For LADN service, the consumer (e.g. SMF) provides the LADN DNN to refer the LADN service area as the AOI.

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

NOTE 2:	For regular analytics scenarios, the Analytics target period is associated with the Analytics Filter Information = AOI, while for the scenario that Analytics ID=UE Mobility and Analytics Filter Information = (AOI and visited AOI(s)), as described in this clause, the Analytics target period is associated with the visited AOI(s) and to obtain the statistics for those UEs that currently reside in the AOI and had previously (i.e. in the "Analytics target period") been in at least one of the Visited AOI(s).

-	Optionally, maximum number of objects;

-	Preferred level of accuracy of the analytics;

-	Preferred order of results for the time slot entries: ascending or descending time slot start;

-	Optionally, preferred granularity of location information: TA level or cell level "below cell level";

NOTE 3:	Definition of "below cell level" is described in clause 6.1.3.

-	Optionally, Preferred orientation of location information: ("horizontal", "vertical", "both");

-	Optionally, Spatial granularity size and Temporal granularity size;

-	UE Location order indicator: indicates the NWDAF should derives and provides the UE Mobility analytics in a UE Location order; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

## Next CHANGE

#### 6.7.2.3	Output Analytics

The NWDAF supporting data analytics on UE mobility shall be able to provide UE mobility analytics to consumer NFs or AFs. The analytics results provided by the NWDAF could be UE mobility statistics as defined in table 6.7.2.3-1, UE mobility predictions as defined in Table 6.7.2.3-2:

Table 6.7.2.3-1: UE mobility statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in clause 5.9.7 of TS 23.501 [2], SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| > UE location (1..max) | Observed location statistics (see NOTE 2). |
| >> UE location (NOTE 4) | TA or cells which the UE stays or location coordinates (see NOTE 3). |
| >> Ratio (NOTE 4) | Percentage of UEs in the group (in the case of a UE group). |
| >> UE's geographical distribution (NOTE 4) | The geographical distribution of the UEs that can be selected by the AF for application service. |
| >> Geographical Identifier (NOTE 4) | Geographical Identifier as specified in TS 23.228 [xx] (see NOTE 5) |
| > UE's direction | The direction of the UEs in the coverage area. |
| NOTE 1: When Target of Analytics Reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics mobility result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE. NOTE 2: If Visited AOI(s) was provided in the analytics request/subscription, the UE location provides information on the observed location(s) that the UE or group of UEs had been residing during the Analytics Target Period. NOTE 3: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information and Spatial granularity size. NOTE 4: Analytics subset that can be used in "list of analytics subsets that are requested". NOTE 5: It depends on the implementation how the NWDAF collects the geographic identifier. |  |

Table 6.7.2.3-2: UE mobility predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in clause 5.9.7 of TS 23.501 [2], or SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of predicted time slots. |
| >Time slot start | Time slot start time within the Analytics target period. |
| > Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| > UE location (1..max) | Predicted location prediction during the Analytics target period. |
| >> UE location (NOTE 3) | TA or cells where the UE or UE group may move into or location coordinates (see NOTE 2). |
| >> Confidence | Confidence of this prediction. |
| >> Ratio (NOTE 3) | Percentage of UEs in the group (in the case of a UE group). |
| >> UE's geographical distribution (NOTE 3) | The geographical distribution of the UEs that can be selected by the AF for application service. |
| >> Geographical Identifier (NOTE 3) | Geographical Identifier as specified in TS 23.228 [xx] (see NOTE 4) |
| > UE's direction | The direction of the UEs in the coverage area. |
| NOTE 1: When Target of Analytics Reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics mobility result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE. NOTE 2: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information and Spatial granularity size. NOTE 3: Analytics subset that can be used in "list of analytics subsets that are requested". NOTE 4: It depends on the implementation how the NWDAF collects the geographic identifier. |  |

Editor's note:	How the NWDAF provides analytic output when Linear distance threshold is included in analytics request is FFS.The results for UE groups address the group globally. The ratio is the proportion of UEs in the group at a given location at a given time.

The number of time slots and UE locations is limited by the maximum number of objects provided as part of Analytics Reporting Information.

The time slots shall be provided by order of time, possibly overlapping. The locations shall be provided by decreasing value of ratio for a given time slot. The sum of all ratios on a given time slot must be equal or less than 100%. Depending on the list size limitation, the least probable locations on a given Analytics target period may not be provided.

If a UE Location order indicator is included in the Analytics Filter information, the NWDAF does not aggregate the UE locations in a long duration but provides the UE locations one by one in their own time period, i.e. the "UE location (1..max)" in the UE Mobility analytics has only one UE location (TA, Cell or a finer granularity UE Location smaller than cell) which indicates the UE is located in this UE location in the duration from the time slot start.

## End of CHANGEs