SA WG2 Meeting #S2-158	S2-2308635

21 – 25 August 2023, Goteborg,Sweden	(revision of S2-230xxxx)

Source:	MediaTek Inc.

Title:	Discussion on Equivalent SNPN used by UE for Localized Services

Document for:	Discussion

Agenda Item: 	9.24.2

Work Item / Release:	eNPN_Ph2/Rel-18

Abstract of the contribution: This paper proposes to discuss how UE uses the Equivalent SNPNs allocated by the serving SNPN

- Discussion

The Equivalent SNPN list is allocated by a serving SNPN to which the UE is registered and each SNPN identity included in the Equivalent SNPN list is considered as equivalent as the currently serving SNPN from the UE point of view.

  - The Assumptions on the Equivalent SNPN list

Before discussion the usage of the equivalent SNPN list for Localized Services, it needs to discuss how the Equivalent SNPN list is generated by the serving SNPN. The below question needs to be clarified first before proposing any solution (i.e. Alt. 1 and Alt. 2) as way forward:

-	Whether each SNPN included in the Equivalent SNPN list offers same services including non-localized services and localized services as the serving SNPN?

Based on the above: two assumptions can be considered for the Equivalent SNPN list:

- 	Assumption (a): Each SNPN included in the Equivalent SNPN list provides the non-localized services and the localized services which are offered in the serving SNPN.

-	Assumption (b): Each SNPN included in the Equivalent SNPN list provides the non-localized services and optionally the localized services which are offered in the serving SNPN.

1.2	The usage of Equivalent SNPN by UE

Based on the above, to solve the below EN, two alternatives below are proposed:

Editor’s note:	Whether the Equivalent SNPN(s) has same validity information as the SNPN that providing access for Localized Services the UE was last registered is for FFS

- Alt. 1: UE considers an Equivalent SNPN is same as currently serving SNPN, i.e. the Equivalent SNPN has same validity information as the serving SNPN.

- Alt. 2: UE does not indicate the support of the Equivalent SNPN feature during registration procedure when accessing an SNPN for Localized Services.

For Assumption (a), Alt. 1 is more suitable than Alt. 2 since UE can get continuous services (i.e. non-localized services and localized services when moving to another SNPN. For Assumption (b), Alt 1 can allow the UE at least to get non-localized services when moving to another SNPN. However, Alt. 2 will require the UE to interrupt all the services and re-select an SNPN.

2	Proposal

It proposes to firstly discuss which of the above assumption is taken, then discuss which of the below alternatives is decided as way forward (S2-2308636 or S2-2308637)