3GPP TSG-WG SA2 Meeting #159	S2-2310285

Xiamen, P.R. China, 09 - 13 October, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4948 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of MBSR authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution proposes the following changes: - Update of MBSR authorization Based on the C1-236002, CT1 discussed how to inform MBSR node about change of MBSR authorization state in the following cases: Case-1) MBSR node is registered and operates as an MBSR and MBSR authorization state changes from "authorized" to "not-authorized" and the MBSR node is allowed to operate as a UE. Case-2) MBSR node is registered and operates as an MBSR and MBSR authorization state changes from "authorized" to "not-authorized" and the MBSR node is not allowed to operate as a UE. Case-3) MBSR node is registered and operates as a UE and MBSR authorization state changes from "not-authorized" to "authorized". Case-4) MBSR node is performing initial registration and indicates the request to operate as a MBSR, and MBSR authorization is "authorized" and the MBSR node is allowed to operate as an MBSR. Case-5) MBSR node is performing initial registration and indicates the request to operate as a MBSR, and MBSR authorization is "not-authorized" and the MBSR node is allowed to operate as a UE. Case-6) MBSR node is performing initial registration and indicates the request to operate as a MBSR, and MBSR authorization is "not-authorized" and the MBSR node is not allowed to operate as a UE. For Case-1 and Case-3, CT1 agreed that generic UE configuration update procedure is used to inform the UE about the MBSR authorization state change. Deregistration procedure was not extended. Deregistration procedure with "re-registration required" indication without any extensions can be used. For Case-2, CT1 agreed that deregistration procedure with existing cause can be used so there is no need of extensions. For Case-4 and Case-5, CT1 agreed that registration accept is used to inform the UE about the MBSR authorization state. For Case-6, CT1 agreed that registration reject with existing cause can be used so there is no need of extensions. This contribution proposes to update MBSR authorization to align with CT1 conclusion. - Clarificaitons on the location granularity used for MBSR authorization determination The granularity of the corresponding location for MBSR authorization is not clear. This paper also proposes that the corresponding location can be represented by a list of Tracking Areas, so AMF can decide by itself whether the MBSR is within the location. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of MBSR authorization |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current descriptions are not aligned with CT1 conclusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR4465 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.35A.4	MBSR authorization

For a MBSR, the subscription information stored in the HPLMN indicates whether it is authorized to operate as MBSR, and the corresponding location and time periods. The corresponding location can be represented by a list of Tracking Areas.

NOTE 1:	For non-roaming MBSRs, the operator local policy can be taken into consideration for MBSR authorization, e.g. based on the network status, or a limit on the number of MBSRs operating in a certain area.

When the MBSR (IAB-UE) performs initial registration with the serving PLMN, it indicates the request to operate as a MBSR as described in clause 5.35A.1. The AMF authorizes the MBSR based on the subscription information, and provides MBSR authorized indication to the MBSR node over NAS and NG-RAN over NGAP as described in the registration procedure in TS 23.502 [3]. The MBSR establishes the connection to OAM system using the configuration information for MBSR operation upon the reception of MBSR authorization indication (authorized).

When MBSR roaming is supported, a roaming agreement between VPLMN and HPLMN regarding MBSR operation is in place. The AMF can make use of the subscription data for authorization of the MBSR in the V-PLMN.

MBSR (IAB-DU) can use IAB-node integration procedure or inter-IAB-donor gNB mobility procedure to integrate into the serving PLMN to provide service.

NOTE 2:	How the MBSR obtains the configuration information for MBSR operation is described in clause 5.35A.2.

If the MBSR operation is not authorized (e.g. due to location or time limitation), the AMF of the MBSR can indicate to the MBSR IAB-UE that it is not allowed to act as an MBSR, i.e. the MBSR authorization indication (not authorized), as part of registration procedure. In this case, the AMF includes the MBSR authorization indication (not authorized) to donor-gNB. The AMF may provide the MBSR authorization indication either in a Registration Accept (if the PLMN allows the MBSR IAB-UE to be registered in the PLMM) or the AMF may send Registration Reject (if the PLMN does not allow the MBSR IAB-UE to be registered in the PLMN) message with existing cause. The MBSR provides the authorization indication (not authorized) to its AS layer.

Editor's note:	Whether the MBSR needs to provide the authorization indication (not authorized) to its AS layer will be further synch with RAN WGs.

When the MBSR authorization state changes for a registered MBSR node (either authorized, or not authorized), the AMF updates the MBSR and the NG-RAN accordingly. Based on the operator configuration, the AMF may use either Deregistration (including "re-registration required" indication) or the UE Configuration Update procedure to inform the MBSR regarding the updated authorization status:

-	When Deregistration procedure is used, AMF provides "re-registration required" indication, then when the MBSR performs Registration procedure after the deregistration.

-	When UE Configuration Update procedure is used, the AMF provides the new authorization indication in the UE Configuration Update Command. The MBSR provides the new authorization indication to its AS layer.

The AMF informs the NG-RAN of the new authorization status using UE Context Modification, Initial Context Setup procedure or the DOWNLINK NAS TRANSPORT message, with the following principles:

-	If the authorization state changes from authorized to not authorized and AMF uses the UE Configuration Updated procedure to update the MBSR, the AMF updates the NG-RAN with the new authorization indication (not authorized) by including this information in the DOWNLINK NAS TRANSPORT message. The NG-RAN completes handover of the UEs served by the MBSR before releasing the F1 connection to the MBSR IAB-DU.

-	If the authorization state changes from authorized to not authorized and the AMF uses the Deregistration procedure to update the MBSR, the AMF sends the UE Context Modification message to NG-RAN before triggering the deregistration towards MBSR after a certain period (e.g. based on the expiration of a timer configured on the AMF).

If a PDU session is used to provide OAM access for MBSR when it is not authorized but remains registered, the AMF may notify the SMF serving the PDU session for O&M access to trigger the release of the PDU Session.

NOTE 3:	The AMF delays the MBSR de-registration to allow the IAB-donor gNB to move all connected UEs via MBSR to other cells as specified in clause 8.9.10 of TS 38.401 [42].

NOTE 4:	The mechanism applies to both roaming and non-roaming MBSR operations.

# * * * * End of changes * * * *