3GPP TSG-WG SA2 Meeting #159	S2-2310324

Xiamen, China, Oct 9 – 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4470 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Support of Time Sensitive Networking (TSN) enabled Transport Network (TN) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As requested by CT4 in S2-xxxxx, it is proposed to add clarification that TL-Containers and related Gate Control information as described clause M.1 of TS 23.501 [2] are part of PDU Session context and should be removed during PDU Session Release or QoS Flow Release. The SMF/CUC may instruct the RAN or UPF to assign a distinct N3 tunnel end point address for the QoS Flow as described in Annex M.1 of TS 23.501 no matter whether AN-TL or CN-TL is supported or not. Besides, If TLs are supported, Step 11~13 should be enforced. Thus Step 11~13 are conditional. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the description on not supporting TLs and change Step 11~13 to Conditional. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description is not aligned with TS 23.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.501 CR 4955 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

F.4	5GS interworking with TSN deployed in the transport network

For 5GS to control the IEEE TSN features deployed in the transport network, the SMF/CUC interacts with the CNC in the transport network (TN CNC).

Figure F.4-1: 5GS Bridge information configuration

1.	The UE establishes a PDU Session as described in clause 4.3.2.2.1.

2.	During the PDU Session Establishment procedure, the SMF/CUC requests the UPF to assign the N3 tunnel information via N4 Session Establishment or Modification procedure. The SMF/CUC includes a TL-Container to the N4 Session Establishment or Modification request including a get-request to the TL-Container, as described in clause 5.28a.2 of TS 23.501 [2].

The UPF responds with a N4 Session Establishment or Modification response. If the UPF supports CN-TL, the UPF includes a TL-Container to the response. The TL-Container includes a get-response as described in clause 5.28a.2 of TS 23.501 [2]. The SMF/CUC stores the get-response.

3.	During the PDU Session Establishment procedure, the SMF/CUC requests the NG-RAN to assign the N3 tunnel information by invoking the Namf_Communication_N1N2MessageTransfer request. The SMF/CUC includes a TL-Container to the N2 SM information in the request, the TL-Container contains a get-request as described in clause 5.28a.2 of TS 23.501 [2].

The NG-RAN responds with a N2 SM information. If the NG-RAN supports AN-TL, the NG-RAN includes a TL-Container to the N2 SM information. The TL-Container includes a get-response as described in clause 5.28a.2 of TS 23.501 [2]. The SMF/CUC stores the get-response.

4.	The AF (TSCTSF, AF, NEF or TSN AF) invokes the Npcf_PolicyAuthorization_Create/Update request. This may be due to reception of Ntsctsf_QoSandTSCAssistance_Create/Update request by the TSCTSF as described in clause 4.15.6.6 or clause 4.15.6.6.a, or due to reception of 5GS Bridge configuration by the TSN AF as described in clause F.2. The TSN AF or TSCTSF determines the TSC Assistance Container for one or more TSC streams and sends them to the PCF.

5-6.When PCF receives the request, the PCF initiates an SM Policy Association Modification procedure. The PCF notifies the corresponding SMF/CUC via Npcf_SMPolicyControl_UpdateNotify message as described in clause 4.16.5.2. The PCF updates the PCC rule to the SMF/CUC. The PCC rule includes the 5GS QoS profile along with TSC Assistance Container.

SMF/CUC triggers the PDU Session Modification procedure as described in clause 4.3.3.2 to establish a QoS Flow to transfer the TSC streams.

7.	During the PDU Session Modification procedure, the SMF/CUC provides the information received in PCC rules to the UPF via N4 Session Modification procedure.The SMF/CUC may instruct the UPF to assign a distinct N3 tunnel end point address for the QoS Flow as described in Annex M.1 of TS 23.501 [2].

The UPF responds with a N4 Session Modification response.

8.	During the PDU Session Modification procedure, the SMF/CUC provides the information received in PCC rules to the NG-RAN by invoking the Namf_Communication_N1N2MessageTransfer request. The SMF also determines the TSCAI for the QoS Flow(s) and sends the TSCAI along with the QoS profile(s) to the NG-RAN. The SMF/CUC may instruct the NG-RAN to assign a distinct N3 tunnel end point address for the QoS Flow as described in Annex M.1 of TS 23.501 [2].

The NG-RAN responds with a N2 SM information.

9.	The SMF/CUC determines the merged stream requirements in the TSN UNI towards the TN CNC as described in Annex M of TS 23.501 [2]. The TN CNC uses the merged stream requirements as input to select respective path(s) and calculate schedules in TN.

10.	Based on the results, the TN CNC provides a Status group that contains the merged end station communication-configuration back to the SMF/CUC.

11.	[Conditional] If the response from TN CNC includes InterfaceConfiguration, the SMF/CUC triggers the PDU Session Modification procedure as described in clause 4.3.3.2 to modify the QoS Flow to transfer the TSC streams.

12.	[Conditional] During the PDU Session Modification procedure, if the UPF supports CN-TL, the SMF/CUC invokes N4 Session Modification procedure and includes a TL-Container to the N4 Session Modification request including a set-request to the TL-Container for the QoS Flow as described in clause 5.28a.2 of TS 23.501 [2].

The UPF responds with a N4 Session Modification response. If the UPF supports CN-TL, the UPF includes a TL-Container to the response. The TL-Container includes a set-response for the QoS Flow as described in clause 5.28a.2 of TS 23.501 [2].

13. [Conditional] During the PDU Session Modification procedure, if the NG-RAN supports AN-TL, the SMF/CUC invokes the Namf_Communication_N1N2MessageTransfer request. The SMF/CUC includes a TL-Container to the N2 SM information in the request, the TL-Container contains a set-request for the QoS Flow as described in clause 5.28a.2 of TS 23.501 [2]. The SMF/CUC may also update the TSCAI in the NG-RAN for the BAT in DL direction as described in Annex M, clause M.1 of TS 23.501 [2], if the SMF/CUC received a TimeAwareOffset or AccumulatedLatency from TN CNC for a downlink stream (i.e. for a Talker in the UPF/CN-TL) in step 7.

The NG-RAN responds with a N2 SM information. If the NG-RAN supports AN-TL, the NG-RAN includes a TL-Container to the N2 SM information for the QoS Flow. The TL-Container includes a set-response as described in clause 5.28a.2 of TS 23.501 [2].

NOTE:  TL-Containers and related Gate Control information as described clause M.1 of TS 23.501 [2] are part of PDU Session context and should be removed during PDU Session Release or QoS Flow release.

# * * * * End of changes * * * *