3GPP TSG-WG SA2 Meeting #159	S2-2310750

# Xiamen, China, October 9 – 13, 2023		(was_S2-2308963)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0901 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Movement behavior analytics updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Congestion in a target area cannot be directly obtained from the output analytics. It can only be derived based on some additional information, which is out of 3GPP scope. This part should be clarified, i.e. this has no relationship with “User Data Congestion Analytics”. Movement behavior analytics works mainly based on finer granularity location information, i.e. eLCS provided 2D/3D reference point in coordinates with higher accuracy. The 3GPP TA ID/cell ID level location information is too coarse-grained. But such UE location can also help to determine the speed, and direction over a target period, and further can help to determine the UE location environment, e.g. indoor/outdoor. Such data is collected as specified in Table 6.21.2-1. As such information is duplicated with what has been collected from eLCS system, the usage needs to be clarified. Preferred level of accuracy per analytics subset is optional. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify congestion is not directly provided via movement behavior analytics, just derived by application. - Add description of how to utilize UE location information collected from AMF - Clarify preferred level of accuracy per analytics subset is optional |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.21.1, 6.21.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change* * * *

## 6.21	Movement Behaviour Analytics

### 6.21.1	General

The movement behaviour analytics provides analytics information (statistics or predictions) regarding the location, direction and velocity of UEs during an analytics target period in a target area. The statistics of movement behaviour collects a map of UE movements in a certain region in a certain period of time. In addition, the consumers can further derive the congestion situation in the target area through such analytics.

The NWDAF provides movement behaviour analytics to a NF (e.g. NEF, AF).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "movement behaviour";

-	Target of Analytics Reporting: any UE;

-	Analytics Filter Information:

-	Area of Interest (AOI): restricts the scope of the movement behaviour analytics to the provided area. The AOI may be described as shown in clause 5.5 of TS 23.273 [39];

-	Optionally, the list of analytics subsets that are requested among those specified in clause 6.21.3;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Optionally, preferred level of accuracy of the analytics;

-	Optionally, preferred level of accuracy per analytics subset (see clause 6.21.3);

-	Optionally, preferred granularity of location information: "longitude and latitude level";

-	Optionally, preferred orientation of location information: ("horizontal", "vertical", "both")"; and

-	Optionally, maximum number of objects.

# * * * * Second change* * * *

### 6.21.4	Procedures

Figure 6.21.4-1 depicts the procedure for movement behaviour analytics provided by NWDAF.

Figure 6.21.4-1: "Movement Behaviour" analytics provided by NWDAF

1.	The NWDAF receives an analytics request from the NWDAF service consumer (as defined in clause 6.1.1.1) or from an AF via NEF (as defined in clause 6.1.1.2) to request movement behaviour analytics within a target area for any UE.

The Analytics ID is set to "Movement Behaviour". The target for analytics reporting is set to be any UE. Analytic filters may be provided as shown in clause 6.21.1. The Area of Interest is the target area that may be defined as a geographical area or as a geopolitical name of an area, in such case, the NEF will translate it as the identities of one or more radio cells or tracking areas.

The Consumer NF can request statistics or predictions or both for a given Analytics target period.

2.	The NWDAF collects input data from AMF as defined in Table 6.21.2-1 via the AMF event exposure service.

The collected UE location information from AMF is optional and may assist in the statistics/prediction of UE moving direction and speed in step 4.

3.	The NWDAF collects the UE location information as defined in Table 6.21.2-1 via Ngmlc_Location services.

Based on the analytics target period, the NWDAF may trigger the immediate 5GC-MT-LR procedure as defined in clause 6.1.2 or the deferred 5GC-MT-LR procedure in clause 6.3.1 of TS 23.273 [39] to obtain UE's location information.

4.	The NWDAF derives requested analytics.

5.	The NWDAF provides the analytics as defined in Table 6.21.3-1 and Table 6.21.3-2 using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Subscribe_Notify, depending on the service used in step 1.

# * * * * End of change* * * *