3GPP TSG-WG SA2 Meeting #159	S2-2311129

# Xiamen, China, Oct 9, 2023 – Oct 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4541 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of the option to use TSCTSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 4.15.6.14 describes two options for AF request to provision traffic characteristics for a group; either by invoking TSCTSF, or invoking PCF via UDR. According to clause 6.1.3.28 in TS 23.503, the AF request that is targeted to a GPSI or External Group ID may contain traffic characteristics. If TSCTSF is not involved, the NEF stores the traffic characteristics (among other QoS information in the request) to the UDR, and UDR notifies the PCF for the content of the request. This means the PCF needs to compose a TSC Assistance Container and include it in PCC Rules; the TSCAC contains the given traffic characteristics, such as Periodicity, Burst Arrival Time, etc. This means that there is no need to involve the TSCTSF to compose the TSCAC, and the option to use TSCTSF can be removed from clause 4.15.6.14. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the option to invoke TSCTSF in clause 4.15.6.14. Other changes: - clarifies what Data Subset is used. - correction in step 12 that the request may be targeted to GPSI/SUPI as well. - correction of the message name in step 13. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TSCTSF is involved even though it does not provide any value to the flow. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 23.501 CR5055, TS 23.503 CR1187 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

#### 4.15.6.14	Procedures for AF requested QoS for a UE or group of UEs not identified by a UE address

Figure 4.15.6.14-1: Procedure for AF requested QoS for a UE or group of UEs not identified by a UE address

0.	When a new PDU Session is established, the PCF subscribes to notifications for Application Data, Data Subset = "AF request for QoS information" from UDR.

The PCF registers to BSF as described in TS 23.503 [20].

1.	The AF sends a request to reserve resources using Nnef_AF_Request_QoS_Create request message (GPSI or External Group ID, AF Identifier, Flow description(s) or External Application Identifier, QoS reference or individual QoS parameters, Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20]), DNN, S-NSSAI) to the NEF. Optionally, a period of time or a traffic volume for the requested QoS can be included in the AF request. The AF may, instead of a QoS Reference, provide the individual QoS parameters. Regardless, whether the AF request is formulated using a QoS Reference or Individual QoS paramaters, the AF may also provide one or more of the parameters that describe the traffic characteristics as described in clause 6.1.3.23 or 6.1.3.23a of TS 23.503 [20]. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as described in clause 6.1.3.22 of TS 23.503 [20].

2.	This step is the same as step 2 in clause 4.15.6.6.

3.	This step is the same as steps 2-4 in clause 4.15.6.7a.

4.	The NEF uses the Nudr_DM service to store the QoS information related to the DNN and S-NSSAI, and Internal Group ID or SUPI in UDR. The information is stored as Application Data in UDR, Data Subset = "AF request for QoS information". If the AF requested for notifications of Resource allocation status or other events, the NEF includes the information required for reporting the event, including the Notification Target Address pointing to the NEF or AF and the Notification Correlation ID containing the AF Transaction Internal ID.

5.	The NEF replies to the AF with Nnef_AF_Request_QoS_Create response.

6.	The UDR notifies the PCF(s) that have subscribed the Application Data for the given DNN/S-NSSAI.

7.	The PCF(s) identifies the active PDU Sessions associated with the data received from UDR. The PCF(s) manages the AF request QoS information as defined in clause 6.1.3.28 in TS 23.503 [20].

8.	When an event condition is met, e.g. that the establishment of the transmission resources corresponding to the QoS update succeeded or failed, the PCF sends a notification NEF as applicable. This step is the same as steps 7-8 in clause 4.15.6.6.

*************************** End of Changes ****************************