SA WG2 Meeting #S2-159	S2-2311189

09 - 13 October, 2023, Xiamen, P.R. China

Title:	[draft]Reply LS on 5G ProSe UE-to-UE relay discovery

Response to:	S2-2310091

Release:	Rel-18

Work Item:	5G_ProSe_Ph2

Source:	SA2

To:	CT1

Cc:

Contact person:	Jung Je Son

jungje dot son at interdigital dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR

1.	Overall description

SA2 thanks CT1 for LS(S2-2310091, C1-236448). SA2 have discussed it and would like to provide following answers:

Q1: How can this issue be resolved?

Answer to Q1: SA2 agree that user info ID of UEs in proximity in 5G ProSe UE-to-UE relay discovery cannot be obtained during 5G ProSe UE-to-UE Discovery. SA2 will update relevant text in TS23.304.

Question 2: How can the 5G ProSe UE-to-UE relay UE tell that the 5G ProSe UE-to-UE Relay Discovery Solicitation message is for the relay reselection so that it can skip finding the target end UE and respond to the source end UE directly (in candidate U2U Relay discovery)?

Answer to Q2 : User info of U2U Relay UE is included as User Info ID of the discoveree 5G ProSe End UE. Therefore, for candidate Relay Discovery, the UE-to-UE Relay will respond when its User Info ID is included in the discovery message as discoveree. And as it is for reusing direct discovery (e.g. RSC is not included), other UE-to-UE Relay does not need to forward discovery message for others.

Q3: How can the 5G ProSe UE-to-UE relay UE know which service the discoverer end UE wants to use before accepting the 5G ProSe UE-to-UE Relay Discovery Solicitation message (in candidate U2U Relay discovery)?

Answer to Q3: When RSC is not included at solicitation message, UE-to-UE Relay does not need to consider RSC for accepting the discovery solicitation message. In negotiated Relay reselection, End UE already selected candidate 5G ProSe UE-to-UE among the UE-to-UE relays supporting the same RSC to the one for the current connection via a UE-to-UE Relay. And the used RSC will be negotiated again during PC5 connection setup.

Question 4: Why are different approaches applied to the user info of the target 5G ProSe end UE (in L3/L2 U2U Relay connection and integrated discovery)?

Answer to Q4: End to end communication via UE-to-UE Relay is an extension of the service provided by direct communication. For UE-to-UE Relay communication, ProSe communication via L3 U2U Relay and ProSe communication via L2 U2U Relay only support UE oriented Layer-2 link establishment (refer to subclause 6.4.3.1 in TS 23.304). For integrated discovery, both service oriented Layer2 link establishment (refer to subclause 6.4.3.1 in TS 23.304) and UE oriented Layer-2 link establishment are supported. And for service oriented Layer2 link establishment, User Info ID of target UE is not needed.

2. Actions:

To CT1:

Action: SA2 kindly asks CT1 to take the above information into account.

3. Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#160 | November 13 - 17th, 2023 | Chicago | US |
| SA2#161 | February 26 th - March 01st, 2023 | Athens | Greece |

Please see https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/