3GPP TSG-WG SA2 Meeting #159	S2-2311321

# Xiamen, China, October 9-13, 2023

# Title:	Reply LS on UE implementing the de-registration inactivity timer

# Response to:	LS on UE implementing the de-registration inactivity timer (S2-2310071/C1-232970)

# Release:	Rel-18

# Work Item:	eNS_Ph3

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Krisztian Kiss

E-mail Address:	krisztian@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 thanks CT1 for their LS in S2-2310071/C1-232970 on UE implementing the de-registration inactivity timer. SA2 would like to provide the following feedback on CT1 consideration whether a solution with network control only could be possible or not, i.e. whether the de-registration inactivity timer on the UE side could be avoided.

SA2 has carefully studied whether to define the de-registration inactivity timer both for UE and AMF for supporting UEs and did not identify any problems doing so. For supporting UEs, both UE and AMF can implicitly deregister the slice without the need to send an updated Allowed NSSAI by the AMF via the UE Configuration Update procedure. SA2 considers this a useful optimization in terms of reducing NAS signaling. For non-supporting UEs, AMF informs the UE about the updated Allowed NSSAI as per pre-Rel-18 specifications.

SA2 recommends CT1 to define the behaviour as per TS 23.501 specification.

2. Actions:

To CT1 group.

ACTION: 	SA2 kindly asks CT1 to take above information into account.

3. Date of Next TSG SA WG2 Meetings:

SA2#160	November 13-17, 2023	Chicago, IL, USA