SA WG2 Meeting #160-Ad Hoc-e	S2-2400647

E-meeting, January 22 – 29, 2024	(revision of S2-24xxxxx)

Source:	LG Electronics

Title:	Term and Architecture Assumptions about multi-hop U2N relaying

Document for:	Approval

Agenda Item:	19.7

Work Item / Release:	FS_5G_ProSe_Ph3 / Rel-19

Abstract of the contribution: This paper proposes Term and Architecture Assumptions about multi-hop U2N relaying.

Discussion

TS 23.304 includes the following terms related to U2N relaying and U2U relaying.

5G ProSe UE-to-Network Relay: A 5G ProSe-enabled UE that provides functionality to support connectivity to the network for 5G ProSe Remote UE(s). 5G ProSe Remote UE: A 5G ProSe-enabled UE that communicates with a DN via a 5G ProSe UE-to-Network Relay. 5G ProSe UE-to-UE Relay: A 5G ProSe-enabled UE that provides functionality to support connectivity between 5G ProSe End UEs. 5G ProSe End UE: A 5G ProSe-enabled UE that connects with another 5G ProSe-enabled UE(s) via a 5G ProSe UE-to-UE Relay.

For multi-hop U2N relaying, we need to consider how to name the relay participated in multi-hop U2N relaying (i.e. intermediate relay).

The definition of "5G ProSe UE-to-UE Relay" specified in TS 23.304 as above reads that the relay serves/supports "5G ProSe End UE" rather than "5G ProSe Remote UE".

So, using "UE-to-UE Relay" for the intermediate relay seems not appropriate for multi-hop U2N relaying although the relay may support some operation or functionality similar to that of UE-to-UE Relay.

Observation: Using "UE-to-UE Relay" for the relay participated in multi-hop U2N relaying seems not appropriate because the definition of "5G ProSe UE-to-UE Relay" specified in TS 23.304 as above reads that the relay serves "5G ProSe End UE" rather than "5G ProSe Remote UE".

Proposal: It is proposed to use "5G ProSe Intermediate UE-to-Network Relay" for the relay participated in multi-hop U2N relaying as shown in the following diagram.

Proposal

It is proposed to agree the following changes into TR 23.700-03.

* * * * Start of 1st Change * * * *

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G ProSe Intermediate UE-to-Network Relay: A 5G ProSe-enabled UE that provides functionality to support connectivity to the network for 5G ProSe Remote UE(s) by using PC5 reference point with other 5G ProSe-enabled UEs, e.g. 5G ProSe Remote UE, 5G ProSe UE-to-Network Relay. The 5G ProSe Intermediate UE-to-Network Relay locates on the path between 5G ProSe Remote UE and 5G ProSe UE-to-Network Relay.

* * * * Start of Next Change * * * *

4.2	Architecture Assumptions

Figure 4.2-1: Multi-hop UE-to-Network relaying

-	5G ProSe Intermediate UE-to-Network Relay can be used to support connectivity to the network for the 5G ProSe Remote UE as shown in Figure 4.2-1.

NOTE:	Regarding how many hops can be supported for multi-hop UE-to-Network relaying,

-	for Layer-2, it will be decided by RAN WGs.

-	for Layer-3, it will be decided during this study.

* * * * End of Changes * * * *