3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2401185

Online, Jan 22 – 29, 2024

Source:	Tencent, Tencent Cloud

Title:	New use case for WT#3.2: NWDAF enhancements to mitigate abnormal behaviours in the 5GS

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN/R19

Abstract: It is proposed a new use case for key issue related to WT#3.2

1. Introduction/Discussion

This paper proposes a new use case of WT#3.2:

WT3: Study enhancements to support NWDAF-assisted policy control and address network abnormal behaviour

-WT3.1 – Study whether and what additionally needs to be supported in order to enhance 5GC NF operations (i.e. policy control and QoS) assisted by NWDAF. The work will firstly identify the specific use cases to be considered, in order to identify the appropriate scope. The work will analyse the result impacts on NWDAF (e.g. the need to understand specific NF functionality), and the compatibility of new solutions wrt existing analytics, in order to determine the need and benefits of new solutions.

-WT3.2 – Study prediction, detection, prevention, and mitigation of network abnormal behaviours i.e. signalling storm with the assistance of NWDAF.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-84.

# * * * * First change (all new text) * * * *

5.1.y	Use Case #y: < NWDAF enhancements to mitigate abnormal behaviours in the 5GS>

5.1.y.1	Description

With the popularity of various applications and increasing number of UEs the network needs to serve, there may be abnormal behaviors which happen in the network causing some problems e.g. signaling storm. Such abnormal behaviors may be caused by many reasons not limited to the protocol itself, but some operational reasons.

As a 5GC internal NF, NWDAF has the capability to collect network status and information related to the network interactions in among NG-RAN and 5GC, and also within 5GC NFs. By analyzing the collected information, the NWDAF may be able to detect some abnormal behaviors from the 5GS including network and UE.

5GC related NFs in the control plane interacts with NWDAF by requesting some analytics. NWDAF can therefore derive some new analytics to provide prediction, detection, prevention, and mitigation of network abnormal behaviours.

For example, NWDAF can train one AI or ML model to predict the future network behaviour by collecting data or other network related resources from RAN and also other NFs including AF. In this way, NWDAF may provide analytics to the AF for application layer configurations and network policy adjustments to achieve QoS requirements of the 5G network.

The NWDAF can also provide assistance to related NF to mitigate the abnormal behaviour if some UE are generating unwanted signalling or data traffic and the related NF can release the connection from the UE or even prevent the UE from causing the unwanted impacts to the 5GS.

# * * * * End of changes * * * *