3GPP TSG-WG SA2 Meeting #161	S2-2403025

Athens, Greece, 26 February – 1 March, 2024

Title:	[DRAFT] LS Reply on Clarification related to the information exposed by the 5GC to NSCE server.

Response to:	LS (C3-235717/S2-2401853) on Clarification related to the information exposed by the 5GC to NSCE server from CT3

Release:	Release 18

Work Item:	NSCALE

Source:	[Huawei to be] SA2

To:	CT3, SA3

Cc:	SA6, CT1, CT4,

Contact Person:

Name:	Haiyang, Sun

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.288 CR 1061

1. Overall Description:

SA2 thanks CT3 for the LS on Clarification related to the information exposed by the 5GC to NSCE server. Please find SA2 answer as below.

Question 1: Which of the network internal information provided in the Network Slice load information/analytics prediction can be exposed to an external AF or the NSCE Server? (For example, Area of Interest in terms of internal identities (e.g. TAI, Cell ID) sent to the NSCE server)

SA2 Answer:

TS 23.288, Clause 6.1.1.2 contains procedures for Analytics subscription by AFs via NEF, and Clause 6.1.2.2 contains procedures for Analytics request by AFs via NEF. The output of Network Slice load statistics/prediction as described in Table 6.3.3A-2 and Table 6.3.3A-4 of TS 23.288 can be exposed, although it was not exposed to the AF, and the attached CR fixed it. Note that Network Slice Instance load statistics/prediction as described in Table 6.3.3A-1 and Table 6.3.3A-3 of TS 23.288 should not be exposed as Network Slice instance is within 5GC and it is assumed external AF or NEF is not aware of Network Slice Instance. Besides, as described in TS 33.501 clause 5.9.2.3, NEF and the AF shall fulfil the security requirements that include that “Internal 5G Core information such as DNN, S-NSSAI etc., shall not be sent outside the 3GPP operator domain.”

Please also note Area of Interest is not part of output but it should be included in the subscription to the Analytic “Load level information” as Analytics Filter Information.  Besides, SA2 has updated TS 23.288 to add AF (NSCE server) in the consumer NF as attached and clarify Area of Interest can be list of TAs or Cells. If an AF provides geographical area, the NEF can map it in to list of TAs or Cells based on operator policy in the related procedures as mentioned above.

Please see the details in the attachment for the above information.

2. Actions:

To CT3 group.

ACTION: 	SA2 asks CT3 and SA3 to take the above information into account

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #161		26th February – 1st March, 2024		Athens, GR

TSG-SA2 Meeting #162		15th – 19th April, 2024			Changsha, CN