SA WG2 Meeting #161	S2-2402164

February 26th – March 1st, 2024, Athens, Greece	(revision of S2-2401636)

Source:	Apple, ETRI, Nokia, Nokia Shanghai Bell, LG Electronics, Interdigital, China Telecom, OPPO, NEC, KDDI

Title:	FS_MASSS DualSteer Key Issue for Session management aspects

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new Key Issue to cover the Session management aspects of WT#1.3 for DualSteer for the FS_MASSS TR 23.700-54.

# 1	Discussion

This paper proposes a new Key Issue to cover the Session management aspects of WT#1.3 of the FS_MASSS SID (SP-2401315).

# 2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.1.0

* * * * Start of Changes * * * *

## 5.1	Key Issue for DualSteer

### 5.1.x	Key Issue #X: Session management aspects for DualSteer

#### 5.1.x.1 	Description

This key issue will study the following potential session management enhancements to support DualSteer:

-	Whether and how to enhance session management functions and procedures for DualSteer traffic steering of a new service to a 3GPP access network and/or the DualSteer traffic switching across two 3GPP access networks belonging to the same PLMN (either HPLMN or VPLMN) or two different PLMNs or PLMN and PNI-NPN, which may further include the following:

- 	Whether and what enhancements are required in PDU Session establishment/modification/release;

-	Whether and what enhancements are required for N4 session management between the SMF and UPF, or between SMF+PGW-C and UPF+PGW-U; and

-	How the network selects the PSA UPF(s) or UPF+PGW-U to allow routing the traffic across 3GPP access networks towards the same PSA UPF or UPF+PGW-U to support DualSteer.

Splitting functionality is not supported for DualSteer in any scenario.

NOTE:   Impact to existing session management functionality related to the change of a service related data between a 3GPP access network and a non-3GPP access network will be considered as part of this key issue.

* * * * End of Changes * * * *